### Section 1: COLREGs / Rules of the Road & Situations (Questions 1–45)

1. What distinguishes a Management-level (Chief Mate) ROR answer from an Operational-level (2nd Mate) answer?

An operational officer focuses on basic steering rules, lights, and day shapes. A Chief Mate must articulate situational risk management: evaluating UKC and traffic density, early engine readiness, master notification thresholds, sound signaling compliance, bridge team task allocation, logging, and coordinating defensive maneuvering well before close-quarters develops.

2. Explain the application and limitations of Rule 19(d).

Rule 19(d) applies when detecting another vessel by radar alone in restricted visibility. If a close-quarters situation or risk of collision exists, take avoiding action in ample time. Avoid: (i) an alteration of course to port for a vessel forward of the beam (other than an overtaken vessel), and (ii) an alteration toward a vessel abeam or abaft the beam.

3. What constitutes a "close-quarters situation" under Rule 8 and Rule 19?

There is no fixed statutory distance in miles; it depends on vessel speed, maneuverability, sea room, stopping distance, and traffic density. It is defined as that distance at which the action of one vessel alone cannot prevent a collision or where maneuvering space becomes dangerously constrained.

4. Can you alter course to port for an overtaken vessel in restricted visibility under Rule 19(d)?

Yes. Rule 19(d)(i) specifically states "other than for a vessel being overtaken", meaning an alteration to port is permitted if you are safely overtaking her, though starboard alterations remain standard good seamanship.

5. What is the definition and operational obligation of the term "not to impede" (Rule 8(f))?

A vessel required not to impede the passage or safe passage of another must take early action to allow sufficient sea room for the other vessel's safe passage. She remains bound to take action if risk of collision subsequently develops. The vessel whose passage is not to be impeded remains fully obligated to comply with the Rules when the two vessels approach each other with risk of collision.

6. You are drifting in open sea with Main Engine on 15 minutes notice; an outbound vessel is on a collision course. What is your legal status and action?

You are an "underway, not making way" power-driven vessel. You cannot claim Not Under Command (NUC) simply because you chose to drift. If the approaching vessel is on your port side, you are technically the stand-on vessel, but drifting limits maneuvering; sound the wake-up signal (5 short & rapid blasts), call the engine room immediately, call the Master, use VHF DSC/safety channel if needed, and kick the engine/thrusters or make emergency maneuvers to avert collision under Rules 2 and 17(b).

7. Differentiate between "Underway" and "Making Way".

"Underway" means a vessel is not at anchor, made fast to the shore, or aground. "Making way" means the vessel is actively moving through the water under mechanical propulsion, sail, or residual momentum.

8. Detail the day/night signals for a vessel engaged in mineclearance operations.

Lights: In addition to power-driven vessel lights or anchor lights, three all-round green lights (one near the foremast head and one at each fore yardarm end).

Shapes: Three black balls in the same configuration.

Significance: Indicates it is hazardous for other vessels to approach within 1,000 meters.

9. What are the lights and shapes for a vessel constrained by her draught (CBD)?

Lights: Masthead light(s), sidelights, and sternlight for a power-driven vessel underway, plus three all-round red lights in a vertical line where best seen.

Shape: A black cylinder, diameter at least 0.6 m and height twice its diameter.

10. What are the lights for a vessel towing where the tow length exceeds 200 meters?

Three masthead lights in a vertical line forward, sidelights, sternlight, a yellow towing light vertically above the sternlight, and a forward second masthead light if over 50 meters (optional abaft if under 50m).

Shape: A diamond shape where best seen; the towed object also displays a diamond if length exceeds 200m.

11. What lights are displayed by a pushing vessel and a vessel pushed ahead when rigidly connected?

They are exhibited as a single power-driven vessel of their combined length (forward masthead light, after masthead light if ≥50 m, sidelights, and sternlight).

12. What lights are displayed when pushing ahead or towing alongisde if NOT rigidly connected?

Pushing vessel: Two masthead lights in a vertical line forward, sidelights, and two towing lights in a vertical line at the stern (yellow over yellow).

Vessel being pushed: Sidelights at the forward end.

13. State the fog signal for a vessel constrained by her draught at anchor.

One prolonged blast followed by two short blasts at intervals of not more than 2 minutes (Rule 35(g)).

14. What are the mandatory fog signals for a vessel aground?

Rapid ringing of the bell for 5 seconds in the forepart, preceded and followed by three separate and distinct strokes on the bell. If ≥100 meters, followed immediately by rapid sounding of the gong for 5 seconds in the afterpart. May additionally sound an appropriate whistle signal (e.g., Letter 'R': short-prolonged-short) to warn approaching vessels.

15. Describe the sound signals used in a narrow channel when overtaking under Rule 9(e).

Overtaking on starboard: Two prolonged blasts followed by one short blast.

Overtaking on port: Two prolonged blasts followed by two short blasts.

Agreement by overtaken vessel: Prolonged, short, prolonged, short (—⋅ —⋅ ).

16. What does Rule 17(a)(ii) permit, and when does 17(b) trigger?

Rule 17(a)(ii) permits the stand-on vessel to take action by maneuver alone as soon as it becomes apparent that the give-way vessel is not taking appropriate action in compliance with the Rules. Rule 17(b) dictates that when collision cannot be avoided by the action of the give-way vessel alone, the stand-on vessel shall take such action as will best aid to avert collision.

17. Why is an alteration to port forbidden by Rule 17(c) for a stand-on vessel?

Rule 17(c) dictates that a power-driven vessel acting under 17(a)(ii) shall not alter course to port for a vessel on her own port side, because if the give-way vessel belatedly complies with the rules by altering to starboard, an alteration to port by the stand-on vessel creates a head-on collision.

18. What are the day shapes and lights for a dredger indicating an obstructed side?

Lights: Masthead lights, sidelights, sternlight; three all-round lights in a vertical line (Red-White-Red for RAM); on the obstructed side, two all-round red lights; on the clear passing side, two all-round green lights.

Shapes: Two balls vertically (obstructed side); two diamonds vertically (safe passing side); ball-diamond-ball vertically in the center.

19. What lights does a vessel engaged in pilotage duty display underway?

Two all-round lights in a vertical line at or near the masthead: White over Red ("White hat, red face"), along with sidelights and sternlight. When at anchor, white over red plus anchor lights.

20. What is a WIG craft and what special lights does it exhibit?

Wing-In-Ground craft. When taking off, landing, or flying near the surface, it exhibits a high-intensity all-round flashing red light in addition to normal power-driven vessel lights.

21. What constitutes "safe speed" under Rule 6?

A speed such that proper and effective action can be taken to avoid collision and the vessel can be stopped within a distance appropriate to prevailing circumstances and conditions. Evaluated against state of visibility, traffic density, vessel maneuverability/stopping distance, background lighting, wind/sea/current, draft vs depth, and radar capabilities.

22. How do you verify risk of collision if target compass bearing changes slightly (Rule 7(d)(i))?

Appreciable bearing change does not eliminate risk, particularly when approaching a very large vessel, a tow, or when approaching at close range. Verify with radar plotting (ARPA vectors/history), check CPA/TCPA trends, and observe target visual aspect changes.

23. Explain Rule 10(b)(i): How must a vessel join or leave a traffic separation lane?

Join or leave at the termination of the lane. If joining or leaving from either side, do so at as small an angle to the general direction of traffic flow as practicable.

24. When crossing a TSS lane, what angle must be maintained under Rule 10(c)?

Cross on a heading as nearly as practicable at right angles to the general direction of traffic flow (minimizing the time spent in the lane and presenting a clear aspect to lane traffic).

25. When can an Inshore Traffic Zone (ITZ) be used under Rule 10(d)?

Vessels of <20 meters, sailing vessels, and vessels engaged in fishing. Other vessels may use ITZ only when en route to/from a port, offshore installation, pilot station, or to avoid immediate danger.

26. Name five recognized distress signals under COLREG Annex IV.

Guns or explosive signals at 1-min intervals; continuous sound with fog signaling apparatus; rockets/shells throwing red stars; Morse code ...---... (SOS); spoken word "Mayday"; code flags "NC"; square flag having above or below it a ball; flames on vessel; rocket parachute flare or hand flare; orange smoke signal; slowly and repeatedly raising and lowering arms outstretched.

27. What are the vertical positioning requirements for masthead lights under Annex I?

On vessels ≥50m, the forward masthead light must be at least 6 meters above the hull (or the vessel's breadth if >6 meters, but need not exceed 12 m). The after masthead light must be at least 4.5 meters higher than the forward one.

28. What is the horizontal separation of masthead lights on a vessel ≥50 meters?

The horizontal distance between the forward and after masthead lights shall not be less than one half of the vessel's length, but need not be more than 100 meters. The forward light must be placed not more than one quarter of the length from the stem.

29. What is the range of visibility of lights on a vessel ≥50 meters?

Masthead light: 6 nautical miles. Sidelights, sternlight, towing light, and all-round lights: 3 nautical miles.

30. What are the day and night signals for an isolated danger mark?

Shape: Black double sphere vertically arranged.

Light: White light, Flashing (2), Group Flashing (2).

31. You see a single white light ahead at night. What could it be?

Sternlight of a vessel ahead; vessel at anchor under 50m; small craft or sailing vessel under oars/power showing single white light; isolated danger mark/safe water mark (if flashing); distant light on shore.

32. What are the characteristics of a safe water mark?

Red and white vertical stripes; single red sphere topmark. Light: White, Isophase, Occulting, 1 Long Flash every 10 seconds, or Morse code 'A' (⋅ —).

33. What is the light rhythm of an East Cardinal mark?

Very Quick Flashing (3) every 5 seconds, or Quick Flashing (3) every 10 seconds. Topmark: two cones bases together (points away from each other). Colors: Black with single broad horizontal yellow band.

34. What is the light rhythm of a South Cardinal mark?

Very Quick Flashing (6) + Long Flash every 10 seconds, or Quick Flashing (6) + Long Flash every 15 seconds. Topmark: two black cones points downward. Colors: Yellow top, black bottom.

35. What is the light rhythm of a West Cardinal mark?

Very Quick Flashing (9) every 10 seconds, or Quick Flashing (9) every 15 seconds. Topmark: two black cones points together. Colors: Yellow with single horizontal black band.

36. What is the light rhythm of a North Cardinal mark?

Continuous Quick or Very Quick flashing white light. Topmark: two black cones points upward. Colors: Black above yellow.

37. Differentiate IALA Region A vs Region B Lateral marks.

Region A: Port hand marks are red (cylindrical/can); Starboard hand marks are green (conical).

Region B (Americas, Japan, Korea, Philippines): Port hand marks are green (cylindrical/can); Starboard hand marks are red (conical - "Red right returning").

38. What is an Emergency Wreck Marking Buoy?

Used for newly discovered hazards. Equal number of vertical blue and yellow stripes. Topmark: upright yellow cross. Light: alternating blue and yellow light, 1 sec blue, 0.5 sec eclipse, 1 sec yellow, 0.5 sec eclipse (B1s + Y1s).

39. Describe the lights for a vessel engaged in trawling underway.

Two all-round lights in a vertical line: Green over White ("Green gills, white belly"). Sidelights and sternlight when making way through the water. A masthead light abaft and higher is optional if <50 m, mandatory if ≥50 m.

40. Describe the lights for a vessel fishing (other than trawling).

Two all-round lights in a vertical line: Red over White ("Red over white, frying tonight"). If outlying gear extends >150 m horizontally, an all-round white light in the direction of the gear. Sidelights and sternlight when making way.

41. What is the hierarchy rule under Rule 18?

Power-driven vessel underway gives way to: NUC, RAM, CBD (by practice/caution), Fishing, Sailing. Sailing vessel gives way to: NUC, RAM, Fishing. Fishing gives way to: NUC, RAM.

42. Does a sailing vessel have right of way over a power-driven vessel in a narrow channel?

No. Rule 9(b) explicitly dictates that a sailing vessel shall not impede the passage of a vessel which can safely navigate only within a narrow channel or fairway.

43. What action do you take upon hearing a fog signal apparently forward of your beam (Rule 19(e))?

Reduce speed to the minimum at which the vessel can be kept on her course. If necessary, take all way off (stop engines). Navigate with extreme caution until danger of collision is over.

44. What are the sound signals for altering course in sight of one another (Rule 34(a))?

1 short blast: "I am altering my course to starboard."

2 short blasts: "I am altering my course to port."

3 short blasts: "I am operating astern propulsion."

45. What is the sound signal for operating in or near restricted visibility for a power-driven vessel making way?

One prolonged blast at intervals of not more than 2 minutes.

### Section 2: Bridge Watchkeeping, Passage Planning & Navigational Emergencies (Questions 46–85)

46. Name the 4 stages of passage planning under IMO Res. A.893(21).

Appraisal, Planning, Execution, and Monitoring.

47. What key nautical publications must be verified during the Appraisal stage?

Sailing Directions (Pilot books), List of Lights, List of Radio Signals (ALRS 1–6), Tide Tables & Tidal Stream Atlases, Routeing Charts, Notices to Mariners (Annual, Cumulative, Weekly), Navigational Warnings (Navareas), Ocean Passages for the World, and Mariners' Handbook.

48. How do you calculate dynamic squat in shallow water?

Using empirical naval architecture formulas:

Open water: Squat(m)=Cb×V2100

Confined channel: Squat(m)=Cb×V250

where Cb is the block coefficient and V is vessel speed in knots through water.

49. Define Under Keel Clearance (UKC) and dynamic vs static draft.

Static draft is measured at rest in calm water. Dynamic draft includes static draft plus changes due to squat, heel/list, rolling, pitching, heave, and water density (FWA/DWA). UKC is the distance between the lowest point of the ship's hull and the seabed at any given instant.

50. What is a "Wheel Over Point" (WOP) and how is it calculated on a passage plan?

The position where the rudder must be applied to begin swinging the ship onto the new track.

Distance to WOP from vertex: D=R×tanθ2+(V×t), where R is turn radius, θ is course alteration angle, V is speed, and t is rudder execution lag (typically 20–30 seconds).

51. What is parallel indexing (PI) and why is it preferred on coastal passages?

A real-time monitoring technique using radar to ensure the vessel remains on track without latency. An index line is set parallel to the planned track at a distance equal to the cross-index range from a prominent fixed radar target, allowing instant visual identification of cross-track error.

52. What is an Abort Point and a Point of No Return?

Abort Point: The final position on the approach where the vessel can safely turn around or abort without entering constrained waters.

Point of No Return: Beyond this point, the vessel is committed to proceeding through the fairway or channel due to insufficient room to turn, stop, or anchor.

53. How do you handle chart correction for a non-updated paper chart out of date for 2 years?

Obtain the Cumulative Notice to Mariners to identify all weekly notices affecting that chart. Work backward or forward sequentially through the Weekly NTMs using the Chart Correction Log (NP 133A), applying permanent corrections, T&P notices, and Navarea warnings before plotting the passage.

54. What are the mandatory checks carried out prior to departure under SOLAS Ch. V Reg. 26?

Steering gear test (full rudder travel, auxiliary power unit, remote control systems, rudder angle indicator vs actual, emergency power supply switchover), communication between bridge and steering flat, bridge engine controls/telegraph, whistles/fog signals, navigational lights, emergency batteries, and internal communications.

55. How often must emergency steering drills take place under SOLAS?

At least once every 3 months. Must include direct control from within the steering gear compartment, communications procedure, and emergency power supply operation.

56. Gyro compass fails at sea. What are your immediate actions as Chief Mate?

Engage hand steering immediately using the magnetic compass. Verify magnetic compass heading and apply current variation/deviation. Inform Master and Duty Engineer. Switch ECDIS, Radar/ARPA, and Autopilot inputs from Gyro to Magnetic/Course-Over-Ground (or North-Up to Head-Up/Course-Up). Re-check compass errors frequently.

57. What is the operational procedure if ECDIS suffers total black-out?

Switch to the official backup arrangement (dual independent ECDIS or fully updated paper chart portfolio). Verify backup sensor inputs (GNSS, gyro, speed log). If completely deprived of electronic charts, transfer DR/EP positions immediately onto plotting sheets/paper charts, increase bridge watch, alert the Master, evaluate stopping or anchoring if in shallow waters, and broadcast navigational warnings if constrained.

58. Explain the Master-Pilot relationship and Bridge Team Management (BTM) on embarkation.

The pilot serves as an advisor with local knowledge; the Master retains ultimate command and responsibility for safe navigation. A formal Master-Pilot Exchange (MPX) must occur: exchange pilot card, confirm draft/squat, agree on transit plan/tugs/berthing strategy, and verify bridge watch assignments.

59. What are the immediate actions when a man falls overboard on the port side?

Throw lifebuoy with light and smoke marker from the port bridge wing. Put wheel hard over to port (Anderson/Williamson turn to clear the stern away from the victim). Sound 3 prolonged blasts on the whistle. Press MOB button on ECDIS/GNSS. Post extra lookouts. Inform Master and Engine Room. Rig rescue boat. Broadcast urgency/distress message on VHF Ch 16.

60. Differentiate Williamson turn from Scharnow turn.

Williamson turn: Rudder hard over to the casualty side; when heading deviates by 60∘, rudder hard over to the opposite side until within 20∘ of reciprocal, then steady on reciprocal course. Best for delayed-action MOB in low visibility.

Scharnow turn: Rudder hard over, when heading deviates by 240∘, rudder hard over to the opposite side until within 20∘ of reciprocal. Only for immediate action; will take the ship back to a point behind her path.

61. What are the Master's calling circumstances under the SMS?

Failure to sight land or navigation marks when expected; sudden deterioration of visibility; unexpected changes in traffic conditions or close-quarters situations; equipment failures (gyro, radar, steering, main engine); heavy weather exceeding limits; dragging anchor; vessel unable to maintain scheduled track; any doubt regarding navigational safety.

62. Your ship is dragging anchor toward a lee shore in heavy wind. Actions?

Call Master and inform Engine Room to give immediate power. Sound anchor watch alarm to muster crew forward. Start deck machinery and windlass. Use engines and rudder to relieve tension and check swing. Pay out more cable if depth/scope permits; if dragging continues, weigh anchor or slip cable and steam into open water.

63. What is an Anchor Clearing procedure for a "Foul Anchor"?

Secure the vessel. Walk back anchor until visible. Pass a wire or heavy fiber rope under the fouling chain/cable using a boat or messenger. Heave the messenger on a mooring winch to take the weight off the foul wire/cable. Slowly veer the ship's anchor chain to disengage the fluke. Cut or release the foul wire and heave the anchor home.

64. Explain the principle and operation of BNWAS (Bridge Navigational Watch Alarm System).

Monitors bridge awareness. Stage 1: Dormant period (3–12 mins), followed by visual flashing alarm. If not reset within 15 seconds, audible alarm sounds on the bridge. Stage 2: If not reset within 15 seconds, alarm sounds in Master and back-up officer cabins. Stage 3: If not reset within 90–180 seconds, alarm sounds in all crew service areas.

65. What are the primary errors associated with GPS/GNSS at sea?

Ionospheric and tropospheric delays, satellite clock bias, orbital (ephemeris) errors, multipath reflections, receiver noise, and Dilution of Precision (HDOP/PDOP) geometric degradation.

66. What is the difference between Geodetic Datum WGS-84 and local datums?

WGS-84 is a global reference ellipsoid geocentric datum used by GPS. Local datums (e.g., European 1950, Tokyo Datum, Indian Datum) fit the earth's geoid regionally. Plotting a WGS-84 coordinate directly onto a chart referenced to a local datum without applying datum shift corrections leads to positional errors of several hundred meters.

67. How does shallow water affect vessel turning circle and stopping distance?

Turning diameter increases significantly due to increased directional stability and hydrodynamic resistance (water cannot escape easily under the keel). Stopping distance increases because propeller astern efficiency is reduced and vessel displacement pushes a trapped water cushion ahead.

68. Describe the "Bank Effect" (bank suction and bank cushion).

Bank cushion: As the vessel moves close to a steep bank, water piles up between the bow and the bank, creating a high-pressure cushion that forces the bow away toward the center of the channel.

Bank suction: The constricted flow along the quarter creates a localized low-pressure zone (Bernoulli's principle), pulling the stern in toward the bank.

69. How does ship-to-ship interaction occur when overtaking in a narrow fairway?

As the overtaking vessel draws abreast:

- Bows push apart (mutual high-pressure cushions).

- As vessels come alongside, midships attract (low pressure/Bernoulli drop).

- As the overtaking ship pulls ahead, its stern pulls the overtaken ship's bow inward, risking a broadside collision if rudder authority is lost.

70. What is the pivot point of a ship when moving ahead vs moving astern?

Moving ahead: Approximately 13 to 14 of the ship's length from the bow.

Moving astern: Approximately 14 of the ship's length from the stern.

Stopped in water: Roughly near midships.

71. How do you anchor in deep water (depth >50 meters)?

Do not free-fall. Walk out the anchor using the windlass motor until the anchor is within 1 to 12 shackle above the seabed, then gently let go or walk back the cable under control as the vessel drifts slowly astern.

72. What are the SOLAS carriage requirements for magnetic compass and spare magnetic compass?

A properly adjusted standard magnetic compass independent of power supply, a steering magnetic compass (or reflection system), a device for taking bearings over 360∘, and a spare magnetic compass bowl interchangeable with the standard compass.

73. What is the procedure for an accidental DSC false distress alert transmission?

Immediately switch off transmitter. Switch back on and tune to the corresponding distress frequency (VHF Ch 16 for VHF DSC; 2182 kHz for MF; appropriate HF distress frequencies). Broadcast an "All Stations" cancellation message giving ship's name, call sign, MMSI, position, and explicitly stating the distress alert was transmitted in error.

74. What is the GMDSS sea area A3 definition?

An area, excluding sea areas A1 and A2, within the coverage of an Inmarsat geostationary satellite (approximately between 70∘N and 70∘S) in which continuous alerting is available.

75. What are the reserve power (battery) requirements for GMDSS installations?

Must be capable of supplying power for at least:

1 hour if the vessel is equipped with an emergency generator compliant with SOLAS regulations.

6 hours if the vessel is not provided with an emergency generator satisfying statutory requirements.

76. How is an AIS SART identified on an ECDIS / Radar screen?

On Radar: Appears as a line of 12 dots radiating 8 nautical miles outward along its line of bearing.

On ECDIS: Displayed as a red circle with an inscribed cross symbol ("X"), accompanied by the MMSI prefix 970xxxxxx.

77. What is the purpose of the LRIT (Long-Range Identification and Tracking) system?

Enables Contracting Governments to receive tracking data from ships flying their flag, ships bound for their ports, or ships operating within 1,000 nautical miles of their coast. Transmits ship identity, position, and timestamp automatically 4 times per day (every 6 hours).

78. What does VDR (Voyage Data Recorder) record and what is its capsule release mechanism?

Records bridge audio, VHF communications, radar screenshots, ECDIS data, speed, heading, position, engine orders, rudder response, watertight door status, and alarms. Retains 48 hours of continuous data on fixed and float-free capsules. Float-free capsules deploy via a hydrostatic release unit (HRU).

79. How do you test the echo sounder prior to entering port?

Turn on unit, verify draught setting (transducer depth offset vs keel depth), test display illumination, verify phasing/ranging scales, check transmission pulse, verify seabed sounding against known charted depth corrected for tide.

80. What is the difference between Transducer Depth and Keel Depth mode on an Echo Sounder?

Transducer mode: Measures water depth below the hull transducer.

Keel mode (DBK): Automatically subtracts vessel's deepest static draught, indicating true Under Keel Clearance.

81. How do you log a distress message in the GMDSS Radio Log?

Record date, UTC time, frequency/channel used, complete text of distress message, identity of transmitting station, vessel position, nature of distress, and all subsequent communications and actions taken.

82. What is the role of an On-Scene Coordinator (OSC) under IAMSAR Vol. III?

Coordinates search and rescue operations of all participating facilities within the designated search area, designs search patterns, maintains communications, receives SITREPs, and reports to the Joint Rescue Coordination Centre (JRCC).

83. Name three standard IAMSAR search patterns.

Expanding Square Search (SS): Used when target location is known within close limits in calm/moderate seas.

Sector Search (VS): Used when target position is reliably known in a small area.

Parallel Sweep Search (PS): Used for large search areas with multiple search craft.

84. What is a SITREP in SAR operations?

Situation Report sent by the OSC to the RCC at regular intervals containing: situation description, identity of search units, weather conditions, search progress, modifications to plan, and future intentions.

85. What are the requirements for keeping a bridge lookout under STCW Code A-VIII/2?

A dedicated lookout must be maintained at all times by day and night to make a full appraisal of the situation and risk of collision, stranding, and other navigational hazards. The lookout must not be assigned any other duties that interfere with watchkeeping.

### Section 3: Radar, ARPA & Electronic Navigation Aids (Questions 86–120)

86. What is the difference between True Motion and Relative Motion radar display?

True Motion: Own ship and targets move across the screen showing their true course and speed over ground/water. Fixed targets remain stationary.

Relative Motion: Own ship stays at the center (or off-centered); targets move relative to own ship, showing relative motion vectors that directly display CPA and TCPA.

87. Explain why Ground-Stabilized vectors must NOT be used for collision avoidance under COLREGs.

Ground stabilization factors in current and leeway, displaying course and speed over the ground (COG/SOG). COLREGs are governed by the vessel's aspect through the water (heading and speed through water). Using ground stabilization can disguise a target's true aspect, leading to erroneous collision avoidance decisions.

88. What is the difference between X-band (3cm) and S-band (10cm) radar?

X-band (9GHz / 3cm): Superior target discrimination, narrower beam width, excellent range/bearing resolution; triggers SARTs and RACONs. Susceptible to rain and sea clutter attenuation.

S-band (3GHz / 10cm): Longer wavelength penetrates heavy rain, fog, and sea clutter effectively; poor bearing resolution, larger antenna, does not trigger standard radar SARTs.

89. What is Index Error on a radar and how is it corrected?

A range measurement error caused by slight internal timing discrepancies between pulse transmission and receiver sweep initiation. Corrected via internal delay/zero-range calibration adjustments.

90. What causes indirect radar echoes (false echoes)?

Reflection of the radar beam from own ship's deck structures (masts, kingposts, cranes) onto an external target and back to the scanner. The echo appears on the screen along the bearing of the reflecting structure at the target's true range.

91. What is Radar Attenuation?

The reduction in energy of a radar beam as it travels through atmospheric gases, rain, snow, or fog, resulting from absorption and scattering of radio energy.

92. How does pulse length affect range discrimination and detection range?

Short pulse: High range discrimination (distinguishes two close targets on the same bearing); lower average transmitted power, shorter detection range.

Long pulse: Lower range discrimination; higher energy output, superior long-range detection.

93. What is a RACON and how does it appear on the radar display?

A Radar Beacon. When triggered by a ship's 3cm radar transmission, it emits a response signal that appears as a line of Morse code characters radiating radially outward from the beacon's position.

94. What is a RAMARK?

A radar marker that transmits continuously without needing to be triggered by an incoming radar pulse. Appears as a continuous radial line of dots and dashes along its bearing on the radar screen.

95. Explain Target Swapping in ARPA.

Occurs when two targets pass close to each other or when a target passes near a fixed buoy. The tracking gate becomes confused, causing vectors to swap between targets or transfer onto the buoy.

96. What is the minimum number of targets an ARPA must track under IMO Performance Standards?

At least 20 targets simultaneously for automatic acquisition, and 10 targets for manual acquisition (modern systems typically handle 40–100+).

97. What is Trial Maneuver mode in ARPA?

A simulation tool that allows the navigator to test the effect of a course or speed change on all tracked targets before executing the alteration, ensuring no new close-quarters situation is created.

98. Differentiate between ECDIS and ECS.

ECDIS (Electronic Chart Display and Information System): Meets IMO/SOLAS Chapter V carriage requirements, runs official encrypted vector/raster charts (ENC/RNC), and features dual backup redundancy.

ECS (Electronic Chart System): Any navigational aid using electronic charts that does not satisfy statutory SOLAS carriage standards.

99. What are the primary differences between Vector Charts (ENC) and Raster Charts (RNC)?

ENC: Database-driven; objects are layered, queryable, scalable without pixelation, provide automated safety contour alarms, and allow selective display customization.

RNC: Direct digital georeferenced scan of paper charts; cannot be queried, text rotates when chart rotates, zooming causes pixelation, no safety contour alarm functionality.

100. What is an ECDIS Safety Contour vs Safety Depth?

Safety Depth: Applied to spot soundings to highlight shallow depths in bold black text.

Safety Contour: The charted depth contour line separating navigable water from unsafe water, derived from: Draft+RequiredUKC+Squat-HeightofTide. ECDIS rounds up to the next available contour if the exact value is unavailable.

101. What is an Isolated Danger Symbol on ECDIS?

A special magenta hazard symbol displayed over underwater obstructions, wrecks, or rocks with depths equal to or less than the selected safety depth within navigable waters.

102. What is the CATZOC classification in ENCs?

Category of Zone of Confidence: Indicates the accuracy of charted hydrographic data based on position accuracy, depth accuracy, and seafloor coverage (ZOC A1, A2, B, C, D, and U for unassessed).

103. If CATZOC is ZOC B, what does it imply for passage planning?

Position accuracy is within ±50 meters; depth accuracy is within $1.0\text{ m} + 2\%$ of depth. Full seafloor search was not achieved; uncharted features or shoals may exist, requiring increased UKC margins.

104. What is SCAMIN in ECDIS?

Scale Minimum: An attribute assigned to chart objects that hides them when zooming out beyond a predetermined display scale, preventing screen clutter.

105. What is the dual-fuel concept in ECDIS?

The capability of an ECDIS to operate in Raster Chart Display System (RCDS) mode using RNCs in geographical areas where official ENCs are not yet published, provided an updated paper chart folio is maintained onboard.

106. What alarms must an ECDIS sound automatically under IMO standards?

Crossing safety contour; deviation from track line (XTD); positioning system failure; approach to critical point/danger area; datum mismatch; malfunction of ECDIS.

107. Explain the Doppler Speed Log principle.

Transmits high-frequency acoustic beams into the water; the frequency shift between transmitted and backscattered signals (Δf) is directly proportional to vessel velocity relative to the seabed (bottom track) or water column (water track).

108. What is Janus configuration in a Doppler log?

Transmitting four acoustic beams angled forward, aft, port, and starboard (60∘ to horizontal). Comparing forward and aft frequencies cancels out vertical vessel pitch motions, providing accurate longitudinal speed.

109. What is the difference between Speed Through Water (STW) and Speed Over Ground (SOG)?

STW measures speed relative to the moving water body (used for collision avoidance and engine slip). SOG measures speed relative to the earth's surface (used for navigation and passage monitoring).

110. How does a Speed Log calculate speed when water depth exceeds 200 meters?

Switches automatically from bottom track to water track mode, measuring speed relative to a water layer 10–30 meters beneath the hull.

111. What causes sea clutter saturation on radar and how do you optimize it?

Caused by radar reflections from wave crests near the vessel. Suppressed by adjusting the Sensitivity Time Control (STC / Sea Clutter) control, which reduces receiver gain at close range while leaving far-range amplification unaffected.

112. What causes rain clutter and how does the FTC control mitigate it?

Caused by backscatter from raindrops over a wide area. Suppressed by the Fast Time Constant (FTC / Rain Clutter) differentiator, which filters out long, continuous return signals while preserving sharp leading edges of vessel targets.

113. What is the maximum display jitter allowed on ARPA?

Target vector jitter should not exceed 0.5∘ in direction or 0.2knots in speed after steady tracking.

114. Explain Blind Sectors and Shadow Sectors on radar.

Blind sectors: Areas where no radar returns are possible due to direct physical obstruction by structures (funnels, masts).

Shadow sectors: Areas where target return strength is significantly attenuated by obstructions.

115. How do you detect an iceberg on radar during night or poor visibility?

Icebergs are poor radar reflectors (absorptive shape, smooth surfaces, low dielectric constant). Detection range is typically less than half that of a ship. Monitor sea clutter closely, look for thermal radar blind spots, tune gain carefully, post forward lookouts, and monitor seawater temperature drops.

116. What is the performance standard for an automatic tracking aid (ATA)?

Must be capable of tracking, processing, and displaying information on at least 10 targets simultaneously with manual or automatic acquisition.

117. What are the dangers of Over-Reliance on ECDIS?

Loss of situational awareness, "screen fixation", failure to maintain a proper visual lookout, blind faith in automated routes, incorrect safety contour settings, and ignoring positioning sensor discrepancies.

118. What is a Look-Ahead Vector / Safety Frame on ECDIS?

A user-defined monitoring zone extending ahead of the vessel (in time and width) that interrogates the ENC database to trigger alarms before the vessel encounters shallow water, dangers, or restricted areas.

119. What is the consequence of entering an incorrect antenna offset in GPS?

The displayed position corresponds to the physical antenna, not the conning station. Incorrect offsets introduce errors in ECDIS display positioning and radar alignment during critical maneuvers.

120. How do you manually fix a position on ECDIS?

Use Line of Position (LOP) feature: enter simultaneous visual compass bearings and/or radar ranges from charted terrestrial objects. ECDIS computes and plots the intersection EP/Fix.

### Section 4: Magnetic & Gyro Compasses (Questions 121–145)

121. Explain the fundamental principles of a Gyro Compass (Inertia & Precession).

Gyroscopic Inertia: The rigidity in space of a spinning mass to maintain its axis pointing in a fixed direction relative to universal space.

Precession: When a torque is applied to alter the spin axis, the axis moves in a direction at right angles to the applied torque.

122. How is a free gyroscope made meridian-seeking (North-seeking)?

By applying a gravity-controlled torque (mercury ballistic, top-weighted or bottom-weighted liquid pots, or pendulum control) that precesses the spinning axis horizontally toward the meridian as the earth rotates.

123. What is Latitude Error (Speed/Latitude Error) in a Gyro Compass?

Caused by the interaction between vessel velocity over the curved earth and the earth's tangential rotation speed. The apparent rotation vector shifts, moving the settling point away from true North. Compensated by setting ship's speed and latitude into the gyro control panel.

124. What is Ballistic Deflection in a Gyro Compass?

When a ship accelerates, decelerates, or alters course, the inertia of the mercury or pendular control system exerts an unwanted torque on the gyro axle, causing a temporary oscillation error.

125. Why does a Gyro Compass become unreliable in polar regions?

The horizontal component of the earth's angular velocity (ΩcosLat) approaches zero as latitude approaches 90∘, eliminating the directive force necessary for the compass to settle on the meridian.

126. What causes Heeling Error in a magnetic compass?

When the ship rolls or heels, symmetrical vertical and horizontal soft/hard iron components shift relative to the compass needle, creating a temporary magnetic field that drags the needle off the meridian.

127. How is Heeling Error corrected?

By placing vertical permanent Heeling Error magnets (green end up in North magnetic latitude, red end up in South magnetic latitude) inside the central bucket directly below the compass gimbal.

128. What are Flinders Bar and Quadrantal Correctors?

Flinders Bar: A soft iron vertical cylinder forward or aft of the binnacle to correct coefficient B caused by vertical soft iron.

Quadrantal Spheres (Kelvin’s Balls): Soft iron spheres on the port and starboard arms to correct coefficient D caused by symmetrical horizontal soft iron.

129. Explain Coefficients A, B, C, D, and E of Magnetic Compass Deviation.

Coefficient A: Constant deviation on all headings (compass mechanical misalignment or horizontal unsymmetrical iron).

Coefficient B: Semicircular deviation proportional to sin(Heading), corrected by fore-and-aft magnets and Flinders bar.

Coefficient C: Semicircular deviation proportional to cos(Heading), corrected by athwartship magnets.

Coefficient D: Quadrantal deviation proportional to sin(2θ), corrected by soft iron quadrantal spheres.

Coefficient E: Quadrantal deviation proportional to cos(2θ), corrected by slewing quadrantal spheres.

130. How do you prepare a Magnetic Compass Deviation Card at sea?

"Swing ship": Steer the vessel onto 8 cardinal and three-letter compass headings (000∘,045∘,090∘, etc.) in calm waters. On each heading, take simultaneous celestial bearings (sun amplitude/azimuth) or compare against an accurate gyro heading to establish magnetic deviation, then plot the deviation curve.

131. What is an Isogonic Line and an Agonic Line?

Isogonic Line: Line joining places of equal magnetic variation on a chart.

Agonic Line: Line joining places of zero magnetic variation.

132. What is an Isoclinic Line and an Aclinic Line?

Isoclinic Line: Line joining places of equal magnetic dip.

Aclinic Line (Magnetic Equator): Line where magnetic dip is zero (compass needle balances completely horizontally).

133. What is Sub-permanent Magnetism in ship construction?

Magnetism retained in the ship's hard iron structure acquired through hammering, vibration, and mechanical stress during construction in the Earth's magnetic field. Decreases slowly over time.

134. What is Temporary Magnetism?

Magnetism induced in the vessel's soft iron parts by the prevailing Earth's magnetic field, changing instantaneously with ship's heading and geographic location.

135. What maintenance is required for a Magnetic Compass?

Check for bubbles in the bowl; verify gimbal freedom and lubricate pivots; check optical periscope mirrors and cleaning; test illumination dimmer; check condition of Flinders bar and correcting magnets; test azimuth mirror for prism defects.

136. How do you remove a bubble from a magnetic compass bowl?

Invert the bowl; unscrew the filling plug on the side/base; inject a mixture of distilled water and pure ethyl alcohol (or pure glycerin/distilled water mix) using a syringe until all air is expelled; screw the plug tightly with a new washer; upright the bowl and verify.

137. How do you calculate Compass Error using Sun Azimuth?

- Record UT time, DR position, and compass bearing of Sun.

- From Nautical Almanac, extract GHA and Declination.

- Calculate LHA (LHA=GHA±Long).

- Compute True Azimuth (Z) using ABC tables or sight reduction formula: tanZ=1cosLat×tanLHA.

- Compare True Bearing with Compass Bearing to obtain total Compass Error.

138. What is the advantage of using Sun Amplitude over Azimuth?

Amplitude calculation is faster and simpler (sinAmp=sinDeccosLat) and can be taken precisely when the sun's lower limb is half a diameter above the horizon (theoretical sunrise/sunset).

139. Why should you avoid taking Sun Amplitude when the sun is higher in the sky?

Atmospheric refraction near the horizon introduces bearing errors, and the amplitude formula is strictly valid only when the center of the celestial body is on the true celestial horizon.

140. What is Gyro Drift vs Gyro Transport Precession?

Drift: Apparent horizontal motion of a free gyro axis caused by the earth's rotation.

Transport Precession: Apparent motion of the gyro axis caused by the physical movement of the vessel over the spherical earth.

141. What is an Optical Azimuth Circle and how is it tested for index error?

An instrument mounted on the compass bowl with an arrow prism and sight vanes. Tested by observing a distant terrestrial object or celestial body direct and inverted, verifying that bearing readings match within 0.1∘.

142. What causes the compass needle to "stick" or become sluggish?

Worn sapphire cap or damaged iridium pivot point; loss of liquid in the bowl; magnetic needle losing its magnetic moment.

143. What is the Retentive Error of a magnetic compass?

Temporary deviation that persists after a ship has been steering on the same heading for a long period, caused by soft iron sluggishly retaining induced magnetism.

144. How does lightning strike affect a vessel's magnetic compass?

Induces massive, unpredictable permanent magnetism in the hull steel, resulting in huge compass errors and necessitating total re-swinging and readjustment of all correcting magnets.

145. What is the maximum acceptable deviation on any heading?

Standard maritime practice requires deviation to be kept within ±3∘ on all headings after proper adjustment.

### Section 5: Meteorology & Oceanography (Questions 146–175)

146. Explain Buys Ballot’s Law.

In the Northern Hemisphere, face the true wind and the low-pressure center will be on your right hand, slightly behind you (90∘ to 135∘). In the Southern Hemisphere, face the true wind and the low-pressure center will be on your left hand.

147. What are the favorable conditions for the formation of a Tropical Revolving Storm (TRS)?

Warm ocean temperature $\ge 26.5^\circ\text{C}$ down to a depth of 50 m; distance at least 5∘ away from the equator (sufficient Coriolis force); high humidity in the troposphere; low vertical wind shear; pre-existing low-level atmospheric disturbance.

148. Why do TRSs not form at the equator?

The Coriolis parameter (f=2Ωsinϕ) is zero at the equator. Without Coriolis force, air rushes directly into the low-pressure core to equalize pressure rather than setting up cyclonic vorticity.

149. Differentiate between Dangerous Semicircle and Navigable Semicircle in a TRS.

Dangerous Semicircle (Right-hand in NH, Left-hand in SH): Wind speeds are reinforced by storm forward speed; winds tend to blow a vessel across the storm track directly into the path of the center.

Navigable Semicircle: Winds blow away from the path of the storm center, and net wind speeds are reduced by storm translation speed.

150. What is the evasive action for a vessel in the Dangerous Semicircle of a TRS (Northern Hemisphere)?

Proceed at maximum safe speed with the wind on the starboard bow (045∘ to 060∘ relative), altering course to starboard as the wind veers to haul away from the path.

151. What is the evasive action for a vessel in the Navigable Semicircle of a TRS (Northern Hemisphere)?

Bring the wind onto the starboard quarter (135∘ relative), maintaining this relative bearing while making maximum safe speed away from the center.

152. What is the evasive action if your vessel is directly on the path of an advancing TRS?

Northern Hemisphere: Put the wind on the starboard quarter (135∘ relative) and run for the navigable semicircle. Southern Hemisphere: Put the wind on the port quarter (045∘ relative).

153. What is an Isallobar?

A line drawn on a meteorological synoptic chart joining places of equal atmospheric pressure tendency (equal pressure change over the preceding 3 hours).

154. What is Barometric Tendency and what drop indicates an approaching TRS?

The net change in barometric pressure over a 3-hour period, corrected for diurnal variation. A drop of more than 3hPa below normal diurnal pressure indicates an approaching tropical storm.

155. Explain Diurnal Variation of atmospheric pressure.

A daily atmospheric tide causing barometric pressure to rise and fall twice every 24 hours. Maxima occur at 1000 and 2200 local time; minima occur at 0400 and 1600 local time (amplitude ≈3hPa in the tropics).

156. Describe the weather sequence during the passage of a Cold Front.

Prior: Warm, backing winds, pressure falling, cirrus turning to altostratus, light rain.

Passage: Sudden pressure surge, sharp wind veer with violent squalls, heavy rain/thunderstorms, rapid temperature drop.

After: Cumulus clouds, clearing visibility, cold air, pressure rising steadily.

157. What is Advection Fog and how does it form?

Forms when warm, moist air blows horizontally over a colder sea surface, cooling the lower air layer below its dew point and condensing water vapor into fog. Common in the Grand Banks and North Pacific.

158. What is Radiation Fog?

Forms over land on clear, calm nights when the ground radiates heat into space, cooling the overlying air below its dew point. Can drift into river estuaries and ports in morning hours.

159. What information is obtained from an Ocean Routeing Chart?

Monthly publication showing: wind roses (prevailing wind direction, force, frequency), ocean currents, tropical cyclone tracks, limits of sea ice and icebergs, load line zones, recommended shipping tracks, dew point temperatures, and fog frequency.

160. How do you interpret a Wind Rose on a routeing chart?

Arrows show wind direction flying with the wind toward the center circle. Shaft length indicates frequency; feathering/thickness indicates Beaufort force; figure inside the circle indicates percentage of calms/light airs.

161. Differentiate between Climatological Routeing and Weather Routeing.

Climatological: Based on historical statistical averages (routeing charts, Ocean Passages for the World).

Weather Routeing: Dynamic real-time tactical passage optimization using medium-range atmospheric forecast models and vessel performance curves.

162. What is the Agulhas Current and why is it dangerous?

A fast-flowing warm western boundary current flowing southwestward along the southeast coast of South Africa. When strong southwesterly gale winds oppose this current, severe, steep "freak" or rogue waves exceeding 20 meters are generated.

163. What are the major cold ocean currents of the world?

Labrador Current, Benguela Current, California Current, Oyashio (Kuril) Current, Peru (Humboldt) Current, and Falkland Current.

164. What causes an El Niño phenomenon?

Weakening or reversal of the easterly Trade Winds across the equatorial Pacific, allowing warm surface water to surge eastward toward the South American coast, disrupting global weather patterns and suppressing upwelling.

165. What is Geostrophic Wind?

A theoretical wind resulting from an exact balance between the horizontal pressure gradient force and the Coriolis force, blowing parallel to straight isobars without friction.

166. What is the difference between Sea waves and Swell waves?

Sea: Waves generated locally by prevailing winds, short-crested, steep, and irregular.

Swell: Waves that have traveled out of their generating area, characterized by long, uniform, rounded crests and regular periods.

167. What are the signs of an approaching Tropical Revolving Storm at sea?

Abnormal swell from storm center; unusual atmospheric clarity followed by cirrus veil ("halo" around sun/moon); uncharacteristic drop in barometer exceeding diurnal variation; cessation of trade winds; brilliant sunset colors.

168. How is the 1-2-3 Rule used in hurricane avoidance?

IMO/USCG danger area avoidance rule: For a 24-hr forecast, add 35NM error radius; for 48-hr, add 70NM; for 72-hr, add 100NM. Avoid navigating within the expanding cone of danger.

169. What is Katabatic Wind?

A gravity-driven wind blowing down a mountain slope, plateau, or glacier when dense, cold air spills downward (e.g., Bora in the Adriatic, Mistral in the Mediterranean, Williwaw in Alaska).

170. What is an Anemometer and how is True Wind calculated?

Measures apparent wind speed and direction. True wind is calculated by vector subtraction of vessel's course and speed over water from the apparent wind vector.

171. What is an aneroid barometer vs barograph?

Aneroid barometer: Uses an evacuated metal capsule that expands/contracts with pressure changes, providing instantaneous dial readings.

Barograph: Connects multiple evacuated capsules to an inked arm recording continuous pressure trace on a clockwork drum.

172. What is dew point temperature?

The temperature to which air must be cooled at constant pressure to become saturated with water vapor, causing condensation.

173. What is the significance of the difference between sea surface temperature and dew point?

If sea surface temperature is lower than the dew point of the air passing over it, advection fog will form.

174. What is a Grib file?

General Regularly-distributed Information in Binary form: A standardized, highly compressed computer-readable data format used by weather services to transmit atmospheric and oceanic forecasts to ships.

175. What is the Intertropical Convergence Zone (ITCZ / Doldrums)?

A low-pressure belt around the equator where the northeast and southeast trade winds converge, characterized by calm winds, oppressive humidity, squalls, and severe convective thunderstorms.

### Section 6: Celestial Navigation & Tidal Theory (Questions 176–200)

176. Explain why "Long by Chron" sight is taken in the morning and "Meridian Sight" at noon.

In the morning, the Sun is near the Prime Vertical (090∘), making the resulting position line run North-South, giving accurate Longitude. At noon, the Sun crosses the observer's meridian (180∘ or 000∘), giving a position line running East-West, yielding precise Latitude.

177. What is an Ex-Meridian sight and when is it taken?

A sight taken when the Sun is slightly off the meridian (within 20–30 minutes before or after transit) because clouds obscured the Sun at exact meridian passage. Corrected to determine noon latitude using Brent's tables.

178. What are the index errors of a Sextant and how are they corrected?

Perpendicularity error: Index mirror not perpendicular to the frame (corrected via first screw behind index mirror).

Side error: Horizon glass not perpendicular to the frame (corrected via top screw behind horizon glass).

Index error: Horizon glass not parallel to index mirror at zero index (corrected via bottom screw behind horizon glass).

Collimation error: Telescope axis not parallel to plane of the arc (corrected via telescope collar screws).

179. Explain how to determine Sextant Index Error "On the Arc" vs "Off the Arc".

Align the true horizon with reflected horizon. If zero on micrometer is above 0∘, it is "on the arc" (subtract error). If zero is below 0∘, it is "off the arc" (add error). Rule: "If it's on the arc, it's off; if it's off the arc, it's on."

180. What is Dip in celestial sight reduction?

The angle between the true horizontal plane through the observer's eye and the apparent visible sea horizon, caused by the observer's height of eye above sea level. Always subtractive: Dip≈0.97×Heightofeye(m).

181. What is Semi-Diameter (SD) and why is it applied to Sun and Moon sights?

The angular radius of the celestial body measured from its center to its limb. Sights are taken on the upper or lower limb, so the semi-diameter must be added (for lower limb) or subtracted (for upper limb) to reduce the observation to the center of the body.

182. What is Augmentation of the Moon?

The apparent increase in the semi-diameter of the Moon as its altitude increases, because the observer is closer to the Moon when it is overhead than when it is on the horizon by up to one Earth radius.

183. What is Parallax in Altitude (PA)?

The angular difference between the position of a body observed from the Earth's surface and the position it would have if observed from the Earth's center. Significant for the Moon due to its proximity to Earth.

184. Why is Polaris (North Star) sight valuable and how is latitude determined?

Polaris is located within 1∘ of the True Celestial North Pole. Its true altitude corrected for refraction and dip, modified by the three a0,a1,a2 corrections from the Nautical Almanac, gives the observer's latitude directly:

Latitude=TrueAltitude-1∘+a0+a1+a2.

185. What is the Intercept Method (Marcq St. Hilaire Method)?

Calculates theoretical zenith distance/altitude (Hc) for an assumed position and compares it against true observed altitude (Ho). Intercept: a=Ho-Hc. If Ho>Hc, plot intercept toward the body's azimuth; if Ho<Hc, plot away.

186. Explain Great Circle Sailing vs Rhumb Line Sailing.

Great Circle: The shortest distance between two points on a sphere, but course changes continuously.

Rhumb Line: A line crossing all meridians at the same angle (constant heading), but longer distance. In practice, a Great Circle route is broken into multiple successive Rhumb Line waypoints.

187. What is Composite Great Circle Sailing?

Used when a pure Great Circle track reaches excessively high latitudes containing sea ice, heavy weather, or land masses. The route follows a Great Circle to a limiting parallel of latitude, runs along that latitude parallel, and resumes another Great Circle track to the destination.

188. What is the Vertex of a Great Circle track?

The point on the Great Circle path that reaches the highest latitude, where the track heading is exactly East-West (090∘ or 270∘).

189. What is Spring Tide vs Neap Tide?

Spring Tide: Occurs at New Moon and Full Moon (Syzygy) when Sun and Moon gravitational forces reinforce each other; highest high waters, lowest low waters, maximum tidal range.

Neap Tide: Occurs at 1st and 3rd Quarters (Quadrature) when Sun and Moon pull at right angles; lowest high waters, highest low waters, minimal tidal range.

190. What is Chart Datum (CD)?

A horizontal reference plane to which soundings on a navigational chart and tidal heights are referred. Almost universally defined as Lowest Astronomical Tide (LAT).

191. What is Lowest Astronomical Tide (LAT)?

The lowest water level that can be predicted to occur under average meteorological conditions and under any combination of astronomical conditions over an 18.6-year tidal cycle.

192. What is Shallow Water Effect on Tides?

As tidal waves advance into shallow coastal waters, friction and seabed contours distort the symmetrical wave profile, generating higher-order harmonics (quarter-diurnal and sixth-diurnal tides) that produce double high waters or prolonged stand times (e.g., Southampton).

193. How do barometric pressure and wind affect tidal heights?

A 1hPa rise in atmospheric pressure lowers sea level by approximately 1cm (inverted barometer effect). Strong onshore winds push water coastal-ward (positive surge), while persistent offshore winds lower tidal heights below predictions (negative surge).

194. What is Tidal Stream vs Tidal Current?

Tidal Stream: Periodic horizontal flow of water caused purely by astronomical gravitational forces of the Moon and Sun.

Tidal Current: The combined horizontal flow of water resulting from both astronomical tidal streams and meteorological factors (wind drift, river runoff).

195. How do you calculate height of tide at an intermediate time using Admiralty Tide Tables?

Extract times and heights of high and low water; calculate tidal range and duration; apply the standard tidal curve for the port by plotting the duration, HW time, and water range to find height at the desired time.

196. What is the Rule of Twelfths for semi-diurnal tides?

An empirical rule for tidal height change over 6 hours:

1st hour: 112 of range

2nd hour: 212 of range

3rd hour: 312 of range

4th hour: 312 of range

5th hour: 212 of range

6th hour: 112 of range

197. What is a Secondary Port and how are its tides determined?

A port for which full daily tidal predictions are not published. Tides are calculated by applying time and height differences to the predictions of a designated Standard Port from the Admiralty Tide Tables.

198. What is the Cotidal Chart?

A chart showing co-tidal lines (connecting points experiencing high water simultaneously) and co-range lines (connecting points having equal tidal ranges).

199. What is an Amphidromic Point?

A point in the ocean where there is zero tidal range (no rise and fall of tide). Cotidal lines radiate outward from this nodal point, and the tidal wave rotates around it (counter-clockwise in NH, clockwise in SH).

200. What is the significance of the 18.6-year Tidal Node Cycle?

The period required for the regression of the Moon's orbital nodes around the ecliptic, completing a full cycle of variation in the Moon's maximum declination (18.3∘ to 28.7∘) and defining the baseline period for calculating Astronomical Tides.

# Chief Mate Oral – Function 2 (Cargo Handling & Stowage): 200 Q&A

Study guide for Indian MMD Chief Mate (FG) oral examination. Answers are short, exam-style points. Always verify against the latest editions of SOLAS, IMSBC, IMDG, CSS, MARPOL, ISGOTT and your company SMS.

## Section 1: Cargo Planning and General Cargo Concepts

Q1. What is a cargo plan (stowage plan)? A plan showing the disposition of cargo by port, hold/tank, and weight, prepared by the Chief Officer before loading. It considers stability, strength, trim, segregation and discharge sequence.

Q2. What factors do you consider while planning cargo? Stability, longitudinal strength, trim and drafts, port rotation, hold capacity, deck strength, segregation and compatibility, cargo properties, cargo gear, weather, and port or terminal restrictions.

Q3. What is stowage factor? The volume occupied by one tonne of cargo (m³/tonne), usually excluding broken stowage unless stated.

Q4. What is broken stowage? Lost space between cargo and ship's structure or between irregularly shaped packages. It is expressed as a percentage of the space used.

Q5. How do you calculate the space required for a cargo? Space = weight × stowage factor, plus an allowance for broken stowage.

Q6. What are the purposes of dunnage? To protect cargo from sweat and contact with steel, allow air circulation and drainage, increase friction, and separate cargoes.

Q7. Differentiate ship's sweat and cargo sweat. Ship's sweat occurs when warm humid air in the hold meets cold steel, typically on a voyage from warm to cold regions. Cargo sweat occurs when warm humid air meets cold cargo, typically on a voyage from cold to warm regions.

Q8. What is the ventilation rule for preventing sweat? Ventilate when the dew point of the outside air is lower than the dew point of the hold air. Do not ventilate if it is higher. Cargo temperature is also compared with the outside dew point.

Q9. What is segregation? Separating incompatible cargoes so that leakage, vapour, dust or reaction cannot damage or endanger the other cargo, the ship or the crew.

Q10. How are hatch covers tested for weathertightness? By hose test, ultrasonic test, or chalk test. Check the rubber packing, compression bars, cleats, drains and coamings.

Q11. What are the three functions of a Bill of Lading? Receipt for goods shipped, evidence of the contract of carriage, and a document of title.

Q12. What is a clean B/L and a claused B/L? A clean B/L shows goods received in apparent good order and condition. A claused B/L carries remarks about damage or deficiency.

Q13. What is a Mate's Receipt? A receipt issued by the ship on receiving cargo, recording its apparent condition. The B/L is issued against it.

Q14. Should a Master accept a Letter of Indemnity for damaged cargo? No. Issuing a clean B/L against LOI for known damage can be fraudulent and may be uninsured by the P&I Club. Consult the Club and the company.

Q15. What is a Notice of Readiness (NOR)? Notice from the Master or agent that the ship is ready in all respects to load or discharge, starting laytime according to the charter party.

Q16. What is a Statement of Facts? A chronological record of port events (arrival, NOR, berthing, commencement and completion, delays, weather), signed by the parties and used for laytime calculation.

Q17. What is a cargo manifest? A document listing all cargo on board, with B/L numbers, marks, quantity, shippers and consignees. It is used for customs clearance.

Q18. What is dead freight? Freight payable by the charterer for space booked but not used.

Q19. What are demurrage and despatch? Demurrage is payment by the charterer for time beyond allowed laytime. Despatch is a reward to the charterer for time saved.

Q20. What is General Average? A voluntary, reasonable sacrifice or expenditure made in time of peril for the common safety of ship and cargo. All parties share the loss proportionally under the York-Antwerp Rules.

Q21. What is Particular Average? Accidental partial loss or damage borne by the owner of the damaged interest alone.

Q22. What is a Note of Protest? A statement by the Master, made before a notary or consul, recording events (heavy weather, damage) to protect the owner against later claims.

Q23. Outline a draught survey procedure. Read drafts at six points, correct for trim, hog/sag, density and ballast/fresh water/bunkers/stores. Determine displacement and subtract light ship and constants to get cargo weight. Do an initial and a final survey.

Q24. What is a Cargo Securing Manual (CSM)? A ship-specific manual, approved by the Administration under SOLAS VI/5.6, describing cargo securing arrangements and procedures. It is mandatory for ships carrying cargo other than solid and liquid bulk.

Q25. What is the Maximum Securing Load (MSL)? The allowable load capacity for a device used to secure cargo to a ship (the MSL is used in lashing calculations, with a safety factor).

## Section 2: Cargo Gear and Lifting Appliances

Q26. Define SWL. Safe Working Load is the maximum load a lifting appliance or loose gear may lift in specified conditions.

Q27. What are the proof load requirements for cranes and derricks? For SWL up to 20 t: SWL × 1.25. Above 20 t up to 50 t: SWL + 5 t. Above 50 t: SWL × 1.1.

Q28. How often must cargo gear be tested and examined? Thorough examination and test after any major repair or alteration, thorough examination at least once every 12 months, and re-test at least every 5 years. Entries are made in the Register of Lifting Appliances and Cargo Handling Gear.

Q29. What is the Register of Lifting Appliances? The record containing certificates of test and examination, annealing records for chains and rings, and details of repairs. It is also called the Chain Register.

Q30. What is union purchase? Two derricks rigged, one over the hatch and one over the side, with their runners joined at a common hook so the load is moved between them.

Q31. What is the maximum angle between runners in union purchase? Keep it below 90° where possible, and never more than 120°. A wider angle greatly increases the load on the runners.

Q32. Name the parts of a derrick rig. Boom, topping lift, cargo runner, guys, preventers, heel block, span block, and gooseneck.

Q33. What is the purpose of guys and preventers? They hold the derrick boom in position athwartships (and fore and aft) against the load, and prevent it from swinging.

Q34. What are the discard criteria for wire ropes? Excessive broken wires in a given length, reduction in diameter beyond limits (about 10%), corrosion, kinking, birdcaging, core protrusion, and heat damage.

Q35. What are the pre-use checks on a ship's crane? Visual check of wires and sheaves, limit switches, brakes, emergency stop, alarms, hydraulic oil levels, load indicator, and lubrication. Test with no load.

Q36. What safety devices does a deck crane have? Overload cutout (SWL indicator), limit switches for hoisting, luffing and slewing, brakes, emergency stop, and warning alarm.

Q37. What are the precautions when using ship's cranes in port? Check weather limits, use trained operators, clear communication with signalman, no loads over people, keep within SWL and radius, and check the ship's list during operations.

Q38. How do you determine the SWL of a wire sling? From the manufacturer's certified breaking load divided by the safety factor, multiplied by a reduction factor for the sling angle.

Q39. What happens to the load on each leg of a sling as the angle between legs increases? The tension in each leg increases. At 120° the tension in each leg equals the load.

Q40. What is meant by the term 'Heavy Lift'? A lift above the SWL of the normal ship's gear, usually handled by a heavy-lift derrick or crane with specially calculated rigging, stability and ballasting.

## Section 3: Solid Bulk Cargoes (IMSBC Code)

Q41. What is the IMSBC Code? The International Maritime Solid Bulk Cargoes Code, mandatory under SOLAS Chapter VI, giving safe stowage and shipment requirements for solid bulk cargoes.

Q42. What are Group A, B and C cargoes? Group A: cargoes which may liquefy. Group B: cargoes with a chemical hazard. Group C: cargoes which neither liquefy nor have chemical hazards.

Q43. What is Transportable Moisture Limit (TML)? The maximum moisture content of a Group A cargo considered safe for carriage in ordinary cargo ships.

Q44. What is the Flow Moisture Point? The percentage moisture content at which a flow state develops. TML is 90% of the flow moisture point.

Q45. What information must the shipper provide? Cargo name, group, properties, stowage factor, moisture content certificate, TML certificate, and any hazards, in the shipper's declaration.

Q46. What is liquefaction? Loss of shear strength of a cargo with moisture above TML, so it behaves like a fluid. It causes free surface, cargo shift, loss of stability and capsize.

Q47. How do you check for liquefaction risk on board? Can test: fill a can about 3/4 full, strike it against a hard surface about 25 times, and look for free moisture or a fluid state. Refuse loading if the cargo shows it.

Q48. Name some cargoes that may liquefy. Nickel ore, iron ore fines, mineral concentrates, and some bauxite and coal fines.

Q49. How may Group A cargoes above TML be carried? Only in specially constructed or fitted cargo ships, as per the Code.

Q50. Why is trimming of bulk cargoes needed? To reduce the risk of cargo shifting, to make the surface level, to ensure stability, and to limit voids and fire risk.

Q51. What is angle of repose? The steepest angle at which the cargo remains stable when piled. A low angle means a free-flowing cargo with a high shift risk.

Q52. What are cohesive and non-cohesive cargoes? Cohesive cargoes (like some concentrates) hold together and may form a rigid pile. Non-cohesive cargoes (such as grain) flow readily.

Q53. What is the BLU Code? The Code of Practice for the Safe Loading and Unloading of Bulk Carriers, with the BLU Manual and Ship/Shore Safety Checklist.

Q54. What should the agreed loading plan contain? Sequence of loading and discharge, tonnages per hold per pass, rates, deballasting plan, draft checks, and stress and stability values.

Q55. Why is hull stress monitoring important in bulk carriers? To keep shear force and bending moment within permissible limits at every stage of loading and discharging, preventing structural damage.

Q56. What is block loading and alternate hold loading? Block loading means loading adjacent holds with heavy cargo. Alternate loading means loading every other hold. Both place high stress on the hull and need checking against the loading manual.

Q57. What are SOLAS XII requirements for bulk carriers? Additional structural strength and survivability requirements, including hold-flooding provisions, water level detectors in holds and forepeak, and loading instruments.

Q58. What are hold water-ingress alarms? Alarms at the forward hold bilges and cargo holds, giving warning of flooding so the crew can react.

Q59. What is the hazard of Group B cargoes? Chemical hazards: toxicity, corrosiveness, self-heating, flammable gas emission, oxygen depletion.

Q60. What are the hazards of coal? Methane emission, spontaneous heating, oxygen depletion, carbon monoxide, and corrosion of steel.

Q61. How is coal carried? Surface ventilation only. Measure methane, CO and temperature without entering the hold. No smoking, no naked lights, no hot work, and keep holds closed otherwise.

Q62. What action is taken for a coal fire? Close all hatches and ventilation, exclude air, and use CO2 or inert gas. Do not use water in the early stage, and consider heading for a port of refuge.

Q63. What is DRI? Direct Reduced Iron. It may evolve hydrogen and heat when wet, so it must be kept dry and carried per the specific schedule, often under inert gas.

Q64. What are the hazards of seed cake and fishmeal? Self-heating and fire, plus oxygen depletion. Fishmeal needs an antioxidant treatment certificate.

Q65. What is oxygen depletion in cargo holds? Cargoes (coal, scrap, ores, timber) can absorb oxygen and make the hold atmosphere unsafe. Test before entry; normal oxygen content is 21%.

Q66. What are the precautions before entering a cargo hold? Entry permit, ventilation, atmosphere testing (O2, toxic and flammable gases), standby person, rescue equipment and communications.

Q67. What is the procedure when fumigating cargo? Seal the hold, post warning notices, keep crew out of affected spaces, follow IMO recommendations on safe use of pesticides, and test gas levels before opening.

Q68. What is the procedure for carrying cargo residues? Treated under MARPOL Annex V. Cargo residues that are harmful to the marine environment (HME) may not be discharged. Others may be discharged only under the stated conditions.

Q69. Who is responsible for cargo information under SOLAS? The shipper provides appropriate cargo information well in advance. The Master may refuse to load if the information is inadequate.

Q70. What is the master's right regarding sampling and testing? The Master can require independent testing of moisture content and other properties when there is doubt, and may refuse loading if the cargo is unsafe.

## Section 4: Grain Cargoes (International Grain Code)

Q71. What is the International Grain Code? The International Code for the Safe Carriage of Grain in Bulk, mandatory under SOLAS Chapter VI. It sets stability and stowage requirements for grain.

Q72. What is the Document of Authorization? A document issued by the Administration (or on its behalf) certifying that the ship complies with the Grain Code. It is accompanied by grain loading information and the grain loading manual.

Q73. What are the intact stability criteria for grain? Angle of heel due to assumed grain shift not more than 12° (or deck-edge immersion angle if less), residual dynamic stability area not less than 0.075 m-rad up to 40° or the flooding angle if less, and initial GM corrected for free surface not less than 0.30 m.

Q74. What is the assumed grain shift? In a filled compartment, the grain surface is assumed to shift by 15° (25° in a partly filled compartment) for the purpose of calculating heeling moments.

Q75. What is the volumetric heeling moment (VHM)? The product of the volume of the void space and its transverse distance, used with the grain stowage factor to calculate the heeling moment from an assumed grain shift.

Q76. What is a filled compartment? A compartment in which the grain is trimmed and filled to the maximum extent, including the hatch ends and hatchways, to remove voids.

Q77. How do you secure a partly filled compartment? By trimming level and then securing the free surface with a saucer, bundling (bagged grain), strapping or lashing, or overstowing with bagged grain or other suitable cargo, in accordance with the Code.

Q78. What is a saucer? A depression formed in the grain surface in the hatch opening, filled with bagged grain or other cargo to restrain the surface. Its depth is at least one-twelfth of the hold breadth or 1.2 m, whichever is greater.

Q79. What are the precautions before loading grain? Holds must be clean, dry, free of residues and odours, and free from insects. Bilge covers and suctions must be in order, hatches weathertight, and a hold cleanliness inspection passed.

Q80. What are the problems with grain cargo in transit? Sweating, heating, moulding, infestation, shifting and, in some cases, fumigation hazard.

Q81. How is grain ventilated? Generally not ventilated, unless required. Follow the shipper's instructions and fumigation requirements. Fumigated cargo is kept sealed.

Q82. Why is stability checked at every stage of a grain voyage? Because consumption of fuel and water changes GM and free surface, so the Grain Code criteria must be met at departure, during the voyage, and on arrival.

Q83. What is a feeder? A structure (such as a hatchway feeder or trunk) that feeds grain into the void caused by settling, reducing the free space and the risk of shift.

Q84. What is grain heeling moment on ships without a document? Ships without a Document of Authorization may carry grain only under restricted conditions set by the Code (for example, fully loaded and trimmed compartments with specified securing). Follow the Code and the Flag State rule.

Q85. What is the hazard of loading grain in a hold previously used for another cargo? Contamination and infestation. The hold must be cleaned, washed and dried and, if required, inspected by a surveyor before loading.

## Section 5: Dangerous Goods (IMDG Code)

Q86. What is the legal basis of the IMDG Code? SOLAS Chapter VII (carriage of dangerous goods in packaged form) and MARPOL Annex III (harmful substances in packaged form). Amendments are issued every two years.

Q87. List the nine IMDG classes. Class 1 Explosives, 2 Gases, 3 Flammable liquids, 4 Flammable solids, 5 Oxidizers and organic peroxides, 6 Toxic and infectious substances, 7 Radioactive material, 8 Corrosives, 9 Miscellaneous.

Q88. What details are given for a dangerous good in the Dangerous Goods List? UN number, proper shipping name, class, subsidiary risk, packing group, special provisions, limited quantities, packing instructions, EmS number, stowage and segregation.

Q89. What are packing groups? PG I indicates great danger, PG II medium danger, PG III minor danger.

Q90. What documents are required for dangerous goods? Dangerous goods declaration, container/vehicle packing certificate, dangerous goods manifest or stowage plan, and emergency information. The ship must carry the special list, manifest or stowage plan (SOLAS II-2/19).

Q91. What are the segregation terms in the IMDG Code? 'Away from', 'separated from', 'separated by a complete compartment or hold from', and 'separated longitudinally by an intervening complete compartment or hold from'.

Q92. What are the stowage categories? Categories A to E, which define on-deck or under-deck stowage permitted for each cargo and ship type (cargo ship or passenger ship).

Q93. What is EmS? Emergency Schedules: F-series for fire (F-A to F-J) and S-series for spillage (S-A to S-Z), giving emergency actions for each substance.

Q94. What is MFAG? Medical First Aid Guide for use in accidents involving dangerous goods, giving treatment information.

Q95. What is a marine pollutant? A substance listed as harmful to the marine environment, requiring the marine pollutant mark on packages and a declaration in the documents.

Q96. What are the divisions of Class 1? Divisions 1.1 to 1.6 by hazard type (mass explosion, projection, fire, etc.), with compatibility groups A to S.

Q97. What are the stowage precautions for explosives? Keep away from accommodation and heat sources, keep cool and dry, segregate from other classes, protect from electrical sparks, and handle only by authorized crew with no smoking.

Q98. What are the Class 7 categories? Category I-White, II-Yellow and III-Yellow, based on radiation level at the surface and the Transport Index.

Q99. What is the Transport Index? The radiation level at 1 m from the package surface in mSv/h multiplied by 100, used to control stowage and exposure limits.

Q100. What are limited quantities? Small quantities of certain dangerous goods in inner packagings, subject to relaxed provisions (marking but no placarding).

Q101. What are special requirements for firefighting arrangements on ships carrying dangerous goods? SOLAS II-2/19 requires water supply, ignition source control, water spray and special equipment depending on the class and ship type.

Q102. Define flammable liquid (Class 3). A liquid with a flashpoint of not more than 60°C closed cup (or 65.6°C open cup).

Q103. What are the divisions of Class 4? 4.1 flammable solids, 4.2 substances liable to spontaneous combustion, and 4.3 substances which emit flammable gas when wet.

Q104. What is Class 5? 5.1 oxidizing substances and 5.2 organic peroxides. Some organic peroxides require temperature control during transport.

Q105. What is Class 6? 6.1 toxic substances and 6.2 infectious substances.

Q106. What is Class 8? Corrosive substances that cause damage to living tissue or materials, including the ship's structure.

Q107. What is Class 9? Miscellaneous dangerous substances and articles, including lithium batteries, dry ice and environmentally hazardous substances.

Q108. What are the divisions of Class 2? 2.1 flammable gases, 2.2 non-flammable non-toxic gases, 2.3 toxic gases.

Q109. What would you do for a leaking drum of dangerous goods? Identify the substance from the manifest, refer to EmS and MFAG, wear PPE and breathing apparatus, contain the leak, avoid ignition sources and, if necessary, jettison or isolate according to EmS.

Q110. What if a container with dangerous goods is on fire? Sound the alarm, identify the cargo from the manifest, consult EmS, use boundary cooling and water from a safe distance, avoid water if the cargo reacts with it, and call for assistance.

Q111. What are the hazards of ammonium nitrate? It is an oxidizer that may support or intensify fire, can decompose and detonate when heated or contaminated, and gives off toxic gases.

Q112. What placards and marks are required on a container carrying dangerous goods? Class label placards (at least 250 mm diamond) on all four sides, plus the UN number, and the marine pollutant mark if applicable.

Q113. What is the position of the Master regarding the carriage of undeclared dangerous goods? The Master may refuse to load, and must follow the IMDG Code and company procedures. Mis-declared goods are a serious breach.

Q114. What action is needed on an incident involving dangerous goods? Report per MARPOL Protocol I and SOLAS to the nearest coastal State, including details of cargo, quantity and type of incident.

Q115. What is the training requirement for dangerous goods? Personnel involved must be trained and certified in dangerous goods handling, in accordance with Chapter 1.3 of the IMDG Code.

## Section 6: Containers, Securing and Ro-Ro

Q116. What is the CSS Code? The Code of Safe Practice for Cargo Stowage and Securing, giving guidance for securing of cargoes (including containers) and the contents of the Cargo Securing Manual.

Q117. Name common container securing fittings. Twist locks, lashing rods, turnbuckles, bridge fittings, base cones, stackers, cell guides and lashing bars.

Q118. What is a twist lock? A fitting that connects containers corner to corner (or to the deck) by rotating cones into corner castings.

Q119. What is the VGM requirement? SOLAS VI/2 requires a verified gross mass of packed containers before loading, from 1 July 2016. The shipper must provide it in the shipping document.

Q120. What are the two methods for VGM? Method 1: weigh the packed container. Method 2: weigh all the packages and cargo, add the tare mass of the container, and sum the weights.

Q121. What is the CSC? The International Convention for Safe Containers, 1972. It requires a CSC safety approval plate and periodic or continuous examination of containers.

Q122. How are containers numbered in a bay system? Bay numbers run from forward, with odd bay numbers for 20-ft and even for 40-ft containers. Rows are numbered from the centreline, with odd to starboard and even to port. Tiers are numbered from the bottom.

Q123. Why should heavier containers be stowed at the bottom? To keep the centre of gravity low, protect stack strength and reduce lashing loads.

Q124. What forces act on cargo at sea? Rolling, pitching and heaving accelerations, wind and sea loads, and gravity. These are calculated in the CSM.

Q125. What is parametric rolling? A sudden, severe resonant rolling that can occur in head or following seas when wave encounter period matches half the roll period, which can cause stack collapse and container loss.

Q126. Why are container fires difficult to fight? Deep seated fires within closed boxes, limited access, unknown contents, mis-declared goods, and stacking. Use detection, boundary cooling and, where available, container firefighting systems.

Q127. How do you handle a lost container at sea? Mark position, report to the nearest coastal State and the company, warn vessels in the area, and report details of any dangerous goods.

Q128. What are the checks on reefer containers? Verify the pre-trip inspection (PTI), connection to power, setpoint, delivery and return air temperatures, and drain and defrost. Monitor and log at regular intervals.

Q129. What is controlled atmosphere (CA) carriage? Carrying produce in an atmosphere with modified oxygen and CO2 levels to slow ripening and extend shelf life.

Q130. What is an out-of-gauge cargo? Cargo exceeding the dimensions of a standard container, carried on flat racks or platforms, which needs special stowage and lashing.

Q131. What are the problems of misdeclared container weights? Stack collapse, wrong stability and lashing assessment, hull stress, and risk to crew and ship.

Q132. What are the hatch cover loading limits? The hatch cover is designed for a specific uniformly distributed load and a container footprint load. The limits are in the CSM and must not be exceeded.

Q133. Why does GM matter on container ships? Deck cargo raises KG. Too large a GM gives very short roll periods and high accelerations, while too small a GM gives low stability. The target is the optimum in the CSM and loading manual.

Q134. What are the points to check on Ro-Ro cargo securing? Lashing points on the vehicle and deck, adequate number and strength of lashings, brakes applied, parking position and tyre pressures.

Q135. What is the rule of thumb for lashing in the CSS Code? The total MSL of the securing devices on each side of a unit should equal the weight of the unit (for the basic approach).

Q136. What are the dangers of tank containers? Partial filling with free surface and liquid movement, so filling is limited to between about 80% and 95% (some products have specific limits).

Q137. When should lashings be checked? Before sailing, after leaving port, and after heavy weather. Tighten as needed and record the checks.

Q138. What is a cell guide system? Vertical guides in holds that allow containers to be stowed without lashings, giving stable and quick handling.

Q139. What is a lashing bridge? A structure on deck or between bays that supports lashings and increases the stack height that can be secured.

Q140. What is the master's responsibility under the CSM? Ensure cargo is stowed and secured in accordance with the CSM, that the crew follows it, and that any deviations are reviewed and recorded.

## Section 7: Oil Tanker Operations

Q141. What is ISGOTT? International Safety Guide for Oil Tankers and Terminals, the industry guide to safe tanker and terminal operations, widely used as the standard.

Q142. What is the Ship/Shore Safety Checklist? A checklist in ISGOTT completed jointly by ship and terminal before cargo operations, confirming safety conditions and communication, signed by both parties.

Q143. What are the IGS requirements for oxygen content? The inert gas delivered must contain not more than 5% oxygen by volume, and the tank atmosphere is kept at 8% or less, with positive pressure maintained.

Q144. Why is inert gas used? To keep the tank atmosphere non-flammable by reducing oxygen below the level that supports combustion of hydrocarbon gas (about 11%).

Q145. What is the typical flammable range of petroleum gas? About 1% (LFL) to 10% (UFL) by volume in air.

Q146. Name the main components of an IGS. Scrubber, blowers, deck water seal, non-return valve, pressure/vacuum breaker, pressure control valve, oxygen analyser and mast riser.

Q147. What is the function of the deck water seal? To prevent hydrocarbon gas and air from flowing back into the engine room or flue gas system.

Q148. What is Crude Oil Washing (COW)? Washing cargo tanks with the crude oil being discharged, using fixed washing machines, to remove sludge and reduce residues.

Q149. What are the requirements for COW? Tank atmosphere under 8% oxygen by volume, an approved COW operations and equipment manual, a COW checklist, inert gas supply, and adequate pump and stripping capacity.

Q150. What are the benefits of COW? Increased outturn of cargo, reduced sludge, reduced use of water for washing, and less pollution.

Q151. What is the Load on Top system? Slops are allowed to settle in a slop tank, the water is discharged, and the next cargo is loaded on top of the retained oil residue.

Q152. What are the MARPOL Annex I conditions for discharge from cargo areas of tankers? The tanker is more than 50 nm from nearest land, is proceeding en route, discharge rate is not above 30 litres per nautical mile, and the total quantity is not more than 1/30,000 of the cargo (for newer ships). ODME must be operating.

Q153. What is ODME? Oil Discharge Monitoring and Control Equipment. It measures the oil content and flow rate of discharge, records it, and stops discharge if limits are exceeded.

Q154. What are segregated ballast tanks (SBT)? Ballast tanks totally separated from cargo and fuel systems, used only for ballast. Protectively located (PL) SBT form part of the protection against collision or grounding.

Q155. What is the double hull requirement? MARPOL Annex I Regulation 19 requires double hull (or equivalent) for new oil tankers, with phase-out dates for single-hull tankers.

Q156. What is SOPEP? Shipboard Oil Pollution Emergency Plan, required by MARPOL Annex I, giving procedures for reporting and controlling oil spills.

Q157. What is recorded in the Oil Record Book Part II? Cargo and ballast operations on tankers, such as loading, discharging, COW, ballasting, tank cleaning, slop transfers and discharges, signed by the officer in charge and the Master.

Q158. What are the safety precautions for a pumproom? Continuous ventilation before and during entry, gas detection, entry permit, bilge alarms and no hot work. Maintain communications and a standby person.

Q159. What is ullage, innage and sounding? Ullage is the distance from the reference point to the liquid surface. Sounding is the distance from the bottom to the surface. Innage is the depth of liquid, equivalent to sounding.

Q160. How do you compute cargo quantity? Measure ullage or sounding, trim and list corrections, get volume from the calibration tables, correct for temperature using the volume correction factor (VCF) and compute mass using density.

Q161. What is the relation between API gravity and density? API gravity = (141.5 / specific gravity at 60°F) − 131.5. A higher API gravity means a lighter oil.

Q162. What are the precautions against static electricity? Limit initial loading velocity (about 1 m/s until the inlet is covered), avoid free fall of oil, do not sample or gauge for the settling period after loading in non-inerted tanks, and bond and earth equipment.

Q163. What is the settling period? A wait of at least 30 minutes after completion of loading before dipping or sampling with metallic equipment in non-inerted tanks.

Q164. What is tank cleaning? Washing of tanks with water (hot or cold) or crude oil to remove residues, followed by gas freeing and inspection.

Q165. What is gas freeing? Replacing the tank atmosphere with fresh air to bring hydrocarbon concentration below 1% LFL (for hot work) and oxygen to 21% for entry.

Q166. What is VECS? Vapour Emission Control System. Under SOLAS and MARPOL Annex VI, tankers must have a vapour collection system for connection to shore terminals when required.

Q167. What is closed loading? Loading with no opening of tank lids for gauging, using fixed gauges and high level alarms, so that vapours are not released.

Q168. What is a high-level alarm and overfill alarm? Independent alarms that warn when a tank reaches a high level (typically 95%) and an overfill level (98%) to prevent spills.

Q169. What is topping off? The final phase of loading, done at reduced rate, with a few tanks at a time, to avoid overflow.

Q170. What is pressure surge? A sudden pressure rise when a valve closes quickly in a flowing pipeline. It can damage pipes and hoses. Use agreed closing times at least as long as the minimum.

Q171. What is the precaution when heating cargo? Monitor the heater, keep temperatures below the maximum to avoid flammability and expansion, and check steam return for oil contamination.

Q172. What is a non-volatile oil? An oil with a flashpoint above 60°C, which can be treated differently for some precautions. ISGOTT treats these with certain relaxations.

Q173. What are the key requirements for hot work on a tanker? Hot work permit, tank gas-freed and tested, adjacent spaces checked, isolation of pipes, fire watch and company/terminal approval. Hot work outside the engine room needs formal authorization.

Q174. Who is the Person in Charge during cargo operations? The Master designates a Person in Charge, normally the Chief Officer or duty officer, responsible for cargo operations and communications with the terminal.

Q175. What are the bonding arrangements for hoses? Bonding cables are not used. Rely on insulating flanges or a single length of hose to prevent a spark. Ensure electrical continuity is broken between ship and shore.

## Section 8: Gas and Chemical Carriers, Timber, Liability and Miscellaneous

Q176. What is the IGC Code? International Code for the Construction and Equipment of Ships Carrying Liquefied Gases in Bulk, covering design and construction of gas carriers.

Q177. What are the three methods for carrying LPG? Fully pressurised, semi-refrigerated (semi-pressurised) and fully refrigerated.

Q178. What are the IGC cargo tank types? Type A, Type B, Type C (pressure vessels) and membrane tanks, plus integral and semi-membrane tanks.

Q179. What is the IBC Code? International Code for the Construction and Equipment of Ships Carrying Dangerous Chemicals in Bulk. Ship types 1, 2 and 3 correspond to decreasing hazard levels.

Q180. What are MARPOL Annex II categories? Category X (major hazard), Y (hazard), Z (minor hazard) and Other Substances (OS). Discharge restrictions are most severe for X.

Q181. What is the P&A Manual? Procedures and Arrangements Manual, approved by the Administration, giving cargo residue, tank washing and discharge procedures under MARPOL Annex II.

Q182. What is a Cargo Record Book? The record of cargo and ballast operations on chemical tankers, required under MARPOL Annex II.

Q183. What is a compatibility chart? A chart showing which chemicals may be safely stowed adjacent or loaded in the same tank or hose system.

Q184. What is the Timber Deck Cargo Code? The Code of Safe Practice for Ships Carrying Timber Deck Cargoes, with requirements for stowage, stability and lashing. The height of deck cargo should generally not exceed one-third of the extreme breadth.

Q185. What stability allowances are made for timber deck cargo? An allowance for increase in weight from water absorption (about 10–15%) and for ice accretion, as the Code requires. GM must stay positive and meet criteria.

Q186. What is the lashing arrangement for timber deck cargo? Lashings are applied at regular intervals (not more than 1.5 m for typical stowage) over the cargo and secured to eye plates, and checked and tightened at sea. Follow the Code and CSM.

Q187. What is the main minimum GM for general ships (intact stability criteria)? The IS Code requires initial GM of at least 0.15 m, with area under the righting arm curve (GZ) at least 0.055 m-rad up to 30° and 0.09 m-rad up to 40°, and maximum GZ at 30° or more of at least 0.20 m.

Q188. What is the effect of free surface? A virtual rise in the centre of gravity (loss of GM) equal to the free surface moment divided by displacement, reducing stability.

Q189. What are the Hague-Visby Rules limits of liability? 666.67 SDR per package or 2 SDR per kilogram of gross weight of goods lost or damaged, whichever is higher, unless the nature and value are declared.

Q190. What are the carrier's basic obligations under the Hague-Visby Rules? Exercise due diligence to make the ship seaworthy and cargoworthy before and at the beginning of the voyage, and properly and carefully load, handle, stow, carry, care for and discharge goods.

Q191. What is cargo ventilation and what are its types? Natural ventilation (by cowls and louvres) and mechanical ventilation (by fans). It is used to remove heat, moisture and gases from the hold.

Q192. What is a cargo-worthy hold? A hold that is clean, dry, free from odours and residues, with sound bilges, ladders, lighting and hatch covers, passing an inspection by the surveyor.

Q193. What are the stowage precautions for steel cargoes? Use sufficient dunnage, spread weight on the tank top, lash against shifting, protect against sweat and rust, and avoid contact with other cargoes.

Q194. What are the precautions when carrying bagged cargo? Check bag condition, stow in a tight pattern with dunnage, prevent slip with dunnage or paper, ventilate as needed, and avoid hooks.

Q195. What are the main hazards when carrying cement in bulk? Dust, caking and moisture damage. Keep dry, with weathertight hatches, and prevent contamination.

Q196. What precautions apply to carrying hot or heated cargoes? Control temperature, provide ventilation, avoid contact with the bilge and bulkheads, and keep the cargo away from other incompatible cargo.

Q197. What is a Hold Ready Certificate? A certificate or report by an independent surveyor or the charterer's representative confirming that the holds are clean and fit to load the intended cargo.

Q198. Why must the Chief Officer monitor cargo during the voyage? To check lashings, ventilation, temperature, moisture and condition, and to maintain a cargo log. This protects the cargo and the ship.

Q199. What is a Cargo Log (Cargo Book)? A record of cargo operations, tank/hold measurements, temperatures and ventilation, and any incidents. It provides evidence in claims.

Q200. What is the role of the Chief Officer in cargo operations? Plans stowage and stability, supervises loading and discharge, ensures safety and compliance with codes, keeps cargo records, maintains cargo gear, informs the Master, and communicates with the terminal and the charterer.

# Chief Mate Oral – Function 3 (Controlling the Operation of the Ship and Care for Persons on Board): 200 Q&A

Study guide for Indian MMD Chief Mate (FG) oral examination. Answers are short, exam-style points. Always verify figures and requirements against the latest editions of SOLAS, MARPOL, Load Line, ISM, ISPS, MLC and your company SMS.

## Section 1: Ship Stability

Q1. Define GM. Metacentric height: the distance between the centre of gravity (G) and the transverse metacentre (M). It is the measure of initial stability.

Q2. How are KM, KB and BM related? KM = KB + BM, where BM = I/V (second moment of the waterplane area about the centreline divided by the volume of displacement). GM = KM − KG.

Q3. Define stable, neutral and unstable equilibrium. Stable: G below M (positive GM), the ship returns upright. Neutral: G coincides with M (GM zero). Unstable: G above M (negative GM), the ship heels further when disturbed.

Q4. What is the righting lever and righting moment? GZ is the horizontal distance between the lines of action of weight and buoyancy. For small angles GZ = GM sinθ. Righting moment = displacement × GZ.

Q5. What is angle of loll? The angle at which a ship with negative initial GM floats at rest, where GZ is zero. She may loll to either side and can flop from one side to the other.

Q6. Difference between list and loll? List: positive GM with G off the centreline, so the ship floats to one side only. Loll: negative GM with G on the centreline, and the ship can loll either way.

Q7. How do you correct an angle of loll? Lower G by filling double-bottom tanks (or shifting low weight), starting with the tanks on the low side first, so that the ship does not flop to the other side. Avoid creating slack tanks and do not move weight to the high side.

Q8. What is free surface effect? When a liquid in a tank has a free surface, it shifts as the ship heels, producing a virtual rise of G (loss of GM). FSM = ρ × (l b³ / 12) for a rectangular tank and the virtual rise GG' = FSM / displacement.

Q9. How can free surface effect be reduced? Keep tanks either full or empty, subdivide tanks with longitudinal bulkheads (dividing into n parts reduces the effect to 1/n²), and use baffles.

Q10. How do you calculate the shift of G when a weight is shifted? GG₁ = w × d / Δ, in the direction of the shift. For transverse shift, list tanθ = GG₁ / GM.

Q11. What is the effect of a suspended weight? The weight acts as if it were at the point of suspension, so G rises immediately when the weight is lifted clear. This reduces GM even before the weight leaves the deck.

Q12. What is the effect of loading a weight above the ship's G? G moves up and toward the weight, so KG rises and GM falls. Loading below G does the opposite.

Q13. Stiff and tender ships. Stiff: large GM, short roll period, violent rolling, strong forces on lashings. Tender: small GM, long roll period, sluggish roll and low reserve of stability.

Q14. How do you estimate GM from the roll period? T = 2CB/√GM, where C is roughly 0.4, so GM = (0.8 B / T)² approximately. Roll period is measured for a full oscillation in calm conditions.

Q15. What is an inclining experiment? A test to determine the lightship KG. Known weights are shifted across the ship and the resulting angle of heel is measured with pendulums, so that GM = w × d / (Δ × tanθ).

Q16. When must a ship be re-inclined? SOLAS II-1 requires a lightweight survey at intervals not exceeding five years. If the lightship displacement differs by more than 2% (or the LCG by more than 1% of L), an inclining test is required.

Q17. What conditions are needed for a good inclining test? Calm weather, ship upright and afloat, mooring slack, no free surface, all weights recorded, people ashore and the ship near the lightship condition with minimum slack tanks.

Q18. Define TPC. Tonnes per centimetre immersion: the mass needed to change the mean draft by one centimetre. TPC = waterplane area × density / 100.

Q19. What is the Fresh Water Allowance? The number of millimetres by which a ship's draft increases when going from salt water to fresh water at the same displacement. FWA = Δ / (4 × TPC) mm.

Q20. What is the Dock Water Allowance? DWA = FWA × (1025 − ρ) / 25, where ρ is the dock water density in kg/m³. It is the allowance applied when loading in water of intermediate density.

Q21. What load lines appear on a ship's side? TF (tropical fresh), F (fresh), T (tropical), S (summer), W (winter) and WNA (winter North Atlantic), with timber lines if she is assigned them.

Q22. What is freeboard? The distance from the upper edge of the deck line down to the upper edge of the relevant load line.

Q23. What is the dimension of the Plimsoll mark? The ring is 300 mm in outer diameter and 25 mm thick, with a horizontal line 450 mm × 25 mm through its centre. The load lines are 230 mm × 25 mm.

Q24. What are the Tropical and Winter freeboards relative to Summer? Tropical is 1/48 of the summer moulded draft less freeboard (the ship may load deeper), and Winter is 1/48 of summer moulded draft more freeboard.

Q25. How long is an International Load Line Certificate valid? For a maximum of five years, with annual surveys within three months either side of the anniversary date.

Q26. What are the IS Code criteria for intact stability? Initial GM at least 0.15 m. Area under the GZ curve at least 0.055 m-rad up to 30° and 0.09 m-rad up to 40° (or flooding angle), and at least 0.03 m-rad between 30° and 40°. GZ at least 0.20 m at 30° or more, with maximum GZ at not less than 25°.

Q27. What is the weather criterion? A test of the ship's ability to withstand the combined effect of beam wind and rolling, comparing the wind heeling lever with the righting lever curve.

Q28. What is reserve buoyancy? The volume of the watertight portion of the ship above the waterline, which gives the ship the ability to withstand loading and flooding.

Q29. What is permeability? The percentage of a compartment's volume that can be occupied by water. Typical values: machinery spaces 85%, accommodation 95%, stores 60%, empty spaces 95%.

Q30. What is floodable length? The maximum length of a compartment, centred at a given point, that can be flooded without the ship's margin line being submerged.

Q31. What is the margin line? A line drawn 76 mm below the upper surface of the bulkhead deck at side, used in subdivision calculations.

Q32. What is the effect of bilging a midship compartment? Buoyancy is lost and the ship sinks bodily and may trim. The lost-buoyancy method treats the flooded space as no longer part of the ship, so KM changes and GM may rise or fall.

Q33. What is the difference between the 'added weight' and 'lost buoyancy' methods? Added weight treats the floodwater as extra mass in the ship. Lost buoyancy treats the flooded compartment as no longer contributing buoyancy. The two give the same final draft, but the lost buoyancy method keeps the ship's displacement unchanged.

Q34. What are KN or cross curves? Curves of KN (the righting lever measured from K) against displacement for different angles of heel. GZ = KN − KG sinθ.

Q35. What is dynamic stability? The work done in heeling the ship to an angle, represented by the area under the GZ curve up to that angle.

Q36. What is the effect of icing on stability? Ice adds weight high above G, raising KG, increasing windage and reducing GM and range of stability. Remove ice, avoid ice zones and consider ballasting.

Q37. What is the effect on stability of turning? The ship heels outward due to centrifugal force. The angle increases with the square of speed and decreases with larger turning radius and larger GM.

Q38. What happens when a ship moves from sea water to fresh water? She sinks deeper (draft increases), because fresh water is less dense. Displaced volume increases, though the mass remains the same.

Q39. What is the loss of GM in dry dock? As the keel touches the blocks, an upthrust P acts at the keel. P = (MCTC × change in trim in cm) / (distance from LCF to the keel contact point). The virtual loss of GM = P × KM / Δ.

Q40. What is the critical instant in dry docking? The moment just before the ship takes the blocks fully (when the upthrust is greatest), which is the time of lowest effective GM. GM must remain positive.

## Section 2: Trim, Hull Strength, Dry Docking and Casualties

Q41. Define trim. The difference between the forward and aft drafts. Trim by the stern is aft draft greater than forward draft.

Q42. What are LCF, LCB and MCTC? LCF is the longitudinal centre of flotation (centroid of waterplane), LCB the longitudinal centre of buoyancy, and MCTC the moment to change trim one centimetre.

Q43. How do you calculate the change of trim when loading a weight? Trimming moment = w × distance from LCF. Change of trim (cm) = trimming moment / MCTC. Add bodily sinkage of w / TPC.

Q44. How are the new forward and aft drafts found? Apply bodily sinkage to both drafts, then apply the change of trim in proportion to the distances of the perpendiculars from the LCF.

Q45. Why does optimum trim matter? It improves fuel economy, steering, propeller immersion, visibility and ship handling, and reduces stress. Charterers often specify a trim.

Q46. What is hogging and sagging? Hogging: the middle is supported by buoyancy more than the ends (ends droop). Sagging: the ends are supported more than the middle (middle droops). Heavy weight at the ends causes hogging, and heavy weight amidships causes sagging.

Q47. What are shear force and bending moment? Shear force is the net vertical force (weight minus buoyancy) at a section. Bending moment is the integrated effect of shear along the hull. Both must be within permissible limits in the loading manual.

Q48. How do you monitor hull stress on board? Use the loading computer or manual calculations against the limits in the approved loading manual, and for bulk carriers, hull stress monitoring at each stage of loading and discharging.

Q49. What are torsion and racking stresses? Torsion is twisting of the hull caused by uneven loads (for example container ships in oblique seas), and racking is transverse deformation due to rolling and uneven cargo.

Q50. What additional stresses arise at sea? Wave bending, slamming, pounding, panting, thermal stresses and sloshing.

Q51. What is the loading manual? A document approved by the Classification Society giving permissible still-water bending moments and shear forces and typical loading conditions.

Q52. What preparations are made before dry docking? Submit the docking plan, arrange tank ullages (limit free surface), ensure a small trim by the stern, secure equipment, and land cargo and unneeded weights. Check stability and notify the dock of the ship's particulars.

Q53. What is a docking plan? A plan showing the keel blocks, side blocks and position of structure, hull openings and fittings that must be avoided when blocking.

Q54. Which items are inspected in dry dock? Shell plating, rudder and pintles, propeller and shaft, sea chests, anodes, bow thruster, stabilisers, anchors and cables, draft marks, and hull coating.

Q55. What checks are made before undocking? Confirm that sea chest covers, plugs and valves are refitted, anodes and draft marks are done, all works and tests are complete, and that tanks are in order. Verify that the dock is flooded gradually.

Q56. What actions are taken after grounding? Stop engines, sound all tanks and spaces, check for damage, assess stability and stress, determine tide and position, avoid use of engines until assessed, inform the company and authorities, and decide on refloating.

Q57. How do you refloat a grounded ship? Lighten the ship (discharge cargo, ballast, fuel), use tugs or anchors (kedging), and wait for a rising tide. Verify the damage first, and avoid refloating if it will lead to flooding.

Q58. What actions follow a collision? Sound the alarm, stop and assess, check for damage and injured, ascertain stability and flooding, exchange details with the other ship, assist if needed, report to the company and authorities, and keep records.

Q59. What are the precautions for heavy weather? Secure cargo and deck items, check hatch covers and ventilators, close weathertight doors, make ballast adjustments, reduce speed, alter course, and keep the stability and strength within limits.

Q60. What is squat? Bodily sinkage and trim of a ship underway due to lower pressure under the hull. In open water, squat (m) ≈ Cb × V² / 100 (V in knots), and it is about double in confined channels.

## Section 3: SOLAS and Fire Safety

Q61. What is SOLAS? The International Convention for the Safety of Life at Sea, 1974, the main treaty on ship safety, with chapters on construction, fire protection, lifesaving appliances, radio, navigation safety, cargo, ISM, security and others.

Q62. Explain the fire triangle and tetrahedron. The fire triangle: fuel, oxygen, heat. The tetrahedron adds the chemical chain reaction. Removing any element extinguishes the fire.

Q63. What are the classes of fire? A (solids), B (flammable liquids), C (gases), D (metals), and F (cooking oils and fats). Electrical fires are treated by the fire class of the burning material, with the power isolated.

Q64. What are the main extinguishing agents? Water, foam, CO2, dry powder, and wet chemical. Each is suited to certain classes of fire.

Q65. Which fixed fire-extinguishing systems are used on ships? CO2, high expansion foam, water spray, sprinklers, water mist, and inert gas on tankers. CO2 is the most common in machinery spaces and cargo holds.

Q66. What is the procedure for releasing CO2 into a space? Confirm all persons are out and counted, close all openings and ventilation, stop fuel and fans, sound the alarm, and release as per instructions. Keep the space closed and cool the boundaries, and do not open it until it has cooled (usually at least 24 hours).

Q67. What are the requirements for fire pumps? Cargo ships must have independent power-driven fire pumps, including an emergency fire pump located outside the machinery space, with a power source and sea suction independent of the main arrangement.

Q68. What is the FSS Code? The International Code for Fire Safety Systems, mandatory under SOLAS Chapter II-2, giving technical specifications for fire safety systems and equipment.

Q69. What types of fire detectors are there? Smoke (ionisation or optical), heat (fixed temperature or rate-of-rise), flame (UV or IR), and sample extraction smoke detection for cargo spaces.

Q70. What are A, B and C class divisions? A class divisions are steel or equivalent, insulated and capable of preventing the passage of smoke and flame for one hour (A-60 gives a 60-minute temperature rise limit). B class divisions are non-combustible and limit flame spread for 30 or 15 minutes. C class are non-combustible.

Q71. What is the Fire Control Plan? A general arrangement plan permanently displayed showing control stations, fire sections, detection and alarm systems, extinguishing arrangements, means of access, ventilation and fixed systems. A copy is kept in a weathertight container outside the deckhouse for shore firefighters (SOLAS II-2/15).

Q72. What does a firefighter's outfit comprise? Protective clothing, boots and gloves, rigid helmet, electric safety lamp, axe, breathing apparatus (BA) with spare cylinders, and a fireproof lifeline. Ships carry a minimum of two outfits.

Q73. What is an EEBD? Emergency Escape Breathing Device: gives at least 10 minutes of air for escape from a hazardous atmosphere. It is not for firefighting.

Q74. What are the requirements for drills? Each crew member must participate in at least one abandon ship drill and one fire drill every month. If more than 25% of the crew did not take part in the last drills before leaving port, drills must be held within 24 hours of departure.

Q75. What is the general strategy for fighting a fire? Raise the alarm, locate and size up the fire, isolate (fuel, power, ventilation), cool the boundaries, attack, search for casualties, and keep a re-ignition watch. Report to the Master and company.

Q76. What are the steps for a machinery space fire? Stop fuel supply and ventilation, close fire flaps and doors, evacuate and muster, boundary cool, and release CO2 or another fixed system after confirming a head count. Do not re-enter until cool and tested.

Q77. What are the actions in a fire in the accommodation? Sound the alarm, send a team to investigate, close fire doors, shut ventilation, rescue people, fight with portable and hose equipment, and cool adjacent bulkheads. Use smoke hoods or BA.

Q78. What is the general method for a fire in a cargo hold? Close hatches and stop ventilation to cut off air, identify the cargo and refer to the IMSBC/IMDG/EmS instructions, apply the fixed system (CO2 or water as appropriate), and cool the boundaries.

Q79. How do you deal with an electrical fire? Isolate the power first, and then use CO2 or dry powder. Do not use water or foam until power is confirmed off.

Q80. What stability concern arises when fighting a fire with large amounts of water? Water accumulates in the space, producing a free surface effect, loss of GM, additional weight high up and possible list. Pump or drain water out and monitor the stability.

Q81. What is the required CO2 quantity for machinery and cargo spaces? For cargo spaces, enough gas for 30% of the gross volume of the largest cargo space. For machinery spaces, enough for 40% of the gross volume of the largest machinery space, including casing (35% under certain conditions).

Q82. What are the requirements for portable extinguishers? As required by the FSS Code and ship type. They must be of approved type, located in accessible positions, and maintained and inspected regularly. Spare charges are carried for a proportion of them.

Q83. What is the Fire Safety Operational Booklet? A booklet carried on each ship giving information and instructions for the safe operation of fire systems, procedures and crew duties. It is available in the working language of the ship.

Q84. What are the fixed foam arrangements on a tanker? A deck foam system with monitors and applicators to cover the cargo deck and, with the foam concentrate in the foam room. The system must be able to deliver on the largest cargo tank area for the specified time.

Q85. What are the key precautions for hot work? Hot work permit, atmosphere testing, removal or protection of combustibles, fire watch, extinguishers at hand, isolation of adjacent spaces and cargo systems, and clearance by the responsible officer.

Q86. How do you deal with a galley fire? Shut off the fuel and electrical supply, use a fire blanket or wet chemical extinguisher on cooking oil fires, and do not use water. Close the ventilation and watch for spread.

Q87. What are the hazards of a paint locker fire? Heavy smoke, toxic gases and rapid spread. Keep the locker doors closed, use the fixed system (usually CO2 or sprinklers), and cool the boundaries.

Q88. What special actions are required if a fire breaks out in port? Sound the alarm and notify the port fire services, state the location and any hazardous cargo, give the fire control plan to shore firefighters, and tell the Master and company.

Q89. What routine testing and checks are done on fire-fighting equipment? Weekly and monthly checks as in the planned maintenance system. Test pumps, hoses, hydrants, detection, alarms, extinguishers and BA sets. Keep records.

Q90. What is the Fire Safety Training Manual? A manual required by SOLAS II-2/15.2.3 in each crew mess room and recreation room (or each cabin) giving instructions for fire safety, fire prevention, and fighting fires.

## Section 4: Life-Saving Appliances and Emergencies

Q91. What is the LSA Code? The International Life-Saving Appliance Code, mandatory under SOLAS Chapter III, giving technical requirements for lifeboats, liferafts, lifejackets, immersion suits, rescue boats, and other equipment.

Q92. What are the lifeboat requirements on a cargo ship? Totally enclosed lifeboats of total capacity for 100% of persons on board on each side, or a free-fall lifeboat for 100% at the stern together with davit-launched liferafts as specified in SOLAS III/31. Check the ship's Safety Equipment Certificate for the exact requirement.

Q93. How many liferafts are carried? Liferafts have to provide the capacity required by SOLAS III/31 (typically 100% of persons, as the Safety Equipment Certificate requires), stowed for easy launching, with a hydrostatic release unit.

Q94. What is a hydrostatic release unit (HRU)? A device that automatically releases a liferaft when it sinks to a depth of about 1.5 to 4 m. The painter stays attached to the ship so that the raft inflates.

Q95. What are the lifejacket requirements? One lifejacket for each person on board, plus extra jackets for watchkeepers and remote stations. Lifejackets must have a light, whistle and retro-reflective tape.

Q96. What are the immersion suit requirements? On cargo ships, an immersion suit for every person assigned to the rescue boat crew, and for every person on board where the ship is not on a regular warm climate voyage and lifeboats are not fully enclosed. Check the exact requirement under SOLAS III/32.

Q97. What is an EPIRB? Emergency Position Indicating Radio Beacon, working on 406 MHz via satellite, with a float-free release, which transmits a distress alert with position and identification.

Q98. What is a SART? Search and Rescue Radar Transponder, which responds to a 9 GHz (X-band) radar signal by producing a series of 12 dots on the radar display of the searching ship or aircraft.

Q99. What is the SOLAS Training Manual? A manual placed in each crew mess or recreation room (or each cabin), with instructions and information on LSA and survival at sea.

Q100. What is a muster list? A list setting out the emergency duties of each person on board, the general emergency alarm signal, and the action to be taken. It is posted in conspicuous places.

Q101. What is the general emergency alarm signal? Seven or more short blasts followed by one long blast on the ship's whistle and alarm bells or sirens.

Q102. What is the procedure for an abandon ship drill? Sound the alarm, muster at the stations, check lifejackets and immersion suits, prepare and launch (or lower) at least one lifeboat as necessary, test the radio equipment, brief the crew on use of LSA, and record the drill.

Q103. What are the weekly and monthly lifeboat checks? Weekly: inspect lifeboats, engines and launching appliances, and run the engine for at least three minutes. Monthly: inspect all LSA to confirm completeness and condition, as per a checklist.

Q104. How often are lifeboats lowered? Each lifeboat is launched with its assigned crew and manoeuvred at least once every three months during an abandon ship drill (free-fall lifeboats at least once every six months).

Q105. What are the duties of the rescue boat? Recovering persons from the water, marshalling liferafts, and rescuing persons from vessels in distress. It must be able to be launched in five minutes.

Q106. What are the main points of survival at sea? Protect from exposure, ration water and food, keep together, keep watch, use signals, stay close to the wreck if appropriate, and wait for rescue.

Q107. What precautions apply during a helicopter rescue? Provide a clear winching area, display wind direction, do not attach the winch wire to the ship, secure loose objects, and use hand signals.

Q108. What is a Williamson turn? A manoeuvre used to return to the track of a person overboard, especially in poor visibility: put the rudder hard over, and after 60° off the original heading, shift the rudder to the opposite side and bring the ship to the reciprocal course.

Q109. What are the actions for a person overboard? Shout, release a lifebuoy with light and smoke, start the manoeuvre, mark GPS position, post lookouts, alert the Master and the engine room, and call mayday if required.

Q110. What is the procedure for rescuing a person from an enclosed space? Raise the alarm and do not enter without BA. Use a rescue team with BA and lifelines, ventilate the space, and give resuscitation and oxygen to the casualty.

Q111. What is the SAR Convention? The International Convention on Maritime Search and Rescue 1979, establishing the global SAR system and requiring states to provide SAR services and coordination.

Q112. What are the GMDSS sea areas? A1: within VHF coast station range (about 20–30 nm). A2: beyond A1 within MF range (about 100 nm). A3: within the coverage of geostationary satellites (Inmarsat). A4: polar regions beyond A3.

Q113. What is a distress alert? An automatic or manual transmission via DSC, satellite or EPIRB which identifies the ship, position and nature of distress, followed by a distress message (Mayday) by voice.

Q114. What is the IAMSAR Manual? The International Aeronautical and Maritime Search and Rescue Manual in three volumes, used for planning, coordinating and conducting SAR operations.

Q115. What are the duties of the Master to assist persons in distress? Under SOLAS V/33, the Master must proceed with all speed to assist persons in distress at sea, unless unable or unreasonable, and inform them accordingly.

## Section 5: MARPOL

Q116. What is MARPOL? The International Convention for the Prevention of Pollution from Ships 1973 as modified by the 1978 Protocol. Annexes: I oil, II noxious liquid substances, III packaged harmful substances, IV sewage, V garbage, VI air pollution.

Q117. What does the Oil Record Book Part I cover? Machinery space operations on all ships over 400 GT: ballasting or cleaning of fuel tanks, discharge of ballast, sludge disposal, bilge water discharge and bunkering. Each entry is signed by the officer and each page by the Master.

Q118. What are the conditions for discharging oily bilge water from machinery spaces? The ship is en route, oil content of effluent without dilution does not exceed 15 ppm, and the oily water separator and oil content monitor are operating. In special areas, no discharge is allowed except through OWS with 15 ppm limit, and no discharge in the Antarctic area.

Q119. What is the 15 ppm alarm? The oil content meter alarms and stops the overboard discharge (diverting to the bilge or holding tank) if the oil content exceeds 15 ppm.

Q120. Name some special areas under Annex I. Mediterranean, Baltic, Black Sea, Red Sea, Gulfs area, Gulf of Aden, Antarctic, North West European waters, Oman area of the Arabian Sea, and Southern South African waters.

Q121. What is the IOPP Certificate? International Oil Pollution Prevention Certificate, valid for five years, with annual and intermediate surveys, issued to ships of 400 GT and above (and oil tankers of 150 GT and above).

Q122. What are the sewage discharge rules under Annex IV? Sewage is discharged more than 3 nm from land if comminuted and disinfected, or more than 12 nm if untreated, at 4 knots or more and a moderate rate. Treated sewage from an approved plant may be discharged in accordance with its standards.

Q123. What are the garbage rules under Annex V? All plastics are banned from discharge. Food waste may be discharged more than 12 nm (or 3 nm if ground to 25 mm or less) and with special area restrictions. Garbage is recorded in the Garbage Record Book and handled under the Garbage Management Plan.

Q124. What documents are required under Annex V? Garbage Management Plan, Garbage Record Book (for ships of 400 GT and above), and placards to inform crew and passengers of the discharge rules.

Q125. What are the main SOx limits under Annex VI? A global fuel sulphur limit of 0.50% m/m (since 1 January 2020), and 0.10% m/m within Emission Control Areas.

Q126. Which areas are Emission Control Areas? The Baltic Sea, North Sea, North American area, and US Caribbean Sea, with the Mediterranean Sea added as a SOx ECA. Check the current IMO list for others.

Q127. What are the NOx rules? Engines above 130 kW installed on ships built on or after 2000 must meet Tier I, II or III limits depending on the date and area. Tier III applies in NOx emission control areas.

Q128. What is a Bunker Delivery Note (BDN)? A document recording details of fuel delivered, including the supplier, quantity, density and sulphur content. It is retained on board for three years, with a representative sample kept for at least 12 months.

Q129. What are the EEDI, EEXI and CII? EEDI is the Energy Efficiency Design Index for new ships. EEXI applies to existing ships (technical efficiency). CII is the Carbon Intensity Indicator, with annual operational ratings from A to E.

Q130. What is a SEEMP? Ship Energy Efficiency Management Plan, a ship-specific plan to improve energy efficiency in operation. It now includes CII requirements.

Q131. What is the Ballast Water Management Convention? It requires ships to manage ballast water to prevent the spread of harmful aquatic organisms. Standard D-1 is ballast exchange. D-2 is the performance standard using a treatment system.

Q132. What are the ballast exchange parameters (D-1)? Exchange at least 200 nm from the nearest land and in water at least 200 m deep, or where not possible, at least 50 nm from land and 200 m deep.

Q133. What are the documents for ballast water management? Ballast Water Management Plan, Ballast Water Record Book, and International Ballast Water Management Certificate.

Q134. What is the AFS Convention? The International Convention on the Control of Harmful Anti-Fouling Systems on Ships, banning organotin (TBT) compounds in anti-fouling paints. An AFS certificate is needed for larger ships.

Q135. What is the Hong Kong Convention? The convention for safe and environmentally sound recycling of ships. Each ship must carry an Inventory of Hazardous Materials (IHM).

Q136. What are the rules on ozone-depleting substances? Deliberate emissions are prohibited under Annex VI. New installations of ozone-depleting substances are prohibited. Ships keep a record of refrigerant use.

Q137. What is the standard discharge connection? A universal flange connection (International shore connection for oily residues) for transferring oily residues to port reception facilities.

Q138. How is sludge handled? Sludge is retained in a sludge tank, incinerated in an approved incinerator, or discharged ashore to reception facilities. It is recorded in the ORB Part I.

Q139. How is an oil pollution incident reported? In accordance with MARPOL Protocol I, the Master reports promptly to the nearest coastal State, with details of the ship, position, time, type and quantity of the oil, and weather.

Q140. What are the first actions in the event of an oil spill on deck or overboard? Stop the operation, raise the alarm, shut valves, stop the leak, contain and clean up using SOPEP equipment, notify the coastal State and the company as per SOPEP.

Q141. What are the precautions when bunkering? Bunkering plan, checklists, sounding of tanks, plugging of scuppers, spill containment, SOPEP equipment ready, communication, and personnel in position. Stop immediately on any leak.

Q142. What is a Port State Control inspection regarding pollution? PSC officers may inspect MARPOL documents, record books, equipment (OWS, ODME), and operational procedures, and may detain the ship for serious deficiencies.

Q143. What is the Polar Code? The International Code for Ships Operating in Polar Waters, with safety and pollution prevention provisions. Ships need a Polar Ship Certificate and a Polar Water Operational Manual.

Q144. What is a MARPOL Annex VI Ship Fuel Oil Consumption Data collection (DCS)? Ships of 5,000 GT and above report annual fuel oil consumption data to the flag State, which transfers it to the IMO database.

Q145. What are Garbage Record Book entries? Date, time, position, category of garbage, estimated quantity discharged to sea, or to reception facilities, or incinerated. Signed by the officer in charge and the Master.

## Section 6: ISM, ISPS, MLC, STCW, Surveys and Port State Control

Q146. What is the ISM Code? The International Safety Management Code, mandatory under SOLAS Chapter IX. It gives an international standard for safe management and operation of ships and for pollution prevention.

Q147. What are the objectives of the ISM Code? To ensure safety at sea, prevent human injury or loss of life, and avoid damage to the environment and property. It requires safe practices, safeguards against all risks, and continuous improvement of safety management skills.

Q148. What are the functional requirements of an SMS? Safety and environmental protection policy, instructions and procedures, defined levels of authority and communication, accident and non-conformity reporting, emergency preparedness, and internal audits and management review.

Q149. What are the DOC and SMC? The Document of Compliance is issued to the company and is valid for five years with annual verification. The Safety Management Certificate is issued to the ship, valid for five years with an intermediate verification.

Q150. Who is the Designated Person Ashore (DPA)? The person who provides a direct link between the company and the ship, with access to the highest level of management, and monitors safety and pollution prevention aspects of operations.

Q151. What is the Master's overriding authority under ISM? The Master has the overriding authority and responsibility to make decisions on safety and pollution prevention, and to request the company's assistance as necessary.

Q152. Define non-conformity, major non-conformity and observation. A non-conformity is an observed situation where objective evidence indicates a failure to fulfil a specified requirement. A major non-conformity is an identifiable deviation that poses a serious threat to safety or the environment. An observation is a statement of fact made during an audit.

Q153. What are internal audits and management review? The company conducts internal safety audits on board and ashore at intervals not exceeding 12 months, and management reviews the SMS to evaluate its effectiveness.

Q154. Why is near-miss reporting important? It identifies hazards before accidents occur, provides data for corrective and preventive action, and supports continuous improvement.

Q155. What does the ISM Code say about maintenance? The company must ensure the ship is maintained in conformity with the rules and regulations, with inspections at appropriate intervals, non-conformities reported with possible causes, corrective action taken and records maintained. Critical equipment must be identified and tested.

Q156. What is a permit-to-work system? A formal procedure controlling hazardous work (hot work, enclosed space entry, work aloft, working over the side, electrical isolation), with checks, authorizations and records.

Q157. What are the steps of a risk assessment? Identify the hazards, assess the risk (likelihood and severity), put controls in place using the hierarchy of control, communicate, implement and review.

Q158. What is the ISPS Code? The International Ship and Port Facility Security Code under SOLAS XI-2, setting out security requirements for ships and port facilities.

Q159. What are the three security levels? Level 1: normal, minimum protective measures. Level 2: heightened, additional measures for a period of time. Level 3: exceptional, specific measures when an incident is probable or imminent.

Q160. Who are the CSO, SSO and PFSO? Company Security Officer, Ship Security Officer (responsible to the Master for security on board and implementing the SSP), and Port Facility Security Officer.

Q161. What is a Declaration of Security? An agreement between a ship and a port facility (or another ship) specifying the security measures each will apply during an interface.

Q162. What is SSAS? Ship Security Alert System: a covert alert that transmits a ship-to-shore security alert to the competent authority, identifying the ship and its position, without raising alarms on board.

Q163. What is the validity of the ISSC? International Ship Security Certificate, valid for up to five years, with an intermediate verification between the second and third anniversary.

Q164. How do you deal with stowaways? Search the ship, secure and treat them humanely, keep them safe and provide food and medical attention, report to the company, P&I Club and authorities, and follow the Guidelines on allocation of responsibilities (FAL Convention).

Q165. What are the main measures against piracy? Follow Best Management Practices (BMP), register with the reporting centres, harden the ship (razor wire, fire hoses, lighting), increase lookouts, plan a citadel, use a safe speed and alter course, and report through SSAS.

Q166. What is the Maritime Labour Convention 2006? The 'seafarers' bill of rights', setting minimum standards for working and living conditions, including employment agreements, wages, hours of work and rest, accommodation, food, health care and repatriation.

Q167. What are the minimum hours of rest? At least 10 hours of rest in any 24-hour period and 77 hours in any 7-day period. Rest may be divided into no more than two periods, one at least six hours, and the interval between periods must not exceed 14 hours.

Q168. What are the DMLC Parts I and II? The Declaration of Maritime Labour Compliance Part I is issued by the flag State (national requirements), and Part II is prepared by the shipowner (measures to ensure continuing compliance). They accompany the Maritime Labour Certificate.

Q169. What is the validity of the Maritime Labour Certificate? Up to five years, with an intermediate inspection between the second and third anniversary.

Q170. What is STCW? The International Convention on Standards of Training, Certification and Watchkeeping for Seafarers, 1978 (as amended, including the 2010 Manila amendments), setting minimum competence standards for seafarers.

Q171. What are the STCW rules on alcohol? The limit for watchkeepers is a blood alcohol content of 0.05% (or 0.25 mg/L breath), and no alcohol may be consumed within four hours before the start of a watch. Many companies have a stricter policy.

Q172. What certificates must a cargo ship carry? Certificate of Registry, Cargo Ship Safety Construction, Safety Equipment and Safety Radio certificates (or a combined certificate), International Load Line, IOPP, SMC, ISSC, MLC, IAPP, and others by ship type and flag.

Q173. What is the Harmonised System of Survey and Certification? A system that aligns the survey and certificate dates of the SOLAS, Load Line and MARPOL certificates so they can be surveyed at the same time.

Q174. What are the types of survey? Initial, annual, intermediate, periodical, renewal and additional (occasional) surveys.

Q175. What is Port State Control? Inspection of foreign ships in a port by the port State to verify compliance with international conventions. Regional MOUs include Paris, Tokyo and Indian Ocean.

Q176. What happens during a PSC inspection? An initial inspection checks certificates and the general condition. If clear grounds are found, a more detailed inspection follows. Serious deficiencies may result in detention until rectified.

Q177. What are the flag State's responsibilities? Registration, survey and certification, enforcement of conventions, investigating casualties, manning and training standards, and delegating authority to recognised organisations.

Q178. What is a Recognised Organisation (RO)? A classification society authorised by the flag State to carry out statutory surveys and issue certificates on its behalf.

Q179. What is the difference between class and statutory surveys? Class surveys verify that the ship is built and maintained to class rules (for insurance and technical standards). Statutory surveys check compliance with the conventions and flag State laws.

Q180. Which Indian legislation applies to Indian ships? The Merchant Shipping Act of India with its rules and DG Shipping circulars and notices (check the Act currently in force, as the 1958 Act has been replaced), along with the Indian Ports and Marine Acts.

## Section 7: Law, Casualties, Medical Care and Crew Management

Q181. What entries are made in the Official Log Book? Births, deaths, marriages, offences and punishments, illness and injury, collisions, crew changes, drills, load line inspections and other events required by flag State law.

Q182. What are the Master's duties after an accident? Save life, protect the ship and environment, report to the company, flag State and coastal State, preserve evidence, make a log entry, note a protest if required, and cooperate with investigators.

Q183. What is a marine casualty investigation? An investigation under the IMO Casualty Investigation Code into the causes of a casualty, to establish the facts and prevent recurrence, not to apportion blame.

Q184. What are Lloyd's Open Form and the 'no cure, no pay' principle? LOF is a standard salvage contract in which the salvor is paid only for a successful result. SCOPIC clauses allow payment for tasks where environmental damage is threatened.

Q185. What are the Master's duties when a salvage offer is made? The Master acts on behalf of the owners and cargo owners and must act in their interest. Consult the company and the Club if possible, and sign a salvage contract only with their authority unless the danger is immediate.

Q186. What is the right to limit liability (LLMC)? The Convention on Limitation of Liability for Maritime Claims allows shipowners to limit liability to an amount based on the ship's tonnage, except in cases of intent or recklessness.

Q187. What certificates relate to pollution liability? CLC (oil pollution civil liability) certificates for oil tankers, Bunkers Convention certificate for ships over 1,000 GT, and Nairobi Wreck Removal certificate, usually evidenced by P&I Club blue cards.

Q188. What is a Sea Protest? A sworn statement by the Master and crew before a notary or consul declaring that damage was caused by circumstances beyond their control (for example, heavy weather).

Q189. What is the Maritime Declaration of Health and free pratique? A declaration on the health of persons on board, submitted to port health authorities. Free pratique is permission for a ship to enter a port, embark and disembark, and load and discharge after health clearance.

Q190. What medical guides must be carried on board? The International Medical Guide for Ships (or equivalent national medical guide), MFAG, and the ship's medicine chest scale and list, as required by the flag State.

Q191. How do you obtain medical advice at sea? Through Radio Medical Advice from Telemedical Assistance Services (TMAS) via radio or satellite, giving a clear description of the patient and the symptoms.

Q192. What is the CPR sequence for an adult? Check for danger and response, call for help, open the airway and check breathing, then give 30 chest compressions (100–120 per minute, 5–6 cm depth) followed by 2 rescue breaths, and use an AED if available.

Q193. What is the first aid for burns? Cool the burn with running water for at least 10 to 20 minutes, remove jewellery and loose clothing near the burn (but not stuck material), cover loosely with a clean non-stick dressing, and seek medical advice.

Q194. What are the signs and treatment of hypothermia? Shivering, confusion, slurred speech and drowsiness. Remove from the cold, remove wet clothing, insulate and warm gradually, handle gently and give warm drinks if conscious.

Q195. What is the treatment of severe bleeding? Apply direct pressure with a clean pad, raise the limb if there is no fracture, apply a firm bandage and keep the casualty lying down. Check for shock.

Q196. How do you deal with a suspected fracture? Immobilise the limb in the position found, support above and below the break, control bleeding and treat for shock, and seek medical advice. Do not move the casualty unless necessary.

Q197. What are the entry requirements for enclosed spaces? Permit to work, ventilation and atmosphere testing (O2 20.9%, flammable gas below 1% LFL, and toxic gases below limits), a standby person, communications, rescue equipment and approval by the Master or officer.

Q198. What leadership qualities are important for a Chief Mate? Clear communication, decisiveness, fairness, setting an example, delegating and supervising, motivating the team, and managing conflict and fatigue.

Q199. What is Bridge/Crew Resource Management? A set of skills for using all available resources (people, equipment and information) to achieve safe operations, including communication, situational awareness, teamwork, and decision-making.

Q200. How do you manage fatigue on board? Monitor and record hours of rest, plan work to allow adequate rest, avoid excessive overtime, provide a suitable environment, and encourage reporting of fatigue as a safety risk.