SECOND MATE (FG) ORALS

Question Bank: Functions 1, 2 and 3

Indian MMD Oral Examination Preparation

FUNCTION 1: Navigation at the Operational Level: 200 questions

FUNCTION 2: Cargo Handling and Stowage at the Operational Level: 200 questions

FUNCTION 3: Controlling the Operation of the Ship and Care for Persons on Board: 226 questions

Notes

•  Each function is numbered serially from 1. The questions in each section run on from the previous section.

•  Functions 1 and 2 come from the uploaded file, re-ordered, renumbered and with broken formulas repaired. Their technical content was not re-checked.

•  The uploaded file’s Function 3 stopped partway through its fourth section. It is kept as Questions 1–61, and the remaining questions are added after it with duplicates removed.

•  Answers marked [Verify] contain figures that should be checked against the latest consolidated SOLAS, MARPOL or Load Line texts before the exam.

•  Indian law: the Merchant Shipping Act 2025 replaced the 1958 Act (in force from 15 March 2026 according to a legal commentary). Confirm what your MMD examiner expects.

# FUNCTION 1: Navigation at the Operational Level

## 1. Bridge Watchkeeping & Handover Procedures (Q1–20)

1.	When taking over a night bridge watch, how do you prepare your vision?

Ans: Arrive at least 15–20 minutes early to allow rod cells to adapt to low-light conditions.

2.	What checks are prioritized before accepting the watch from the outgoing officer?

Ans: Verify ship's position, intended track, gyro/magnetic courses, speed, traffic, weather, and standing/night orders.

3.	What if a collision avoidance maneuver is underway during watch handover?

Ans: Defer the handover until the maneuver is fully completed and the vessel is clear of immediate danger.

4.	What if the relieving officer appears unfit for duty?

Ans: Refuse to hand over the watch, maintain bridge control, and notify the Master immediately.

5.	Under what circumstances may the Officer of the Watch (OOW) leave the bridge?

Ans: Only when properly relieved by a qualified officer or the Master.

6.	Who remains responsible for safe navigation when the Master arrives on the bridge?

Ans: The OOW remains responsible until the Master explicitly states they are assuming the conn and it is logged.

7.	When can a sole lookout be permitted during navigation?

Ans: Only during daylight hours after assessing visibility, traffic density, and proximity to hazards.

8.	How does the OOW use the Master's Night Order Book?

Ans: Read, understand, and sign it before taking the night watch, following all specific call-out criteria.

9.	When must you call the Master immediately without hesitation?

Ans: Upon restricted visibility, traffic distress/confusion, equipment failure, unexpected land/buoy sighting, or failure to maintain track.

10.	What are the primary duties of a helmsman vs. a lookout?

Ans: A helmsman steers the ordered course and cannot act as lookout; the lookout focuses solely on visual/auditory situational awareness.

11.	How frequently should you switch between auto-pilot and hand steering?

Ans: Test manual steering at least once per watch and well before entering congested waters or heavy traffic.

12.	What checks are performed during an anchor watch?

Ans: Monitor swinging circle, GPS anchor drag alarm, visual transit bearings, radar range rings, cable tension, and passing traffic.

13.	How do you detect if your vessel is dragging anchor?

Ans: Changing beam transit bearings, position moving outside the calculated swinging circle, and irregular vibrations/jerks on the cable.

14.	What is the calculation for the radius of an anchor swinging circle?

Ans: Length of cable paid out plus the distance from the hawsepipe to the bridge/stern, converted to cables/meters.

15.	What bridge checks are done before departure from port?

Ans: Synchronize clocks, test steering gear, whistle, telegraphs, comms, nav lights, radars, ECDIS, and verify draft/stability data.

16.	How is the engine movement book (bell book) maintained?

Ans: Record exact times of telegraph orders, thruster usage, pilot boarding/disembarking, and critical navigational landmarks.

17.	What are your duties when navigating in coastal and congested waters?

Ans: Use largest scale charts, plot frequent positions by multiple independent methods, and keep engines ready for immediate maneuver.

18.	How do you monitor weather during a routine watch?

Ans: Log barometric pressure tendencies, wind direction/force, sea state, visibility, and check incoming meteorological warnings.

19.	What is Bridge Resource Management (BRM)?

Ans: Effective coordination of all human, informational, and technical equipment resources on the bridge to eliminate single-person errors.

20.	What is an error chain in bridge watchkeeping?

Ans: A sequence of minor oversights, distractions, or misjudgments that compound toward a navigational incident unless broken early.

## 2. Passage Planning & Chart / ECDIS Management (Q21–40)

21.	What are the four distinct stages of passage planning?

Ans: Appraisal, Planning, Execution, and Monitoring.

22.	What happens during the Appraisal stage?

Ans: Gather all relevant publications, charts, weather routing data, port guides, draft limitations, and nav warnings.

23.	What is marked on charts during the Planning stage?

Ans: No-go areas, margins of safety, track lines, wheel-over points, abort points, parallel indices, and reporting lines.

24.	What is a Point of No Return (Abort Point)?

Ans: The final position along a channel where a vessel can still safely turn around or anchor if proceeding becomes unsafe.

25.	What is a Wheel-Over Point?

Ans: The advance position before an alteration waypoint where helm is applied so the vessel settles cleanly onto the new leg.

26.	How do you determine No-Go Areas on a chart?

Ans: Mark contours equal to or less than the vessel's dynamic draft plus required under-keel clearance (UKC), adjusted for tide.

27.	What is the difference between RNC and ENC?

Ans: Raster Navigational Charts are scanned digital images of paper charts; Electronic Navigational Charts are vector databases supporting smart alarms.

28.	What are the four critical safety depth/contour settings on ECDIS?

Ans: Safety Depth, Safety Contour, Shallow Contour, and Deep Contour.

29.	What happens if your desired Safety Contour is unavailable on the ENC?

Ans: The system automatically defaults to the next deeper available contour line for safety.

30.	What are the mandatory sensor inputs for an operational ECDIS?

Ans: Position (GNSS), Heading (Gyro), and Speed through water (Log).

31.	What does CATZOC indicate on an electronic chart?

Ans: Category of Zone of Confidence; it uses star symbols (A1 to D, and U) to show hydrographic survey accuracy for depth and position.

32.	How do you verify a paper chart is fully corrected up to date?

Ans: Check the Cumulative List of Notices to Mariners and verify the last weekly Notice number logged in the bottom-left corner.

33.	What are Temporary and Preliminary (T&P) Notices?

Ans: Time-limited or early-stage hazard notifications plotted in pencil on paper charts or managed as user overlays in ECDIS.

34.	What information is contained in Admiralty Sailing Directions (Pilot Books)?

Ans: Detailed coastal descriptions, port entry procedures, local winds/currents, pilotage, and anchorages not fully shown on charts.

35.	What are the volumes of the Admiralty List of Radio Signals (ALRS)?

Ans: Vol 1: Maritime Radio Stations; Vol 2: Radio Aids/Time/DGPS; Vol 3: Weather/Safety Info; Vol 4: Met Observation; Vol 5: GMDSS; Vol 6: Pilot/Port Ops.

36.	What information is found in the Mariner's Handbook (NP 100)?

Ans: General maritime guidance on chart usage, tides, currents, ice navigation, sea meteorology, and buoyage.

37.	What is a Routeing Chart and when do you use it?

Ans: Monthly ocean charts showing prevailing winds, currents, ice limits, load line zones, and recommended ocean tracks during Appraisal.

38.	What is Ocean Passages for the World (NP 136)?

Ans: A publication detailing standard global ocean routes for power and low-powered vessels considering seasonal weather and currents.

39.	What is a Clearing Bearing?

Ans: A charted bearing line to an identifiable object marking the boundary between safe water and shoal/danger areas.

40.	How does the ECDIS look-ahead safety frame work?

Ans: An adjustable time/distance sector ahead of the vessel that scans vector chart data to trigger anti-grounding and hazard alarms.

## 3. Steering, Gyro & Magnetic Compass Operations (Q41–60)

41.	What are the two fundamental properties of a free gyroscope?

Ans: Gyroscopic inertia (rigidity in space) and precession (tilting in response to applied torque).

42.	How is a free gyroscope converted into a north-seeking gyro compass?

Ans: By makes it north-seeking (control element via gravity/mercury ballistic) and damped (settling element to stop oscillation).

43.	Why does a traditional gyro compass become unreliable at very high latitudes?

Ans: The horizontal directive force diminishes with the cosine of latitude, becoming too weak near the geographic poles.

44.	What is Course and Speed (Steaming) Error in a gyro compass?

Ans: Error induced by the ship's northward or southward speed component combining with Earth's rotation vector.

45.	What is Ballistic Deflection Error?

Ans: Temporary gyro error caused by rapid changes in north-south speed accelerating the mercury/gravity control system.

46.	What is Rolling Error on a gyro compass?

Ans: Error produced on intercardinal headings in heavy seas due to lateral accelerations acting on the sensitive element.

47.	Define Magnetic Variation.

Ans: The angle between True North and Magnetic North at a given place, caused by Earth's geomagnetic field and changing slowly each year.

48.	Define Magnetic Deviation.

Ans: The angle between Magnetic North and Compass North caused by the vessel's own induced and permanent shipboard magnetism.

49.	What is the purpose of the Flinders Bar on a magnetic compass binnacle?

Ans: A vertical soft-iron bar placed fore or aft of the binnacle to compensate for induced magnetism from vertical soft iron structures.

50.	What is the purpose of Kelvin's Balls (Quadrantal Spheres)?

Ans: Two soft-iron spheres mounted port and starboard on the binnacle to correct quadrantal deviation from horizontal soft iron.

51.	What is Heeling Error in a magnetic compass?

Ans: Deviation oscillation caused when the ship rolls or lists, bringing vertical magnetic components into the horizontal compass plane.

52.	How is Heeling Error corrected?

Ans: By adjusting permanent vertical magnets inside a central brass bucket directly beneath the compass bowl.

53.	What liquid is used inside a modern wet card magnetic compass bowl?

Ans: A mixture of distilled water and pure alcohol (or clear low-viscosity mineral oil) to prevent freezing and dampen card movement.

54.	How do you remove an air bubble from a wet magnetic compass bowl?

Ans: Invert the bowl on its gimbals, unscrew the expansion chamber filling plug, top up with distilled water/alcohol mix, and seal.

55.	When is a vessel required to swing the compass and produce a new Deviation Card?

Ans: After dry-docking, structural repairs, lightning strikes, carrying magnetic cargoes, or when recorded deviations become excessive.

56.	How often must compass errors be determined and logged at sea?

Ans: At least once every watch when conditions permit, and after every major course alteration.

57.	What are the operational testing requirements for steering gear before departure?

Ans: Within 12 hours before departure, test main/auxiliary steering gear, remote controls, bridge indicators, and power failure alarms.

58.	What is the SOLAS performance requirement for main steering gear?

Ans: Must put the rudder from 35° on one side to 35° on the other cleanly, and from 35° on either side to 30° on the other within 28 seconds at maximum service speed.

59.	What is the performance requirement for auxiliary steering gear?

Ans: Must move the rudder from 15° on one side to 15° on the other within 60 seconds while running at half speed or 7 knots (whichever is greater).

60.	How do you execute an emergency change-over to local steering in the steering flat?

Ans: Inform Master and Engine Control Room, engage trick wheel/solenoid pins in the steering flat, establish bridge comms, and steer via emergency repeater.

## 4. Radar, ARPA & Bridge Instruments (Q61–80)

61.	What is the difference between X-band and S-band marine radars?

Ans: X-band (9 GHz, 3 cm) gives superior target definition and triggers SARTs; S-band (3 GHz, 10 cm) penetrates heavy rain and sea clutter better.

62.	What is Pulse Repetition Frequency (PRF) in radar?

Ans: The number of transmitted pulses per second; lower PRF is used on long range scales, higher PRF on short ranges.

63.	How does Pulse Length affect minimum detection range?

Ans: Short pulses improve range discrimination and minimum detection range; long pulses put more energy on distant targets.

64.	What are Side Echoes on a radar display?

Ans: False echoes appearing as arcs at the same range on either side of a strong close target, caused by antenna side lobes.

65.	What are Multiple Echoes on radar?

Ans: False targets appearing at regular intervals behind a close, high-sided ship due to energy bouncing repeatedly between hulls.

66.	What is an Indirect (Shadow) Echo?

Ans: A false target appearing on the bearing of an own-ship obstruction (like a mast or funnel) when radar pulses reflect off it toward a real target.

67.	How do you carry out a Performance Monitor test on radar?

Ans: Activate the monitor (plumehole/sun-ray pattern) and measure the plume length against the manufacturer's commissioning baseline to detect magnetron/receiver drop-off.

68.	What is Parallel Indexing (PI) on radar?

Ans: Drawing a fixed reference line on the radar screen parallel to the planned track at a set perpendicular distance from a fixed coastal radar target.

69.	Why must ARPA be sea-stabilized (water-stabilized) for collision avoidance?

Ans: Water stabilization uses speed through water (log) to compute true target aspect and heading; ground stabilization (GPS) distorts aspect in strong currents.

70.	When is ground-stabilization preferred on radar?

Ans: During coastal navigation, anchoring, or parallel indexing where movement over the seabed relative to fixed landmarks is monitored.

71.	What is Target Swap in ARPA?

Ans: When two tracked targets pass close to each other or through clutter and the ARPA tracking gate transfers vectors between them.

72.	What are the basic operational alarms on an ARPA unit?

Ans: CPA/TCPA violation alarm, Guard Zone intrusion alarm, Lost Target alarm, and System/Sensor failure alarm.

73.	What is the principle of an Echo Sounder?

Ans: Measures the time interval for an acoustic pulse (typically 50–200 kHz) to travel from the hull transducer to the seabed and return, using ~1500 m/s sound speed.

74.	What are the main operational errors of an Echo Sounder?

Ans: Velocity of sound error (salinity/temp variations), Pythagoras error (separated transducers in shallow water), aeration, and multiple bottom echoes.

75.	What is Phasing Error on a traditional recording echo sounder?

Ans: Incorrect depth reading caused by misaligning the range phase dial or incorrect stylus/belt rotational speed.

76.	What is a Janus Configuration in a Doppler Speed Log?

Ans: Four transducers angled fore, aft, port, and starboard that cancel out vertical ship pitching and heaving errors when measuring speed.

77.	What parameters are recorded by a Voyage Data Recorder (VDR)?

Ans: Date/time, GPS position, speed, heading, bridge audio, VHF voice, radar/ECDIS screen captures, echo sounder, alarms, rudder/engine orders, and wind.

78.	Why and how do you preserve VDR data after an incident?

Ans: To prevent the continuous loop from overwriting evidence; press the emergency backup/save button within the retention window and notify the Master.

79.	What is the difference between AIS Class A and Class B?

Ans: Class A is mandatory on SOLAS ships with higher transmit power (12.5W) and dynamic/voyage updates; Class B is lower power (2W) for smaller craft.

80.	Why should AIS never be used as the sole basis for collision avoidance?

Ans: Not all vessels carry AIS, targets may turn it off, and transmitted heading/position relies on the other vessel's potentially faulty sensors.

## 5. Collision Avoidance & Traffic Situations (Q81–100)

81.	When do the Steering and Sailing Rules apply?

Ans: Some apply in any condition of visibility, some only when vessels are in sight of one another, and one specific rule applies strictly in restricted visibility.

82.	How do you determine if Risk of Collision exists?

Ans: Take a series of compass bearings of an approaching vessel; if the bearing remains steady or nearly steady with decreasing range, risk exists.

83.	Can risk of collision exist even if the compass bearing is noticeably changing?

Ans: Yes, particularly when approaching a very large vessel, a tow, or any vessel at very close range.

84.	What factors determine Safe Speed?

Ans: Visibility, traffic density, stopping distance/turning ability, background shore lights, wind/sea/current state, draft vs. depth, and radar characteristics.

85.	What actions must a vessel take along a Narrow Channel?

Ans: Keep as near to the outer limit on her starboard side as safe and practicable, avoid impeding vessels confined to the channel, and avoid anchoring if possible.

86.	How do you join or leave a Traffic Separation Scheme (TSS)?

Ans: Join or leave at the termination of a lane, or from either side at as small an angle to the general direction of traffic flow as practicable.

87.	How must a vessel cross a traffic lane in a TSS?

Ans: Avoid crossing if possible, but if obliged to do so, cross on a heading as nearly as practicable at right angles to the general traffic flow.

88.	Does a vessel constrained by her draught have unconditional right of way?

Ans: No; while power-driven, sailing, and fishing vessels must avoid impeding her safe passage, she must navigate with extreme caution aware of her condition.

89.	Who gives way when a fishing vessel is overtaking a power-driven vessel?

Ans: The fishing vessel must keep clear because the overtaking obligation overrides standard vessel hierarchy.

90.	When does an overtaking situation cease to exist?

Ans: Never until the overtaking vessel is finally past and clear; subsequent bearing changes cannot convert it into a crossing situation.

91.	What action do you take in a head-on encounter with another power-driven vessel in sight?

Ans: Alter course substantially to starboard so each vessel passes cleanly on the port side of the other.

92.	In a crossing situation, why is the stand-on vessel discouraged from altering course to port for a vessel on her own port side?

Ans: Because the give-way vessel on the port side will likely alter to starboard, resulting in both vessels turning into the same water.

93.	What are the three stages of action for a stand-on vessel?

Ans: Maintain course and speed; may take action by her maneuver alone when the give-way vessel fails to act; must take best anti-collision action when action by give-way alone cannot prevent collision.

94.	In restricted visibility, you detect a target by radar alone forward of the beam with a close CPA—what alterations are avoided?

Ans: Avoid altering course to port for a target forward of the beam (other than a vessel being overtaken).

95.	In restricted visibility, what alteration is avoided for a target abeam or abaft the beam?

Ans: Avoid altering course toward a vessel abeam or abaft the beam.

96.	What must you do when you hear a fog signal apparently forward of your beam?

Ans: Reduce speed to the minimum at which the ship can be kept on course, or take all way off if necessary, and navigate with extreme caution.

97.	What is the priority hierarchy among vessels in sight of one another?

Ans: Not Under Command / Restricted in Ability to Maneuver, Constrained by Draught, Engaged in Fishing, Sailing, and Power-Driven (with Seaplanes/WIG craft lowest).

98.	Two sailing vessels approach with wind on different sides—who keeps clear?

Ans: The vessel with the wind on the port side keeps out of the way of the vessel with wind on the starboard side.

99.	Two sailing vessels have wind on the same side—who keeps clear?

Ans: The vessel to windward keeps out of the way of the vessel to leeward.

100.	What distinguishes a vessel Not Under Command (NUC) from one Restricted in Ability to Maneuver (RAM)?

Ans: NUC is caused by an exceptional circumstance (like engine/steering breakdown); RAM is caused by the nature of her work (like dredging, cable-laying, or replenishment).

## 6. Lights, Shapes & Sound Signals (Q101–120)

101.	What are the arc of visibility and color of a Masthead Light?

Ans: White light shining over an unbroken arc of 225° (112.5° on each side from right ahead).

102.	What are the arc of visibility and colors of Sidelights?

Ans: Green on starboard, red on port, each shining over an unbroken arc of 112.5° from right ahead to 22.5° abaft the beam.

103.	What is the arc of visibility of a Sternlight and a Towing Light?

Ans: 135° (67.5° from right aft on each side); Sternlight is white, Towing Light is yellow.

104.	What are the minimum visibility ranges of lights on a vessel 50 meters or more in length?

Ans: Masthead light: 6 miles; Sidelight, Sternlight, Towing light, and All-round lights: 3 miles.

105.	What day shape and lights are displayed by a vessel Not Under Command (NUC)?

Ans: Two all-round red lights in a vertical line (plus sidelights/sternlight only when making way) and two black balls by day.

106.	What day shapes and lights are displayed by a vessel Restricted in Ability to Maneuver (RAM)?

Ans: Red-White-Red all-round lights vertically (plus steaming lights when making way) and Ball-Diamond-Ball shapes vertically by day.

107.	What day shape and lights identify a vessel Constrained by her Draught (CBD)?

Ans: Three all-round red lights in a vertical line by night and a black cylinder by day, in addition to normal power-driven lights.

108.	How do you distinguish a trawler from a non-trawling fishing vessel at night?

Ans: A trawler displays Green over White all-round lights; a fishing vessel other than trawling displays Red over White all-round lights.

109.	What lights and shapes are shown when a towing length exceeds 200 meters?

Ans: Three masthead lights in a vertical line on the tug, and a black diamond shape by day on both tug and tow.

110.	What are the lights and day shapes for a vessel aground?

Ans: Anchor lights plus two all-round red lights in a vertical line (steaming lights off), and three black balls in a vertical line by day.

111.	What lights does a Pilot Vessel on duty display?

Ans: White over Red all-round lights in a vertical line near the masthead, plus sidelights/sternlight when underway or anchor lights at anchor.

112.	What are the standard dimensions and vertical spacing of day shapes?

Ans: Balls and cones have a minimum diameter/base of 0.6 m; cylinders are 0.6 m diameter by 1.2 m height; vertical spacing between shapes is at least 1.5 m.

113.	What are the maneuvering sound signals for vessels in sight of one another?

Ans: 1 short blast: altering to starboard; 2 short blasts: altering to port; 3 short blasts: operating astern propulsion.

114.	What are the overtaking sound signals in a narrow channel?

Ans: 2 prolonged + 1 short: intend to overtake on your starboard side; 2 prolonged + 2 short: on your port side; agreement reply is Prolonged-Short-Prolonged-Short.

115.	What is the fog signal for a power-driven vessel making way vs. underway but stopped?

Ans: Making way: 1 prolonged blast at intervals not exceeding 2 minutes; stopped: 2 prolonged blasts (2 seconds apart) every 2 minutes.

116.	What is the fog signal for NUC, RAM, CBD, Sailing, Fishing, or Towing vessels?

Ans: One prolonged blast followed by two short blasts at intervals not exceeding 2 minutes.

117.	What is the fog signal for a vessel at anchor vs. a vessel aground?

Ans: At anchor: rapid ringing of bell for 5s every minute (plus gong aft if ≥100m); Aground: 3 distinct bell strokes before and after the rapid ringing (plus gong if ≥100m).

118.	What sound signal may an anchored vessel give to warn an approaching ship?

Ans: One short, one prolonged, and one short blast (Morse "R") to warn of her position and possibility of collision.

119.	What signal is used when in doubt about the intentions or actions of another vessel in sight?

Ans: At least five short and rapid blasts on the whistle, supplemented if desired by at least five short and rapid flashes of light.

120.	List five recognized Distress Signals from Annex IV.

Ans: Red parachute rocket flare, orange smoke signal, continuous sounding of fog-horn, Mayday by radio, EPIRB/SART activation, or NC flags.

## 7. Buoyage Systems & Pilotage (Q121–140)

121.	What is the difference between IALA Buoyage Region A and Region B?

Ans: Region A (Europe, Asia, India, Australia) uses Red to Port when entering from seaward; Region B (Americas, Japan, Korea, Philippines) uses Red to Starboard.

122.	Describe a North Cardinal Mark and the action when sighting it right ahead while steering 000°.

Ans: Black over yellow body, two cones pointing up, continuous quick/very quick white flashes; safe water lies North, so check chart and alter to pass north or stay clear of the southern hazard.

123.	Describe an East Cardinal Mark and the action when sighting it dead ahead while steering 090°.

Ans: Black with a single broad horizontal yellow band, two cones base-to-base, 3 flashes every 5 or 10 seconds; safe water is to the East, meaning you are currently on the danger side—check chart/echo sounder and maneuver out immediately.

124.	Describe a South Cardinal Mark.

Ans: Yellow over black body, two cones pointing downward, 6 quick/very quick flashes followed by 1 long flash every 10 or 15 seconds; pass to the South.

125.	Describe a West Cardinal Mark.

Ans: Yellow with a single broad horizontal black band, two cones point-to-point (wineglass), 9 quick/very quick flashes every 10 or 15 seconds; pass to the West.

126.	Describe an Isolated Danger Mark.

Ans: Black with one or more broad horizontal red bands, topmark of two black spheres vertically, White Group Flashing (2) light.

127.	Describe a Safe Water Mark and its purpose.

Ans: Red and white vertical stripes, single red sphere topmark, White Isophase, Occulting, Long Flash (10s), or Morse "A" light; marks mid-channel or landfall.

128.	Describe a Special Mark.

Ans: Yellow body, single yellow 'X' (St. Andrew's cross) topmark, yellow light with rhythm distinct from white lights; marks cables, pipelines, or spoil grounds.

129.	Describe an Emergency Wreck Marking Buoy (EWMB).

Ans: Equal vertical blue and yellow stripes (4 to 8), upright yellow cross topmark, alternating Blue (1s) and Yellow (1s) flashes with 0.5s eclipses.

130.	What is a Preferred Channel Mark?

Ans: A modified lateral mark with a horizontal band of the opposite color indicating the main vs. secondary channel at a junction.

131.	When is a Combination Rig (accommodation ladder + pilot ladder) required for pilot boarding?

Ans: Whenever the freeboard from sea level to the point of access exceeds 9 meters.

132.	What are the key rigging specs for a Pilot Ladder?

Ans: Side ropes at least 18 mm manila; steps at least 40 cm wide hardwood steps spaced 31–35 cm apart, every 5th step a 1.8 m spreader, lowest step rubber, resting firmly against the hull.

133.	What equipment must be kept ready at the Pilot Boarding Station?

Ans: Lifebuoy with self-igniting light, heaving line, stanchions/bulwark ladder, adequate deck lighting, and a responsible officer with VHF communication to the bridge.

134.	What information is exchanged via the Master/Pilot Information Exchange Card?

Ans: Draft, UKC, air draft, engine/thruster specs, turning circles, anchor readiness, gyro error, tug allocation, and berth plan.

135.	What is the legal status of a Pilot on board?

Ans: The Pilot acts as a local advisor to the Master; the Master and OOW retain legal responsibility for safe navigation (except in the Panama Canal).

136.	What is displayed on the Wheelhouse Poster?

Ans: Manoeuvring characteristics in loaded and ballast conditions: turning circles, stopping distances, squat tables, and blind sector diagrams.

137.	What is the Pilot Card?

Ans: A concise booklet/sheet handed to the pilot upon boarding containing current draft, displacement, engine status, steering gear details, and equipment defects.

138.	What signals are associated with a Pilot Vessel?

Ans: By day when pilot is on board: Flag "H" (White/Red vertical); requesting a pilot: Flag "G" (Yellow/Blue vertical stripes).

139.	How do you monitor the vessel's progress when a Pilot is conning?

Ans: Continue plotting positions independently, cross-check helm and engine orders, monitor UKC, and challenge any doubtful action immediately.

140.	What is Drift Angle when turning?

Ans: The angle between the ship's fore-and-aft centerline and the tangent to her curved path; it is minimal near the pivot point forward and largest at the stern.

## 8. Celestial & Terrestrial Navigation Principles (Q141–160)

141.	What are the three adjustable errors of a marine Sextant in correct adjustment order?

Ans: 1st: Perpendicularity of index mirror; 2nd: Side error (horizon glass perpendicularity); 3rd: Index error (parallelism of mirrors).

142.	What is Collimation Error in a sextant?

Ans: When the optical axis of the telescope is not strictly parallel to the plane of the sextant arc.

143.	How do you check and remove Index Error on a sextant?

Ans: Clamp index arm at zero, view the true horizon or a star; if split, turn the second adjustment screw behind the horizon glass until aligned (or apply "On the arc, off; Off the arc, on").

144.	Define Celestial Amplitude and when it is observed.

Ans: The arc of the rational horizon between the Prime Vertical and the rising/setting body; observed when the Sun's center is on the rational horizon (lower limb ~half diameter above visible horizon).

145.	Define Celestial Azimuth.

Ans: The arc of the rational horizon (or angle at the zenith) measured clockwise from the observer's North/South celestial meridian to the vertical circle of the body.

146.	Explain how you obtain a Noon Position (Latitude by Meridian Altitude).

Ans: Calculate approximate Local Apparent Noon (LAN), begin observing the Sun just before culmination until it dips, apply Index Error, Dip, and Total Correction to get True Altitude, calculate Zenith Distance (90° −True Alt), and combine with Declination.

147.	What do you do if clouds obscure the Sun precisely at Meridian Passage?

Ans: Take an Ex-Meridian sight within ~25–30 minutes before or after culmination and apply Ex-Meridian tables from Norie's to reduce it to meridian altitude.

148.	What is the PZX Triangle?

Ans: The spherical navigational triangle formed on the celestial sphere by the Elevated Pole (P), Observer's Zenith (Z), and Celestial Body (X).

149.	What is the Equation of Time?

Ans: The time difference between Apparent Solar Time (true Sun) and Mean Solar Time (fictitious uniform Sun), tabulated twice daily in the Nautical Almanac.

150.	What are Civil, Nautical, and Astronomical Twilight?

Ans: When the Sun's center is 6°, 12°, and 18° below the rational horizon respectively; Nautical Twilight provides both visible stars and a clear horizon for sights.

151.	How many stars do you typically shoot during twilight sights and why?

Ans: 4 stars separated by roughly 90° in azimuth (or 3 stars 120° apart) to minimize systematic horizon/refraction errors and form a tight cocked hat.

152.	What is a Great Circle track vs. a Rhumb Line track?

Ans: A Great Circle is the shortest distance on a sphere (cuts meridians at changing angles); a Rhumb Line maintains a constant course angle (spiral toward poles).

153.	How are Great Circle and Rhumb Line tracks represented on Mercator vs. Gnomonic charts?

Ans: On Mercator, a Rhumb Line is straight and a Great Circle curves toward the pole; on Gnomonic, a Great Circle is a straight line.

154.	What is a Composite Great Circle route?

Ans: A Great Circle track modified with a limiting parallel of latitude to prevent the vessel from entering dangerous high-latitude ice or storm zones.

155.	What is a Running Fix?

Ans: Determining a position by taking two bearings of a single terrestrial object separated by a time interval and transferring the first position line forward by course and distance made good.

156.	What is Horizontal Sextant Angle (HSA) fixing and why is it accurate?

Ans: Measuring horizontal angles between three charted shore objects using a sextant and plotting via station pointer; it is completely independent of gyro/compass error.

157.	What is Vertical Sextant Angle (VSA) used for?

Ans: Measuring the vertical angle from the waterline to the top of a lighthouse of known charted height to find distance off via Norie's Tables.

158.	Why do you check if the base of a lighthouse is above or below the visible horizon during VSA?

Ans: Because standard VSA distance tables assume the object's base is between the observer and the visible horizon.

159.	What are Spring and Neap Tides?

Ans: Spring tides occur at New and Full Moon (Sun and Moon aligned, maximum tidal range); Neap tides occur at First and Third Quarters (Sun and Moon at 90°, minimum range).

160.	What is Chart Datum?

Ans: The low-water reference plane (typically Lowest Astronomical Tide - LAT) below which depths are charted and above which tidal heights and drying heights are measured.

## 9. Marine Meteorology & Tropical Cyclones (Q161–180)

161.	What are the corrections applied to a Precision Aneroid Barometer?

Ans: Index error (checked periodically) and Height above sea level correction (approx. +0.1 hPa per meter of bridge height).

162.	How do you correct Index Error on an Aneroid Barometer?

Ans: Compare with a calibrated port meteorological reference barometer and turn the small adjustment screw on the back casing using a screwdriver.

163.	What is Diurnal Variation of atmospheric pressure in the tropics?

Ans: A regular twice-daily pressure wave peaking at ~1000 and 2200 local time and dipping at ~0400 and 1600 local time by ~1.5 to 3 hPa.

164.	What are the advance warning signs of an approaching Tropical Revolving Storm (TRS)?

Ans: Long low swell from the storm center, failure of diurnal pressure variation (or drop >3 hPa below normal), lurid cirrus clouds/halos, and freshening gusty winds.

165.	State Buys Ballot's Law.

Ans: Face the true wind: in the Northern Hemisphere, the low-pressure center lies 90° to 135° on your right (in the Southern Hemisphere, to your left).

166.	How do you determine whether you are in the Dangerous or Navigable Semicircle of a TRS?

Ans: If the wind is veering (shifting clockwise), you are in the Right-Hand Semicircle; if backing (counter-clockwise), you are in the Left-Hand Semicircle (NH: Right = Dangerous; SH: Left = Dangerous).

167.	Define Path, Track, Vertex, and Vortex in a TRS.

Ans: Track is the route already traveled; Path is the predicted future trajectory; Vertex is the westernmost point of recurvature; Vortex is the calm central eye.

168.	What evasive action do you take in the Northern Hemisphere if caught in the Dangerous Semicircle?

Ans: Place the wind 1 to 4 points (10°–45°) on the starboard bow and make maximum safe speed, altering to starboard as the wind veers.

169.	What evasive action do you take in the Northern Hemisphere if in the Navigable Semicircle or Directly in the Path?

Ans: Put the wind about 4 points (45°) on the starboard quarter and make best speed away from the track.

170.	What evasive actions apply for a TRS in the Southern Hemisphere?

Ans: Dangerous Semicircle (Left-hand): keep wind on the port bow and alter to port as wind backs; Navigable/In Path: keep wind on the port quarter.

171.	Why is the Dangerous Semicircle more hazardous than the Navigable Semicircle?

Ans: Wind speeds combine additively with the storm's forward translation speed, and the storm tends to recurve directly over the dangerous quadrant.

172.	What is the difference between Advection Fog and Radiation Fog?

Ans: Advection fog forms at sea when warm, moist air flows over a cold ocean surface; Radiation fog forms over land on clear, calm nights and drifts into estuaries.

173.	What is Sea Smoke (Steam Fog)?

Ans: Fog formed when bitterly cold air passes over a significantly warmer sea surface, causing rapid evaporation and immediate condensation.

174.	How do you measure Relative Humidity and Dew Point using a Mason's Hygrometer?

Ans: Read Dry Bulb and Wet Bulb temperatures inside the Stevenson Screen, find the wet-bulb depression, and look up Dew Point and Relative Humidity in meteorological tables.

175.	How does the Dew Point reading help predict cargo sweating or fog?

Ans: If sea surface temperature approaches the air's Dew Point, advection fog is imminent; if hold steel cools below cargo air Dew Point, ship sweat forms.

176.	Compare Katabatic and Anabatic winds.

Ans: Katabatic wind flows downhill at night as cold, dense air drains off high coastal slopes; Anabatic wind flows uphill by day as sun-heated slopes warm the air.

177.	What are Geostrophic and Gradient Winds?

Ans: Geostrophic wind balances Pressure Gradient Force and Coriolis force along straight isobars; Gradient wind adds Centrifugal force along curved isobars.

178.	What causes the Southwest and Northeast Monsoons in the Indian Ocean?

Ans: Seasonal thermal low/high pressure shifts over the Asian landmass combined with cross-equatorial trade wind deflection due to Coriolis force.

179.	What is the Beaufort Wind Scale?

Ans: An empirical scale (Force 0 to 12) correlating observed sea state descriptions with wind speed in knots at 10 meters height.

180.	Under SOLAS, when is a Master obliged to transmit a Tropical Storm warning?

Ans: Whenever encountering winds of Beaufort Force 10 or above (or sub-freezing air with gale winds causing ice accretion) for which no storm warning has been received.

## 10. Navigational Emergencies, SAR & GMDSS (Q181–200)

181.	What are your immediate actions as OOW upon witnessing a Man Overboard (MOB)?

Ans: Release bridge wing MOB lifebuoy (light & smoke), put helm hard over toward the casualty (Williamson/Anderson turn), press GPS MOB button, post lookouts, sound 3 prolonged blasts ("O"), alert Master and Engine Room.

182.	Compare the Williamson Turn, Anderson Turn, and Scharnow Turn.

Ans: Williamson (60° then hard reverse until 20° short of reciprocal) is best at night/poor visibility to return onto exact track; Anderson (single 250°–270° turn) is fastest when casualty is in sight; Scharnow (240° turn) is for delayed discovery.

183.	Why might a MOB maneuver differ inside a congested TSS?

Ans: Blindly swinging across opposing traffic lanes risks collision; assess astern/adjacent traffic first and consider an Anderson turn or controlled crash-stop while alerting VTS.

184.	What are the standard IAMSAR Search Patterns and when is each used?

Ans: Expanding Square (single ship, datum well known); Sector Search (datum highly accurate, e.g., MOB); Parallel Track (large area, uncertain datum, 1 or more ships); Coordinated Creep (ship + aircraft).

185.	What contents are found inside IAMSAR Volume III?

Ans: Mobile Facilities manual carried on the bridge, covering rendering assistance, On-Scene Coordinator (OSC) duties, search patterns, and ship-aircraft comms.

186.	What action do you take if you sight a red parachute flare at sea?

Ans: Take a compass bearing and estimate distance, plot position, call the Master, check radar/AIS, log the event, contact MRCC/coast station, and prepare to proceed to assist.

187.	What are your immediate actions if your vessel runs aground?

Ans: Stop engines, sound general alarm, close watertight doors, display aground lights/shapes, call Master, take soundings around hull and all tanks/bilges, check for pollution, and fix position.

188.	Why shouldn't you immediately go full astern after running aground?

Ans: Astern propulsion may tear open damaged bottom plates, pull the ship off into deep water where she sinks, or draw mud/sand into engine cooling sea chests.

189.	What are your immediate actions following a collision with another vessel?

Ans: Stop engines, sound general alarm, close watertight doors, assess damage/stability, stand by the other vessel to save life, exchange ship name/port of registry/ports of call, and preserve VDR.

190.	What are the four GMDSS Sea Areas?

Ans: A1: VHF DSC coverage (~20–30 nm); A2: MF DSC coverage (~150 nm, excluding A1); A3: Inmarsat/Geostationary satellite belt (70° N to 70° S); A4: Polar regions beyond A1/A2/A3.

191.	How do you test a 9 GHz Radar SART and a 406 MHz EPIRB?

Ans: SART: Switch to 'Test' briefly while observing concentric rings/arcs on the X-band radar; EPIRB: Push spring-loaded 'Test' switch to watch self-test LED/strobe flash without transmitting a live alert.

192.	What are the daily, weekly, and monthly GMDSS checks?

Ans: Daily: DSC internal self-test, battery on-load/off-load voltage, printer paper; Weekly: DSC live test call to a coast station, reserve generator run; Monthly: EPIRB/SART/handheld VHF & battery specific gravity/condition.

193.	What is an On-Load vs. Off-Load test of the GMDSS Reserve Battery?

Ans: Off-load voltage is read with normal AC charging on/disconnected at rest (~24V); On-load test switches off the AC charger and keys a transmitter to ensure voltage drop remains within safe limits.

194.	How many NAVAREAs exist globally and which one covers India?

Ans: 21 global NAVAREAs under the World-Wide Navigational Warning Service; India is the Area Coordinator for NAVAREA VIII.

195.	What is the difference between AMVER and INSPIRES?

Ans: AMVER is the global US Coast Guard Automated Mutual-Assistance Vessel Rescue system; INSPIRES is the Indian Ship Position and Information Reporting System operated by the Indian Coast Guard for vessels in the Indian search and rescue region.

196.	How do you cancel an accidental false distress alert on VHF DSC or Inmarsat?

Ans: Switch off transmitter immediately, tune to Ch 16 (or relevant voice/text channel), broadcast a Mayday cancellation to all stations giving ship name, call sign, MMSI, and time of false alert, and inform nearest RCC.

197.	What is Squat and what causes it?

Ans: Bodily sinkage combined with trim change when moving through shallow water; water accelerating under the hull creates a Bernoulli low-pressure drop.

198.	What signs indicate your vessel has entered Shallow Water?

Ans: Increased bow and stern wave height, sluggish helm response, increased hull vibration, falling RPM, and muddy water churning astern.

199.	How is Squat calculated approximately and how do you minimize it?

Ans: For full-form ships: maximum squat (m) ≈ Cb × V² / 100 (V in knots), in open shallow water; roughly double in confined channels. Since squat varies with the square of speed, halving speed reduces squat to about one quarter.

200.	What determines whether a vessel trims by the bow or stern during Squat?

Ans: Block Coefficient (C b): full-bodied ships (C b >0.70, like tankers/bulkers) trim by the head; fine-lined ships (C b <0.70, like container vessels) trim by the stern.

# FUNCTION 2: Cargo Handling and Stowage at the Operational Level

## 1. General Cargo Operations, Stowage & Mate's Receipts (Q1–20)

1.	Define Stowage Factor (SF).

Ans: The volume occupied by a unit weight of cargo (expressed in m³ /MT or ft³ /ton) when normally stowed, excluding broken stowage.

2.	Define Broken Stowage and why it is added in cargo calculations.

Ans: Empty, unoccupied space lost between packages, frames, pillars, and dunnage; added as a percentage to SF to find the true volume required in a hold.

3.	Define Load Density of a deck or tank top.

Ans: The maximum permissible weight per unit area (MT/m²) that a structural deck or double-bottom tank top can safely support without deformation.

4.	How do you calculate the maximum height to which a homogeneous cargo can be piled?

Ans: Multiply the deck's permissible Load Density (MT/m²) by the cargo's Stowage Factor (m³ /MT).

5.	How do you load a heavy unit whose footprint exceeds the tank top's Load Density?

Ans: Lay heavy timber bearers/dunnage transversely across internal double-bottom floors and longitudinal girders to spread the weight over a larger area.

6.	What is the difference between Bale Capacity and Grain Capacity?

Ans: Grain Capacity measures to the outer shell plating and deck beams (for free-flowing cargo); Bale Capacity measures inside cargo battens and bottom of beams (for packaged cargo).

7.	What is the difference between Ship Sweat and Cargo Sweat?

Ans: Ship Sweat condenses on cold ship steel when warm, moist hold air is cooled in cold waters (ventilate if outside dew point is lower); Cargo Sweat condenses directly on cold cargo when warm, humid outside air is blown into the hold (restrict ventilation).

8.	State the "Three Degrees / Dew Point Rule" for cargo ventilation.

Ans: Ventilate when the outside air dew point is lower than the hold air dew point (or when outside air is at least 3°C cooler than cargo temperature for hygroscopic cargoes).

9.	How do you prepare a general cargo hold to load bagged rice?

Ans: Sweep, wash, and dry the hold, test bilges and cover strum boxes with burlap, ensure odour/insect free, lay clean dunnage in a double criss-cross layer for air circulation, and fit spar ceiling/kraft paper against steel sides.

10.	What is the purpose of Dunnage in a cargo hold?

Ans: Protects cargo from condensation and bilge moisture, distributes weight, creates friction against sliding, and provides channels for air ventilation.

11.	What are the precautions before lifting a Heavy Lift onboard?

Ans: Check stability (huge immediate rise in KG when hook takes weight—eliminate free surface), rig additional stays if needed, inspect rigging/padeyes, clear non-essential crew, and adjust ballast for heel.

12.	What is a Mate's Receipt?

Ans: A provisional acknowledgment signed by the Chief Officer confirming the quantity and apparent condition of cargo received onboard before the Bill of Lading is issued.

13.	What is the difference between a Clean and a Claused (Foul) Mate's Receipt?

Ans: A Clean receipt has no remarks on defects; a Claused receipt records specific observations such as torn bags, rust, or short count.

14.	What are the three primary functions of a Bill of Lading (B/L)?

Ans: It serves as a receipt for goods shipped, evidence of the contract of carriage, and a negotiable document of title to the goods.

15.	What does it mean that a Bill of Lading is "Negotiable"?

Ans: Ownership of the cargo can be legally transferred from one party to another while the ship is at sea simply by endorsing the B/L.

16.	What do you do at a discharge port if no one arrives with the Original Bill of Lading?

Ans: Never discharge or release cargo without sighting an endorsed Original B/L; notify the Master, P&I Club, and owners immediately.

17.	When does the Master lodge a Note of Protest?

Ans: After encountering heavy weather that may have damaged cargo, when charterers load improper cargo, or when shore delays/stevedore damage occur.

18.	What is a Cargo Manifest?

Ans: A comprehensive customs and commercial inventory listing all bills of lading, shipper/consignee details, marks, weights, and stowage locations on board.

19.	What are your duties as OOW while taking over a port cargo watch?

Ans: Check mooring lines, gangway, draft/trim, loading sequence plan, working hatches, ballast operations, fire/safety readiness, and communication with stevedore foreman.

20.	How do you prevent pilferage of high-value cargo in port?

Ans: Stow in lockable lockers or overstow with heavier cargo, maintain strict deck access control, tally packages continuously, and place watchmen inside the working hold.

## 2. Dry Bulk Cargoes & IMSBC Operational Principles (Q21–40)

21.	What are the three cargo groups defined in the IMSBC Code?

Ans: Group A: Cargoes that may liquefy; Group B: Cargoes possessing chemical hazards; Group C: Cargoes that neither liquefy nor possess chemical hazards.

22.	Can a cargo belong to both Group A and Group B?

Ans: Yes (e.g., certain coals or mineral concentrates that can both liquefy at high moisture and self-heat/emit toxic gases).

23.	Define Flow Moisture Point (FMP).

Ans: The percentage moisture content at which a flow state develops in a representative sample of Group A solid bulk cargo under laboratory compaction/vibration.

24.	Define Transportable Moisture Limit (TML).

Ans: The maximum moisture content of a Group A cargo considered safe for sea carriage without liquefaction; set at 90% of the Flow Moisture Point.

25.	What happens during Cargo Liquefaction?

Ans: Ship vibration and rolling compact the wet granular mass, raising pore water pressure until friction between solid particles vanishes and the cargo sloshes like a heavy liquid, causing sudden capsizing.

26.	How do you perform a Can Test (Spot Check) on board for a Group A cargo?

Ans: Fill a cylindrical metal can (approx. 0.5–1 L) with cargo, tap it sharply 25 times against a hard surface every 1–2 seconds; if free moisture or a glossy fluid surface appears, the cargo fails and requires laboratory testing.

27.	If the Can Test shows free water on top, can you load the cargo if the shipper's certificate says Moisture < TML?

Ans: No; stop loading that parcel, notify the Master, and demand an independent laboratory re-test because actual moisture may have increased from rain or poor stockpiling.

28.	How recently before loading must the TML and Moisture Content certificates be dated?

Ans: TML certificate within 6 months prior to loading; Actual Moisture Content test within 7 days prior to loading (and re-tested if exposed to rain).

29.	Define Angle of Repose.

Ans: The maximum angle between a horizontal plane and the cone slope obtained when a non-cohesive granular solid bulk cargo is poured freely onto a flat surface.

30.	Which cargo is more prone to dry shifting: Angle of Repose 28° or 38° ?

Ans: 28° is far more dangerous; under the IMSBC Code, non-cohesive cargoes with an angle of repose ≤30° flow like grain, and those ≤35° require thorough level trimming.

31.	Why do we trim solid bulk cargoes?

Ans: To reduce the likelihood of the cargo shifting transversely in a seaway and to distribute weight evenly across the tank top to minimize hull stresses.

32.	What is the BLU Code?

Ans: The Code of Practice for the Safe Loading and Unloading of Bulk Carriers, establishing standardized ship-shore safety checklists and loading/deballasting synchronization.

33.	Why must loading rate and deballasting rate be synchronized on a bulk carrier?

Ans: To prevent excessive Shear Forces (SF) and Bending Moments (BM) that could buckle or fracture the hull girder, and to stay within terminal air-draft limits.

34.	What information must the Shipper provide via the Bulk Cargo Shipping Declaration (BCSN)?

Ans: BCSN name, IMSBC Group, Stowage Factor, TML & Moisture Content (if Group A), Angle of Repose, chemical properties, and trimming requirements.

35.	What is the procedure before loading a high-density bulk cargo (SF≤0.56 m³ /MT)?

Ans: Verify hold mass curves in the Stability Booklet, decide between homogeneous vs. alternate hold loading, check tank top load density, and prevent free-fall dropping from high grab heights.

36.	What structural markings are welded inside holds of single-side-skin bulk carriers carrying high-density cargo?

Ans: Solid equilateral triangles (500 mm sides, painted red) on the shell plating if the ship has restrictions against alternate hold loading.

37.	Where are Water Ingress Detection Sensors located in a bulk carrier hold?

Ans: Two sensors in the aft part of each hold: one at 0.5 m above the inner bottom and one at 15% of hold depth (max 2.0 m), triggering visual/audible alarms on the bridge.

38.	What additional spaces require water ingress alarms under SOLAS Chapter XII?

Ans: Ballast tanks forward of the collision bulkhead and any dry/void space (like the bosun store) extending forward of the foremost cargo hold.

39.	How do you prepare cargo holds and bilges before loading a dusty bulk cargo like cement or alumina?

Ans: Dry sweep, wash, dry completely, seal bilge covers with burlap/tape after testing suction and non-return valves, and inspect hatch cover rubber packing via hose/ultrasonic test.

40.	How do you conduct a Draft Survey?

Ans: Read six drafts (Fwd, Mid, Aft on Port & Stbd), measure dock water density with a hydrometer, calculate Quarter Mean Draft, apply trim and list corrections, deduct ballast/bunkers/constants, and compare initial vs. final net displacement.

## 3. High-Risk Bulk Commodities (Coal, Iron Ore, Concentrates) (Q41–60)

41.	What are the two primary hazards of carrying Coal in bulk?

Ans: Emission of flammable Methane (CH₄) gas creating explosion risks, and self-heating/spontaneous combustion producing toxic Carbon Monoxide (CO) and depleting Oxygen.

42.	Where does Methane accumulate in a hold vs. Carbon Monoxide?

Ans: Methane is lighter than air and accumulates at the top under hatch covers; Carbon Monoxide has nearly the same density as air and mixes throughout.

43.	How do you ventilate a hold loaded with Coal?

Ans: Perform surface ventilation for the first 24 hours (and subsequently if methane >20% LEL) to remove methane, unless the coal is self-heating, in which case all ventilation must be sealed off.

44.	What instrument is used to monitor the atmosphere of a Coal cargo hold without opening hatches?

Ans: A 4-gas detector (CH₄%LEL, CO ppm, O₂%, H₂S) sampling through dedicated gas sampling ports, plus hold temperature pipes.

45.	At what temperature or CO threshold do you suspect Coal is self-heating?

Ans: If hold temperature exceeds 55°C or if Carbon Monoxide levels rise steadily toward 50 ppm.

46.	What action do you take if Coal self-heating is detected at sea?

Ans: Seal the hold completely (shut vents, tape hatch joints), never apply water into the hold (creates explosive water-gas), boundary cool adjacent bulkheads if needed, and seek expert shore/P&I advice.

47.	What are the hazards of carrying Iron Ore and Iron Ore Fines?

Ans: Very high bulk density (structural overstress/local tank top damage if dropped from height, very large GM causing violent rolling), and Iron Ore Fines are Group A (liquefaction hazard).

48.	What are the hazards of Direct Reduced Iron (DRI)?

Ans: Reacts with moisture/seawater to evolve explosive Hydrogen (H₂) gas and generate intense heat leading to auto-ignition and oxygen depletion.

49.	How is the atmosphere maintained inside a hold carrying DRI?

Ans: Holds are kept strictly dry, weather-tight, and inerted with Nitrogen (N₂) to keep oxygen below 5% and hydrogen below 1% (25% LEL).

50.	What are the hazards of Sulphur in bulk?

Ans: Easily ignited by friction/sparks from steel grabs, evolves toxic and corrosive Sulphur Dioxide (SO₂) gas upon burning, and forms corrosive sulphuric acid when wet with bilge water.

51.	How do you prepare holds for loading bulk Sulphur?

Ans: Limewash or apply protective coating to steelwork/bilges to prevent acid corrosion, isolate non-intrinsically safe electrical circuits, and use fine water fog/spray to suppress dust during loading.

52.	What are the hazards of Ammonium Nitrate-based Fertilizers?

Ans: Supports combustion (oxidizer), can undergo self-sustaining thermal decomposition if heated by hot bulkheads or cargo lights, evolving toxic nitrogen oxide fumes.

53.	What precautions are taken before loading Ammonium Nitrate Fertilizers?

Ans: Isolate all hold electrical cables at the main switchboard, ensure adjacent bunker tanks are not heated above 50°C, prohibit hot work, and strictly enforce no-smoking.

54.	What are the hazards of Bauxite and Nickel Ore?

Ans: Both can contain high fine particles and moisture (Group A liquefaction hazard); Nickel Ore is notorious for rapid tropical rain absorption and hidden liquefaction.

55.	What are the hazards of Seed Cake (Oil Cake) in bulk?

Ans: Group B chemical hazard: residual vegetable oil oxidizes slowly, leading to spontaneous heating, combustion, and oxygen depletion.

56.	What is the main hazard of Wood Pellets or Timber in closed holds?

Ans: Rapid oxygen depletion and carbon monoxide generation through organic oxidation; treat the hold and adjacent access trunks as lethal enclosed spaces.

57.	What are the requirements when carrying Timber Deck Cargo?

Ans: Comply with the TDC Code: stow compactly, restrict height when in Winter Seasonal Zones (≤ 1/3 extreme breadth above weather deck), rig uprights/hog lashings with slip hooks, and fit lifelines for crew access.

58.	Why does a ship with a Timber Deck Cargo get a deeper assigned Load Line?

Ans: Because properly stowed and lashed deck timber adds reserve buoyancy and reduces green seas boarding the weather deck.

59.	How can Timber Deck Cargo endanger stability during a winter voyage?

Ans: Timber absorbs water spray or accumulates ice on top while bottom fuel/ballast is consumed, causing KG to rise and potentially inducing an Angle of Loll.

60.	What are the hazards of carrying Scrap Metal (Swarf / Ferrous Turnings)?

Ans: Sharp edges damage tank tops and coatings, oil-soaked swarf undergoes rapid oxidation and self-heating, and unknown drums/ordnance may be hidden inside.

## 4. Grain Carriage & Stability Considerations (Q61–80)

61.	What is the International Grain Code and which ships must comply?

Ans: A mandatory code under SOLAS Chapter VI governing the safe carriage of wheat, maize, oats, rye, barley, rice, pulses, and seeds in bulk on all ships regardless of size.

62.	What are the primary hazards of carrying Grain in bulk?

Ans: Low angle of repose (∼20°–30°), settling during the voyage creating void spaces at the top of holds (5% settling), and transverse dry shifting causing a permanent list.

63.	What is the difference between a "Filled Compartment (Trimmed)" and a "Partly Filled Compartment"?

Ans: Filled compartment has grain trimmed up to the underdeck and inside the hatch coaming; Partly Filled compartment is not loaded to capacity and has a larger exposed free grain surface.

64.	Why is it called "Assumed Volumetric Heeling Moment" (AVHM) in the Grain Loading Booklet?

Ans: Because the Grain Code assumes a standardized worst-case void depth underneath longitudinal deck girders and hatches, along with fixed surface shift angles (15° in filled holds, 25° in partly filled holds).

65.	What are the three mandatory Intact Stability Criteria under the Grain Code?

Ans: 1) Angle of heel due to grain shift ≤12° (or deck edge immersion if smaller); 2) Residual dynamic area between heeling arm and GZ curve up to 40° (or flooding/max GZ angle) ≥0.075 m-rad; 3) Initial GM₀ (corrected for free surface) ≥0.30 m.

66.	How do you reduce the Volumetric Heeling Moment in a partly filled grain hold?

Ans: Level the grain surface and "overstow" (strap/secure) it with separation cloth/burlap and bagged grain or lashed general cargo, or fit longitudinal shifting boards.

67.	What is "Saucering" in a filled grain hold?

Ans: Creating a saucer-shaped depression in the bulk grain right inside the hatch square, lining it with separation cloth, and filling it tightly with bagged grain to plug the feeder space.

68.	What is "Bundling" of bulk grain?

Ans: Lining a saucer depression with heavy tarpaulin/canvas, filling it with loose bulk grain, and lashing the canvas edges tightly over the top to act as a plug.

69.	Can a modern bulk carrier load grain without fitting shifting boards or bagging?

Ans: Yes, provided her Document of Authorization and Grain Stability Booklet demonstrate that she meets the three Grain Code stability criteria in the "Untrimmed Ends" or "Partly Filled" conditions.

70.	What is the Document of Authorization for the Carriage of Grain?

Ans: A statutory certificate issued by Flag/Class confirming the ship is capable of complying with the International Grain Code; without it, loading faces strict limits.

71.	Under what emergency exemption can a ship load grain without a Document of Authorization?

Ans: Total grain weight ≤ 1/3 DWT, all filled holds fitted with centerline shifting boards, all partly filled holds leveled and overstowed with bags, and GM≥0.30 m (or satisfying the Administration).

72.	How do you prepare a cargo hold for loading bulk grain?

Ans: Hospital-clean sweep and fresh-water wash, remove all rust scale/paint flakes and previous cargo residues, dry thoroughly, test bilges and cover with burlap, and pass USDA/local phytosanitary hold inspection.

73.	What is Fumigation in Transit and what precautions are required?

Ans: Placing aluminum phosphide tablets/sleeves in grain holds to kill insects via toxic Phosphine (PH₃) gas; seal holds, post warning signs, check adjacent accommodation daily with phosphine detectors, and carry gas masks/antidotes.

74.	How do you calculate the Angle of Heel (θ) due to a grain shift from the Grain Booklet?

Ans: Calculate the total volumetric heeling moment (m⁴), divide by the stowage factor (m³/MT) to get the grain heeling moment (MT-m), divide by displacement to get λ₀, and compare on the statical stability curve (or tanθ ≈ heeling moment / (Δ × GM)).

75.	Why is λ₄₀ taken as 0.8×λ₀ when plotting the Grain Heeling Arm curve?

Ans: Because as the ship heels toward 40°, the shifted wedge of grain hits the upper deck/hatch side and cannot shift further horizontally, reducing the effective lever by cos40° ≈0.8.

76.	What is a "Wet Shift" in cargo vs. a "Dry Shift"?

Ans: A Dry Shift occurs when free-flowing dry granules slide when roll angle exceeds the angle of repose; a Wet Shift (liquefaction) occurs when moisture exceeds FMP and the cargo flows as a fluid.

77.	How do you prevent insect infestation from spreading between grain holds?

Ans: Clean holds meticulously between voyages, inspect grain at the loading spout for live weevils, and follow scheduled in-transit fumigation protocols.

78.	Why do you check the Stowage Factor of grain during loading via test weight (hectolitre weight)?

Ans: If actual SF is higher (lighter grain) than planned, the holds may cube out before reaching the target deadweight, leaving unexpected slack holds with huge heeling moments.

79.	How many slack (partly filled) holds do you aim to have when planning a grain stowage?

Ans: Ideally only one (or maximum two) slack holds, keeping all other holds completely filled and trimmed to minimize total Volumetric Heeling Moments.

80.	What checks are done before opening a fumigated grain hold at the discharge port?

Ans: Verify exposure period is complete, wear SCBA, open hatches to ventilate, test atmosphere with a Phosphine gas tube/detector until PH₃ is below TLV (0.3 ppm), and dispose of spent fumigant sleeves safely.

## 5. Container Operations, Bay Plans & Securing (Q81–90)

81.	Explain the 6-digit Bay-Row-Tier numbering system on a container ship.

Ans: First 2 digits: Bay (odd numbers for 20' containers, even for 40' from bow to stern); Middle 2 digits: Row (00 on centerline, odd to Stbd, even to Port); Last 2 digits: Tier (even numbers 02, 04... in holds; 80, 82, 84... on deck).

82.	What markings are displayed on the CSC Safety Approval Plate of a container?

Ans: Date of manufacture, ID number, Maximum Operating Gross Weight, Allowable Stacking Weight for 1.8g, and Transverse Racking Test Load value.

83.	On what grounds would you reject a container during loading?

Ans: Damaged corner castings, bowed structural corner posts, leaking contents, missing CSC plate, overweight without verified gross mass (VGM), or missing/defective IMDG placards.

84.	What is the Verified Gross Mass (VGM) requirement under SOLAS?

Ans: Every packed container must have a shipper-signed VGM document obtained by weighing the packed box or summing all contents + tare weight before it is loaded onto the ship.

85.	What is the Cargo Securing Manual (CSM) and which ships must carry it?

Ans: A Flag/Class-approved manual required on all ships carrying non-bulk cargoes (containers, general cargo, ro-ro; not required on pure bulkers or tankers), detailing fixed/portable lashing gear SWL and lashing patterns.

86.	List the standard portable securing devices used for containers on deck.

Ans: Base twistlocks, semi-automatic/manual mid-locks, bridge fittings (at the top tier between rows), lashing rods, turnbuckles, and extension hooks.

87.	What is the difference between Maximum Securing Load (MSL) and Safe Working Load (SWL) in the CSM?

Ans: They are functionally equivalent terms for lashing gear; MSL is generally 50% of the Breaking Strength for steel rods/chains and twistlocks.

88.	How are Refrigerated (Reefer) Containers stowed and monitored onboard?

Ans: Stowed near power sockets with clear airflow around the machinery end (usually facing aft away from green seas), plugged in promptly, and supply/return temperatures logged twice daily.

89.	What action do you take if an Out-of-Gauge (OOG) or Flat-Rack container is loaded?

Ans: Stow on top tiers or inside cell guides using adaptor frames, secure with custom wire/chain lashings calculated via the CSM for transverse/vertical accelerations, and protect from weather if required.

90.	What is Parametric Rolling on a container ship and how do you break it?

Ans: Violent resonant rolling reaching 30°–40° in head or following seas when wave encounter period matches half the ship's natural roll period; break it immediately by altering course or speed.

## 6. Packaged Dangerous Goods (IMDG Concepts) (Q91–100)

91.	List the 9 Classes of Dangerous Goods under the IMDG Code.

Ans: Class 1: Explosives; Class 2: Gases (2.1 Flammable, 2.2 Non-flammable, 2.3 Toxic); Class 3: Flammable Liquids; Class 4: Flammable Solids (4.1 Solids, 4.2 Spontaneous combustion, 4.3 Dangerous when wet); Class 5: Oxidizers (5.1) & Organic Peroxides (5.2); Class 6: Toxic (6.1) & Infectious (6.2); Class 7: Radioactive; Class 8: Corrosives; Class 9: Miscellaneous.

92.	How many volumes are in the IMDG Code and what is in each?

Ans: Volume 1: General provisions, classification, packing, consignment, construction, and stowage/segregation rules; Volume 2: The Dangerous Goods List (DGL sorted by UN Number) and Limited/Excepted Quantities; plus the Supplement.

93.	What is contained in the IMDG Code Supplement?

Ans: The Emergency Response Procedures for Ships Carrying Dangerous Goods (EmS Guide for Fire and Spillage), Medical First Aid Guide (MFAG), Reporting Procedures, Packing of CTUs, and Safe Use of Pesticides.

94.	What are the four levels of Segregation in the IMDG Code?

Ans: 1) "Away from" (≥3 m horizontally for open packages); 2) "Separated from" (≥6 m on deck or separate holds); 3) "Separated by a complete compartment or hold from" (≥12 m on deck); 4) "Separated longitudinally by an intervening complete compartment or hold from" (≥24 m).

95.	How does the "Away From" (1) segregation requirement apply to closed Freight Containers?

Ans: Two incompatible substances having an "Away from" requirement cannot be packed inside the same container, but once in separate closed containers, they need no horizontal separation on board (can be stowed side-by-side).

96.	What are UN Packaging Groups I, II, and III?

Ans: Indicators of the degree of danger within a class: Packing Group I = Great danger; Packing Group II = Medium danger; Packing Group III = Minor danger (Note: Classes 1, 2, 5.2, 6.2, and 7 do not use Packing Groups).

97.	What documents are required on board to carry Packaged Dangerous Goods?

Ans: Document of Compliance for Carrying Dangerous Goods (issued by Flag/Class showing permitted holds/deck areas), Dangerous Goods Manifest/Stowage Plan, Shipper's DG Declaration, Container Packing Certificate, and MSDS.

98.	What action do you take in the event of an IMDG chemical spillage on deck?

Ans: Sound alarm, stay upwind, identify UN number from the DG Manifest, consult the EmS Spillage Schedule in the IMDG Supplement, don full chemical suit and SCBA, isolate ignition sources, wash/contain as directed by EmS, and consult MFAG if anyone is exposed.

99.	What is the INF Code?

Ans: The International Code for the Safe Carriage of Packaged Irradiated Nuclear Fuel, Plutonium and High-Level Radioactive Wastes on Board Ships (classifying ships as INF 1, INF 2, or INF 3).

100.	What is a Limited Quantity (LQ) vs. a Marine Pollutant mark?

Ans: LQ is dangerous goods packed in small inner receptacles below threshold limits (marked with a diamond having black top/bottom corners); Marine Pollutant is marked with the dead tree & fish symbol and stowed preferentially under deck or inboard.

## 7. Lifting Gear, Derricks, Cranes & Rope/Wire Care (Q101–110)

101.	What is a Union Purchase rig and how is its SWL determined?

Ans: Two derricks coupled together (one plumbed over the hatch, one over the quay) with their cargo runners joined at a single union hook; the SWL is typically 1/3 to 1/2 of the smaller derrick's single-swing SWL, with the angle between runners kept below 120°.

102.	What is the purpose of Preventer Guys on a Union Purchase rig?

Ans: Heavy wire ropes secured alongside the slewing guys to absorb the high lateral shock loads generated as the load is transferred across between the two derricks.

103.	What is the Register of Ship's Lifting Appliances and Cargo Handling Gear (Chain Register)?

Ans: An official statutory record book containing test certificates, annual thorough examination reports, and 5-yearly load test records for all cranes, derricks, winches, blocks, shackles, and wires.

104.	What are the proof load testing intervals and percentages for shipboard cranes?

Ans: Tested before initial commissioning and every 5 years thereafter (plus after major repairs): SWL up to 20t = SWL+25%; SWL 20t to 50t = SWL+5 tons; SWL over 50t = SWL+10%.

105.	What does a wire rope designation like " 6×19" or " 6×36" mean?

Ans: The first number (6) is the number of outer strands laid around the core; the second number (19 or 36) is the number of individual steel wires inside each strand (more wires = greater flexibility).

106.	Compare Ordinary (Regular) Lay and Lang's Lay in wire ropes.

Ans: In Ordinary Lay, wires twist opposite to the strands (resists kinking/unlaying); in Lang's Lay, wires and strands twist in the same direction (greater wearing surface over sheaves, used on crane runners with fixed ends).

107.	When must a steel wire rope on a crane or lifeboat fall be condemned and replaced?

Ans: If visible broken wires exceed 10% within a length of 8 diameters, or if there is kinking, bird-caging, core protrusion, severe corrosion, or a 7–10% reduction in nominal diameter.

108.	What safety limit switches are fitted on a modern shipboard deck crane?

Ans: Hoisting upper/lower limit, luffing (jib angle) upper/lower limit, overload cutout, slack-wire switch, and emergency stop button.

109.	How do you inspect and maintain Cargo Blocks and Shackles?

Ans: Check for crown wear, pin distortion, elongated eyes, clear SWL and certificate stamps, free rotation of sheaves, and grease nipples regularly.

110.	What is the Factor of Safety connecting SWL and Breaking Stress?

Ans: SWL = Breaking Strength / Factor of Safety (typically Factor of Safety is 5 for cargo wires/chains and 6 for natural/synthetic fibre ropes).

## 8. Oil Tanker Operations, Inert Gas & COW Concepts (Q111–130)

111.	What is the difference between a Clean Petroleum Product (CPP) and a Dirty Petroleum Product (DPP)?

Ans: CPP includes refined light distillates like gasoline, naphtha, jet fuel, and gasoil; DPP includes crude oil, heavy fuel oil, and residual feedstocks.

112.	Define Flashpoint and explain the 60°C threshold in SOLAS.

Ans: The lowest temperature at which a liquid gives off sufficient vapor to form an ignitable mixture with air near its surface; cargoes with Flashpoint <60°C are classified as volatile/low-flashpoint requiring Inert Gas protection.

113.	Explain the Flammability Diagram (Hydrocarbon% vs. Oxygen%).

Ans: Plots the flammable envelope bounded by Lower Flammable Limit (LFL ~1% HC by vol), Upper Flammable Limit (UFL ~10% HC by vol), and minimum Oxygen needed for combustion (~11% O₂); keeping tanks inerted below 8% O₂ ensures you never enter the flammable zone during dilution with air.

114.	What is the maximum permissible Oxygen content in the Inert Gas main vs. inside a cargo tank?

Ans: In the IG main supply line: ≤5% Oxygen by volume; inside an inerted cargo tank: ≤8% Oxygen by volume.

115.	What action must you take immediately if the Inert Gas System fails during discharge or COW?

Ans: Stop all cargo discharge, stripping, and COW operations immediately, close the deck isolating valve, notify the terminal and Master, and do not resume until IG is restored (to prevent air/vacuum from being drawn into tanks).

116.	Why is Pyrophoric Iron Sulphide a major hazard on crude oil tankers?

Ans: Hydrogen Sulphide (H₂S) in sour crude reacts with rusty tank steel in the absence of oxygen to form Iron Sulphide (FeS); if air enters the tank, FeS oxidizes exothermically and glows incandescently, igniting hydrocarbon vapors.

117.	Trace the components of a Flue Gas Inert Gas System from boiler to cargo tanks.

Ans: Boiler Uptake Valve → Scrubber Tower (cools gas & removes SO₂ /soot) → Demister → IG Blowers → IG Pressure Regulating Valve → Deck Water Seal → Mechanical Non-Return Valve → Deck Isolating Valve → Cargo Tanks (with P/V Breaker on the IG main).

118.	What is the purpose of the Deck Water Seal and what are its three types?

Ans: Acts as the primary hydraulic barrier preventing hydrocarbon gases from flowing back from cargo tanks to the hot engine room; types are Wet, Semi-Dry, and Dry.

119.	What is the difference between a High-Velocity P/V Valve and a Liquid-Filled P/V Breaker?

Ans: Individual tank P/V valves relieve routine thermal/loading pressure (~1400 mmWG) and vacuum (~-350 mmWG) while jetting vapor upwards at ≥30 m/s; the P/V Breaker is a secondary emergency safety device on the common IG main (~1800–2100 mmWG).

120.	What are Static Accumulator Cargoes and what loading precautions apply?

Ans: Refined clean oils (kerosene, jet fuel, clean diesel) with low conductivity (<50 pS/m) that hold static charges; restrict initial linear flow velocity to ≤1 m/s until the tank drop line inlet is fully submerged, wait 30 minutes after completion before introducing metallic dipping tapes, and never use steam inside tanks.

121.	What does a UTI (Ullage-Temperature-Interface) tape measure?

Ans: A portable intrinsically safe electronic tape passed through a vapour lock valve to measure exact ullage height, cargo temperature, and the oil/water interface at the bottom of the tank.

122.	What are the SOLAS requirements for High-Level and Overfill (95% & 98%) Alarms on tankers?

Ans: Must be completely independent of the primary radar tank gauging system, give visual and audible alarms on deck and in the CCR at 95% (High Level) and 98% (Overfill), and be tested prior to every cargo operation.

123.	What is a Vapour Emission Control System (VECS)?

Ans: A closed vapour return pipeline collecting displaced tank gases during loading and returning them to the shore terminal for recovery/incineration, fitted with high/low manifold pressure alarms and detonation arrestors.

124.	What are the pre-arrival and pre-operation checks before starting Crude Oil Washing (COW)?

Ans: Verify IG working and tank O₂ <8%, pressure test COW lines to 1.0 MPa (10 bar) and log it, drain water from the crude supply tank (discharge bottoms by 1m first), check heater is blanked off, and complete the COW checklist.

125.	Which crude oils are unsuitable for COW?

Ans: High-viscosity crudes, high pour-point / high-wax crudes that solidify on bulkheads unless heated, and high vapour pressure crudes that gas up excessively.

126.	What is the proper sequence for Stopping Cargo Discharge at full rate with the shore terminal?

Ans: Give advance notice to shore, gradually reduce cargo pump RPM to lower flow and line pressure, let shore close their valve ONLY after flow is stopped or directed by shore to avoid catastrophic water-hammer line bursts, then shut ship's manifold valve.

127.	What are the key updates in ISGOTT 6th Edition and the Ship/Shore Safety Check-List (SSSCL)?

Ans: Reorganized into human factors and shared ship/shore sections, explicit enclosed space and gas testing tables, expanded VECS/volatile organic compound controls, and mandatory repetitive interval checks.

128.	How do you prepare cargo tanks on a tanker when switching from Dirty Oil to Clean Diesel?

Ans: Complete COW (if crude), strip dry, butterworth wash with hot water (start with cold water first if high-volatile residues), strip washings to slop tank, purge, gas-free for inspection, mop/dry bottoms and pipelines, and re-inert if required.

129.	What are the four stages of Gas Freeing a tanker cargo tank?

Ans: 1) Stripping & Washing; 2) Purging with Inert Gas until Hydrocarbon gas drops below 2% by volume (below the Critical Dilution Line); 3) Ventilating with fresh air until O₂ =20.9%, HC<1% LEL, and toxic gases (H₂S, Benzene) are below TLV.

130.	What are the pipeline systems found on oil tankers?

Ans: Ring Main system, Direct Line system, Free-Flow system, and Individual Deepwell Pump (Framo) system (where each tank has its own hydraulic submerged pump and dedicated deck line).

## 9. Chemical & Gas Carrier Fundamentals (Q131–150)

131.	What code governs Chemical Tankers and what are the three Ship Types?

Ans: The IBC Code (International Bulk Chemical Code); Type 1: Maximum preventive measures (tanks ≥B/5 from side, for extreme hazards); Type 2: Significant preventive measures (tanks ≥760 mm from side); Type 3: Moderate survival capability (double hull).

132.	What are the four Pollution Categories of Noxious Liquid Substances (NLS) under MARPOL Annex II?

Ans: Category X: Major hazard to marine resources (prohibited discharge; requires prewash to shore); Category Y: Hazard (limited discharge); Category Z: Minor hazard (less stringent discharge); OS: Other Substances.

133.	Why is edible Soya Bean Oil classified under the IBC Code / MARPOL Annex II rather than Annex I?

Ans: Because Annex I covers only petroleum mineral oils; vegetable oils like soya bean oil are Category Y Noxious Liquid Substances that polymerize and smother marine life/seabirds if spilled.

134.	What is the Certificate of Fitness (CoF)?

Ans: A mandatory statutory certificate issued under the IBC Code (for chemical tankers) or IGC Code (for gas carriers) listing every specific cargo the vessel is approved to carry and its tank conditions.

135.	Why is Nitrogen (N₂) used instead of Boiler Flue Gas to inert tanks on Chemical Tankers?

Ans: Boiler flue gas contains CO₂, SO₂, moisture, and soot particles that contaminate high-purity chemicals or react chemically with cargoes; membrane/PSA Nitrogen Generators produce 99.5%–99.9% pure N₂.

136.	What is the USCG / IBC Chemical Compatibility Chart used for?

Ans: To verify whether two chemicals in adjacent tanks or sharing vapour/piping headers could react violently (generating heat or toxic gas) if a bulkhead leaks.

137.	What are Inhibited Cargoes (e.g., Styrene Monomer) and what certificate is required?

Ans: Monomers that can self-polymerize exothermically; require a Certificate of Inhibitor from the shipper stating inhibitor name, concentration, valid duration, temperature limits, and whether the inhibitor is oxygen-dependent (never inert oxygen-dependent monomers with pure N₂ !).

138.	What is "Padding" vs. "Inerting" on a chemical tanker?

Ans: Inerting maintains a non-flammable atmosphere (<8% O₂); Padding fills the ullage space with Nitrogen or dry air solely to keep moisture out of water-reactive cargoes (like TDI/MDI or sulphuric acid).

139.	How do you strip a Framo submerged hydraulic pump and deck line on a product/chemical tanker?

Ans: Shut the discharge manifold valve, open the purging/stripping valve, apply pressurized nitrogen or air above the pump impeller stack to push trapped liquid out through the small-bore stripping line into the shore manifold.

140.	What code governs Liquefied Gas Carriers and what are the four ship types?

Ans: The IGC Code: Type 1G (most hazardous gases like Chlorine), Type 2G, Type 2PG, and Type 3G.

141.	What is the fundamental difference between LPG and LNG in sea transport?

Ans: LPG (Propane/Butane, boiling point ≈−42°C to −0.5°C) can be liquefied by pressure, refrigeration, or both; LNG (Methane, boiling point −162°C, critical temp −82.6°C) cannot be liquefied by pressure at ambient temperature and must be carried fully refrigerated at atmospheric pressure.

142.	Why do Gas Carriers maintain positive tank pressure and avoid any vacuum?

Ans: Because drawing air into a liquefied gas tank immediately creates an explosive mixture and introduces moisture that freezes inside cryogenic valves/reliquefaction plants.

143.	What are the main Cargo Containment Tank types on Gas Carriers?

Ans: Independent Tanks (Type A prismatic, Type B spherical Moss-Rosenberg, Type C pressurized cylindrical) and Membrane Tanks (Invar or stainless steel supported by hull insulation).

144.	What is Boil-Off Gas (BOG) and how is it handled on LPG vs. LNG ships?

Ans: Vapour generated by ambient heat ingress; on LPG ships it is compressed, condensed in a reliquefaction plant, and returned to tanks; on LNG ships it is either burned as dual-fuel propulsion gas or reliquefied.

145.	What is an Emergency Shutdown (ESD) system on a Gas or Chemical Tanker?

Ans: A linked ship-shore safety system (pneumatic, fiber-optic, or electric) that automatically stops cargo pumps and closes ESD manifold valves within seconds upon fire, fusible plug melting (98°C–104°C), tank overfill (98%), or manual push-button activation.

146.	What is the purpose of the Water Spray (Deluge) System on a Gas Carrier deck?

Ans: Protects tank domes, manifolds, and accommodation bulkhead against radiant heat during a fire and prevents brittle fracture of deck steel in the event of a cryogenic liquid spill.

147.	Why is Dry Chemical Powder (DCP) used as the fixed deck fire-fighting system on Gas Carriers?

Ans: Water and foam would warm a cryogenic liquid spill, accelerating vaporization and feeding the gas fire; potassium/sodium bicarbonate DCP chemically interrupts the flame chain reaction.

148.	What is a Decontamination Shower and Eye-Wash Station requirement on Chemical Tankers?

Ans: Prominently marked, heated/insulated against freezing, operable in all weather, located near the cargo manifold and on deck to flush chemical splashes immediately with fresh water.

149.	What is contained in a Material Safety Data Sheet (MSDS)?

Ans: 16 standardized sections covering chemical identity, hazard classification, composition, first-aid measures, fire-fighting, spill response, handling/storage, PPE/exposure limits, physical/chemical properties, reactivity, and toxicology.

150.	What are Emergency Escape Breathing Devices (EEBDs) in the Cargo Pump Room / Compressor Room?

Ans: Self-contained hood units providing at least 10–15 minutes of breathable air purely for escaping a sudden toxic or oxygen-deficient atmosphere (never used for fire-fighting or entry).

## 10. Hatch Covers, Ballast Management & Cargo Space Inspections (Q151–200)

151.	What are the main types of steel Hatch Covers on dry cargo ships?

Ans: Folding (hydraulic or wire-operated), Side-Rolling (common on Capesize/Panamax bulkers), Piggy-Back (stacking), and Lift-Away Pontoon covers (common on container ships).

152.	What is the difference between "Weathertight" and "Watertight"?

Ans: Weathertight means water will not penetrate into the ship in any sea conditions from above (e.g., hatch covers, ventilator closures); Watertight means water will not pass through in either direction under a static head of water (e.g., collision bulkhead, double bottom).

153.	Compare the Hose Test, Chalk Test, and Ultrasonic Test for Hatch Covers.

Ans: Chalk Test checks only compression contact between steel compression bar and rubber packing; Hose Test uses a water jet (2 bar from 1.5 m distance) to verify no leakage; Ultrasonic Test places a transmitter inside the closed hold and reads decibel leakage outside with a microphone (most accurate and cleanest).

154.	Why must you never place thicker rubber packing or tighten cleats beyond the steel-to-steel Bearing Pads?

Ans: Landing pads transfer the heavy weight of the hatch cover directly to the coaming structure; over-compressing the rubber ruins its elasticity permanently ("permanent set"), causing leaks when the hull flexes at sea.

155.	What is the purpose of the Cross-Joint Drain Channel and Non-Return Drain Valve on hatch coamings?

Ans: Any minor water that seeps past the outer rubber lip during hull flexing is caught in the inner drainage channel and drained onto the deck via a float/ball non-return valve.

156.	What safety precautions must be followed before operating hydraulic or wire-pulled Hatch Covers?

Ans: Check trackways are clear of cargo debris, remove all quick-acting cleats and locking pins, post a lookout on the off-side, verify hydraulic lines for leaks, and insert eccentric locking pins / safety hooks immediately once fully open.

157.	What are the two methods of Ballast Water Exchange at sea (D-1 Standard)?

Ans: 1) Sequential Method (pump tank completely dry until suction is lost, then refill with clean ocean water—achieve 95% volumetric exchange); 2) Flow-Through / Dilution Method (pump 3 times the tank volume through the tank—300% exchange).

158.	What are the geographical criteria for conducting Ballast Water Exchange at sea?

Ans: Must be conducted at least 200 nautical miles from the nearest land and in water at least 200 meters deep (or at least 50 nm and 200 m deep if 200 nm is impossible).

159.	What is the difference between the BWM Convention D-1 and D-2 Standards?

Ans: D-1 is the volumetric mid-ocean ballast water exchange standard; D-2 is the biological performance standard requiring an approved onboard Ballast Water Treatment System (BWTS using UV, electro-chlorination, or filtration) to limit viable organisms per cubic meter.

160.	What are the stability and structural hazards during Sequential Ballast Water Exchange?

Ans: Temporary loss of GM due to large Free Surface Moments while tanks are slack, increased Shear Forces and Bending Moments, reduced forward draft causing propeller emergence or bow slamming, and risk of over-pressurizing or collapsing a tank if air vents are blocked.

161.	Why is it dangerous to overflow a Ballast Tank through the deck air pipe when usingFlow-Through exchange?

Ans: If the pumping rate exceeds the air pipe capacity (especially if the wire gauze flame/debris screen is rusted or clogged), the hydraulic head can bulge or rupture the tank top plating.

162.	How do you test Cargo Hold Bilge Suction and Non-Return Valves before loading?

Ans: Pour a measured bucket of water into the bilge well, start the bilge eductor/pump from the engine room, verify prompt suction, stop the pump, and confirm water does not flow back into the bilge well.

163.	Why are Cargo Hold Bilge Lines fitted with Non-Return Valves?

Ans: To prevent seawater from accidentally flooding into a dry cargo hold if an engine room valve is mis-lined during bilge or ballast operations.

164.	What do you inspect during a Cargo Hold Structural Inspection after grab discharge?

Ans: Check hopper tank slopes, lower stool plating, side shell frames and brackets (for cracks, tripping, or detachment), sounding pipes, air pipes, ladders, and tank top plating for grab indentations.

165.	Which part of a Bulk Carrier's hold structure is most vulnerable to corrosion and fatigue cracking?

Ans: The lower weld toes of side shell frames ("web frames") where they connect to the hopper tank sloping plate, due to flexing under wave pressure and moisture trapped in cargo residues.

166.	What is the Enhanced Survey Programme (ESP)?

Ans: A mandatory, rigorous structural inspection regime under SOLAS Chapter XI-1 applicable to Bulk Carriers and Oil Tankers, requiring close-up surveys and ultrasonic thickness measurements recorded in a dedicated ESP File on board.

167.	How do you calculate the maximum cargo a ship can lift when passing from a Summer Zone into a Winter Zone?

Ans: Check whether the vessel is restricted by Departure Summer Displacement or Arrival Winter Displacement (adding fuel and fresh water consumed during the leg from the departure port to the Winter Zone boundary).

168.	What is Hydrometer Draft Correction (Dock Water Allowance - DWA)?

Ans: The amount by which a ship may submerge her salt-water load line when loading in brackish dock water (ρ between 1.000 and 1.025 t/m³): DWA (mm) = FWA × (1025 − dock density) / 25.

169.	Why do you monitor Hogging and Sagging deflection during loading?

Ans: Comparing Mean of Fore/Aft Drafts against Midship Draft reveals whether the hull is Hogging (midship draft less than mean—ends heavy) or Sagging (midship draft greater than mean—amidships heavy), which must also be accounted for in Draft Surveys and UKC.

170.	How do you convert Quarter Mean Draft (Mean of Means) into True Displacement in a Draft Survey?

Ans: Enter the Hydrostatic Tables with Quarter Mean Draft to get raw displacement, then apply First Trim Correction (for Center of Flotation offset from midships) and Second Trim Correction (for change in waterplane shape at large trim).

171.	What is a "Ship's Constant" in cargo operations?

Ans: The difference between the calculated lightship weight plus known deductibles and the actual displacement measured by draft survey (comprising mud in ballast tanks, unpumpable bilges, paint, crew effects, and spare parts).

172.	What should you do if the calculated Ship's Constant suddenly increases by 200 MT compared to previous voyages?

Ans: Suspect an error in draft reading, dock water density, or tank sounding (e.g., an unrecorded ballast tank, blocked sounding pipe, or incorrect trim correction) and re-check all figures before signing the B/L.

173.	What is "Ullage" vs. "Sounding" and when is each used?

Ans: Sounding measures liquid depth from the bottom striking plate upwards (used for ballast, bilges, and nearly empty tanks); Ullage measures empty space from the top reference point down to the liquid surface (used for loaded oil/chemical cargo tanks).

174.	What is Wedge Formula used for on an Oil Tanker?

Ans: Calculating the small residual volume of liquid or ROB (Remaining On Board) collected in the aft wedge of a cargo tank when the ship is trimmed by the stern and the liquid does not cover the entire tank bottom.

175.	How do you calculate API Gravity vs. Specific Gravity on a tanker?

Ans: API Gravity = 141.5 / (SG at 60°F) − 131.5; higher API means lighter, less dense oil; used with ASTM Tables (Table 54B and Table 6B) to convert observed volume at tank temperature to Standard Volume at 15°C.

176.	What is a Line Displacement check during tanker loading or grade changes?

Ans: Pumping a known volume from one tank or shore line and verifying the exact receipt in the receiving tank after displacing the pipeline volume to confirm valve alignment and line cleanliness.

177.	What are the safety precautions before disconnecting Cargo Hoses or Chiksan Arms at the manifold?

Ans: Confirm pumps are stopped, drain and purge the manifold arm with nitrogen/air into the ship's slop tank or shore drain, verify zero pressure on manifold gauges, blank/blind the flange cleanly over the drip tray, and wear PPE.

178.	Why must Scupper Plugs be fitted during oil/chemical cargo and bunker operations?

Ans: To contain any accidental deck spill on board within the fishplate coaming so it can be pumped to the slop tank via pneumatic Wilden pumps rather than escaping overboard.

179.	When should Scupper Plugs be momentarily eased on a tanker in port?

Ans: During heavy rain squalls to drain clean rainwater off the deck, under constant direct visual watch by a deck rating who immediately re-plugs them if any oil sheen appears.

180.	What is the purpose of an Insulating Flange or a single length of Non-Conducting Hose at the tanker manifold?

Ans: Prevents stray galvanic/cathodic protection electrical currents between the ship's hull and the shore pipeline from arcing when the metal manifold flange is connected or disconnected (preferred over a ship-shore bonding cable).

181.	How do you secure a Gantry or Jib Deck Crane for sea passage?

Ans: Lower the jib into its crutch/rest, secure the mechanical locking pins/turnbuckles, house the cargo hook tightly to a deck padeye, switch off power in the crane cab, and lash all loose wires.

182.	What is the difference between a Snatch Block and a Heel Block?

Ans: A Snatch Block has a hinged side plate allowing a bight of rope/wire to be inserted without reeving the end; a Heel Block is mounted at the gooseneck base of a derrick to lead the cargo runner to the winch.

183.	What is "Fleeting" a load during cargo operations?

Ans: Transferring the weight of a suspended load gradually from one lifting appliance or tackle to another so the point of suspension shifts horizontally within a hold.

184.	How do you care for Natural Fibre Ropes (Manila/Sisal) vs. Synthetic Ropes (Polypropylene/Nylon)?

Ans: Natural fibre must be dried before stowing on wooden gratings to prevent rot and kept away from chemicals; Synthetic ropes must be shaded from UV sunlight, kept off hot steam pipes, and never surged rapidly on steel winch drums (risk of friction melting).

185.	Which synthetic rope floats on water and why is that useful?

Ans: Polypropylene has a specific gravity of ~0.91 (less than water), making it ideal for lifebuoy buoyant lines and boat painters, though it has a lower melting point (165°C).

186.	What is "Snap-Back" and how is it prevented during mooring or cargo work?

Ans: The lethal high-velocity recoil of a synthetic rope or wire when it parts under tension; stand well clear of the line of pull and never stand inside a bight.

187.	What are Cargo Battens (Spar Ceiling) in a general cargo hold?

Ans: Removable horizontal timber planks fitted into cleats along the inside of the ship's side frames to keep general cargo packages from touching cold, sweating shell plating.

188.	Why are Cargo Battens removed before loading Bulk Grain or Bulk Coal?

Ans: Loose timber battens get smashed by bulldozers and grabs, contaminate the bulk cargo, and trap pockets of grain or coal dust behind the frames.

189.	What is a "Rose Box" (Strum Box) in a cargo hold bilge?

Ans: A perforated plate or basket fitted over the open end of the bilge suction pipe inside the bilge well to prevent cargo debris from entering and choking the bilge pipeline.

190.	What is the function of Sounding Pipe Striking Plates?

Ans: A thickened steel doubling plate welded directly onto the inner bottom plating beneath the bottom of a sounding pipe to prevent the repeated impact of the brass sounding bob from holing the double-bottom tank top.

191.	What happens if a Self-Closing Cock or Counterweighted Cap on a double-bottom sounding pipe inside the engine room is tied open?

Ans: If the ship runs aground and holes that double-bottom tank, seawater will flood unimpeded up the sounding pipe into the engine room (or fuel will spill out causing an engine room fire).

192.	How do you check the watertight integrity of a Manhole Cover after closing a double-bottom ballast tank?

Ans: Inspect the rubber gasket (replace if flattened or cracked), clean the seating studs, tighten nuts evenly in a diagonal star pattern, and visually inspect the manhole perimeter when the tank is first pressed up with ballast.

193.	What is "Pressing Up" a ballast tank and why is it done slowly near the top?

Ans: Filling a ballast tank until water just appears at the deck air pipe to eliminate Free Surface Effect; pump rate must be slowed near 90% capacity so air can escape freely without creating a destructive hydraulic pressure surge.

194.	What is the Loadicator and how is it verified onboard?

Ans: A Class-approved loading computer calculating intact/damage stability, draft, trim, and hull Shear Force/Bending Moments at standard readout frames; verified periodically against the approved Test Conditions in the Stability Booklet.

195.	What is the difference between Sea Condition and Harbour Condition limits on the Loadicator?

Ans: Harbour Condition permits higher Shear Force and Bending Moment limits (100% Harbour limit) because there are no wave-induced dynamic stresses; before sailing, stresses must be brought within 100% Seagoing limits.

196.	What action do you take if Cargo Hold Ventilation Fans must be run during a voyage?

Ans: Check weather and dew point rules first, open the mechanical fire dampers/flaps before switching on the electric fan starters, and record fan hours and hold/outside temperatures in the Cargo Ventilation Log.

197.	How are Ro-Ro (Roll-on/Roll-off) vehicle decks ventilated and secured?

Ans: Continuous mechanical exhaust ventilation (10 to 20 air changes per hour) removes explosive petrol/exhaust fumes from the low deck level, while vehicles are lashed down on multiple points to deck elephant-feet/D-rings per the CSM.

198.	What is the CSS Code?

Ans: The Code of Safe Practice for Cargo Stowage and Securing, providing international guidelines on securing non-standardized heavy items, steel coils, project cargo, and containers against ship accelerations.

199.	How are Heavy Steel Coils stowed and secured in a bulk carrier hold?

Ans: Stowed on double rows of flat timber dunnage with their cylindrical axes fore-and-aft, wedged tightly with timber chocks, locked using "key coils" in the second tier, and wire-lashed to prevent rolling.

200.	What is a Statement of Facts (SOF) at the end of port cargo operations?

Ans: A chronological log signed by the Master, Port Agent, and Terminal recording exact times of NOR tendered, berthing, commencement/suspension/completion of cargo, weather delays, and draft survey completion (used to calculate Laytime and Demurrage/Despatch).

# FUNCTION 3: Controlling the Operation of the Ship and Care for Persons on Board

## 1. Transverse Stability, GM & Loll Management (Q1–20)

1.	Define Center of Gravity (G), Center of Buoyancy (B), and Transverse Metacenter (M).

Ans: G is the point through which the ship's total weight acts vertically downward; B is the geometric centroid of the underwater displaced volume acting vertically upward; M is the intersection of vertical lines through B at upright and very small angles of heel (<10°–15°).

2.	Define Metacentric Height (GM) and Righting Lever (GZ).

Ans: GM is the vertical distance between G and M (measuring initial stability); GZ is the horizontal perpendicular distance between the downward force through G and the upward buoyancy force through B when heeled (so Righting Moment=Δ×GZ).

3.	Compare Stable, Neutral, and Unstable Equilibrium.

Ans: Stable: G is below M (GM>0, ship returns upright when heeled); Neutral: G coincides with M (GM=0, rests at small inclination); Unstable: G is above M (GM<0, heels further until reaching Angle of Loll or capsizing).

4.	Compare a Stiff Ship and a Tender Ship.

Ans: A Stiff ship has a very large GM (short, violent rolling period—typical of a loaded iron ore carrier); a Tender ship has a very small positive GM (long, slow, easy roll—typical of a timber carrier).

5.	What is the difference between List and Loll?

Ans: List occurs when G moves off the centerline (GG₁ transverse) while GM remains positive; Angle of Loll occurs when G remains on the centerline but rises above M (GM is negative), causing the ship to flop to either side until B moves out far enough vertically below G.

6.	What is the value of GZ at the Angle of Loll?

Ans: Zero (GZ=0); the vessel is in temporary stable equilibrium at that heeled angle, though with severely reduced residual range of stability.

7.	How do you distinguish between List and Loll at sea if unsure?

Ans: Check tank soundings and cargo distribution first; a listing ship rolls equally around a fixed angle on one side, whereas a lolling ship hesitates upright and flops lazily from port Loll to starboard Loll in waves.

8.	How do you safely correct an Angle of Loll?

Ans: Eliminate upper free surfaces, secure weights, and add ballast to a divided double-bottom tank starting on the LOW side first (so the initial increase in list is controlled rather than causing the ship to slam violently across to the other side), then fill the high-side tank.

9.	Why must you NEVER start correcting an Angle of Loll by filling a double-bottom tank on the HIGH side?

Ans: Because adding free surface before the added weight lowers G below M will temporarily worsen the negative GM, and once the high-side weight takes effect, the ship will roll violently past upright to an even greater angle on the opposite side.

10.	What are the mandatory IMO Intact Stability Criteria for general cargo ships?

Ans: 1) Area under GZ curve up to 30° ≥0.055 m-rad; 2) Area up to 40° (or flooding angle θf) ≥0.090 m-rad; 3) Area between 30° and 40° (or θf) ≥0.030 m-rad; 4) GZ≥0.20 m at an angle ≥30°; 5) Maximum GZ occurs at an angle preferably >30°, but never <25°; 6) Initial GM₀ ≥0.15 m.

11.	What is Free Surface Effect (FSE) and how do you minimize it at the design or operational stage?

Ans: When a partially filled tank heels, the liquid wedge shifts toward the low side, shifting the tank's center of gravity and causing a virtual rise in the ship's KG (FSC = i × ρ tank / Δ); dividing a tank longitudinally into n equal compartments reduces FSE to 1/n² of its original value.

12.	Does the quantity of liquid inside a slack rectangular tank change its Free Surface Moment?

Ans: No (as long as the liquid neither hits the tank top nor uncovers the tank bottom during heel); FSM depends solely on the second moment of area of the liquid surface (L×B³/12) and liquid density.

13.	What happens to a ship's Center of Gravity (G) when a crane lifts a 50 MT weight off the jetty or hold bottom?

Ans: The exact instant the weight clears the deck or jetty, its center of gravity jumps instantaneously to the head of the crane jib (point of suspension), causing an immediate virtual rise in KG and loss of GM.

14.	What is the Wall-Sided Formula and when is it used?

Ans: GZ = sinθ (GM + ½ BM tan²θ); used to calculate GZ at moderate angles of heel (beyond 10°–15° up to deck edge immersion) and to derive the Angle of Loll formula (tanθ(loll) = √(−2GM / BM)).

15.	What information is extracted from a Curve of Statical Stability (GZ Curve)?

Ans: Initial GM₀ (from the tangent at 1 radian = 57.3°), Maximum GZ and the angle at which it occurs, Range of Positive Stability, Angle of Vanishing Stability, and Dynamical Stability (area under the curve).

16.	What is Dynamical Stability?

Ans: The mechanical work done in heeling a ship to a given angle; represented by the displacement multiplied by the area under the Statical Stability (GZ) curve up to that angle (Δ×∫GZ dθ).

17.	What are KN Cross Curves of Stability?

Ans: Curves plotted in the Stability Booklet giving values of KN against Displacement for fixed angles of heel (15°,30°,45°,60°,75°,90°) assuming KG=0; actual GZ=KN−KGsinθ.

18.	Why and how is an Inclining Experiment performed?

Ans: Conducted on a newly built ship (or after major structural modification) by shifting known weights transversely in calm water and measuring pendulum deflection to accurately determine Lightship Displacement and Lightship KG/LCG.

19.	What conditions are required for a valid Inclining Experiment?

Ans: Calm water, minimal wind, moored loosely fore and aft, gangway landed, all tanks either completely full or empty (no free surface), boilers at working level, and non-essential personnel ashore.

20.	What is Angle of Downflooding (θf)?

Ans: The angle of heel at which the lowest non-watertight opening in the hull, superstructure, or deckhouse (like an engine room ventilator) immerses beneath the waterline.

## 2. Longitudinal Stability, Trim, Squat & Hull Stresses (Q21–40)

21.	Define Center of Flotation (F / LCF).

Ans: The geometric centroid of the ship's waterplane area; it is the pivot axis about which the vessel trims longitudinally and heels transversely.

22.	Why is Change of Draft Forward different from Change of Draft Aft when a weight is loaded?

Ans: Because the Center of Flotation (LCF) is rarely located exactly amidships; trim divides proportionally to the distance of F from the forward and aft perpendiculars.

23.	How do you load a small parcel of cargo so that the Draft Aft remains completely unchanged?

Ans: Load the cargo at a distance x forward of the Centre of Flotation (F) such that the bodily sinkage at the aft perpendicular is exactly cancelled by the trim effect: x = (MCTC × L) / (TPC × la), where L = length between perpendiculars and la = distance of F from the aft perpendicular. Loading further forward than x lifts the stern; loading less far forward increases the aft draught.

24.	Define Moment to Change Trim by 1 Centimeter (MCTC).

Ans: The longitudinal trimming moment (MT-m) required to change the ship's trim between the forward and aft perpendiculars by 1 cm; MCTC = Δ × GML / (100 × L).

25.	Define Tons Per Centimeter Immersion (TPC).

Ans: The mass in tonnes that must be loaded or discharged to change the vessel's mean draught by 1 cm; TPC = Awp × ρ / 100 (Awp = waterplane area in m², ρ in t/m³).

26.	What happens to a vessel's Mean Draft when passing from River Water (1.000) to Sea Water (1.025) without changing displacement?

Ans: The ship rises bodily by the Fresh Water Allowance, FWA (mm) = Δ / (4 × TPC), and may change trim slightly because the centres of buoyancy and flotation shift relative to G.

27.	What are Shear Force (SF) and Bending Moment (BM) in a ship's hull girder?

Ans: Shear Force is the vertical tendency of adjacent hull sections to slide past one another due to unequal weight and buoyancy at that frame; Bending Moment is the cumulative torque causing the entire hull girder to bend into Hogging or Sagging.

28.	Where along the ship's length do Maximum Shear Force and Maximum Bending Moment typically occur?

Ans: Maximum Bending Moment typically occurs near amidships (where Shear Force crosses zero); Maximum Shear Force typically occurs near the quarter-lengths (¼L from bow and stern, or at bulkheads between full and empty holds).

29.	What are Racking, Torsional, Panting, and Pounding (Slamming) stresses?

Ans: Racking: transverse distortion during heavy rolling; Torsion: twisting in quartering seas (critical on wide-hatch container ships); Panting: in-and-out breathing of bow plating due to wave pressure changes; Pounding: violent slamming of the flat forward bottom when pitching in ballast.

30.	What is the Margin Line in passenger ship damage stability?

Ans: An imaginary line drawn at least 76 mm (3 inches) below the upper surface of the Bulkhead Deck at the ship's side.

31.	Define Floodable Length and Permeability (μ).

Ans: Floodable Length at any point along the hull is the maximum portion of length centered at that point that can be flooded without submerging the Margin Line; Permeability is the percentage of a compartment's total volume that can be occupied by water (e.g., 95% for empty tanks, 85% for engine room, 60% for cargo holds).

32.	What is the "Lost Buoyancy" method vs. "Added Weight" method in bilging calculations?

Ans: Lost Buoyancy treats the flooded compartment as no longer part of the buoyant hull (KG stays constant, waterplane decreases, B rises); Added Weight treats the incoming water as liquid cargo loaded with free surface.

33.	What happens to GM when a midship compartment is bilged (flooded)?

Ans: Bodily sinkage increases KB, while loss of intact waterplane area reduces BM (BM=I/V); depending on beam vs. draft, GM may increase or decrease.

34.	What are Symmetrical vs. Unsymmetrical Flooding and how is Cross-Flooding managed?

Ans: Symmetrical flooding extends equally port and starboard; Unsymmetrical flooding (due to longitudinal bulkheads) causes severe list and must be equalized within 15 minutes using automatic cross-flooding ducts.

35.	What is a Damage Control Plan and Booklet?

Ans: A statutory diagram displayed on the bridge/CCR showing watertight subdivision boundaries, openings, closing appliances, and cross-flooding arrangements, accompanied by a booklet guiding the Master on damage stability recovery.

36.	Define Hydrostatic Draft.

Ans: The true mean draft measured at the Center of Flotation (LCF), which must be used when entering the Hydrostatic Particulars tables so trim distortions are eliminated.

37.	Why does a vessel lose GM immediately upon grounding on a rock or shoal at a falling tide?

Ans: The upward ground reaction force (P) at the keel acts exactly like discharging P tons from the keel (K), causing a virtual rise in the Center of Gravity (G) while the lost buoyancy lowers M.

38.	At what point during Dry Docking is a vessel at maximum risk of tipping or losing stability?

Ans: The Critical Instant, which starts when the stern post first touches the blocks (if trimmed by the stern) and peaks just before the entire keel settles flat along the blocks.

39.	Why is it preferable to enter a Dry Dock with minimal trim (less than 1% of length)?

Ans: A large stern trim builds up a massive upward force P at the aft keel block before the rest of the hull is supported by bilge shores/blocks, risking structural crushing of the stern frame or capsizing on the blocks.

40.	How do you prepare a ship for Dry Docking?

Ans: Minimize trim and ensure zero list, maximize positive GM, press up or completely empty tanks to eliminate free surface, secure all heavy gear, prepare the Docking Plan (showing plugs, transducers, sea chests), and record all tank soundings before entering.

## 3. Ship Construction, Framing & Structural Members (Q41–60)

41.	Compare Transverse Framing, Longitudinal Framing, and Combined Framing.

Ans: Transverse: closely spaced ribs/frames (good for general cargo ships); Longitudinal: closely spaced longitudinal stiffeners supported by wide web frames (resists hogging/sagging bending on long tankers and bulkers); Combined: longitudinal framing on deck and double bottom with transverse framing on side shells.

42.	Define Garboard Strake, Sheer Strake, and Bilge Strake.

Ans: Garboard Strake: the bottom shell plating strake immediately adjacent to the flat plate keel on either side; Sheer Strake: the uppermost strake of side shell plating meeting the strength deck; Bilge Strake: the curved plating joining the flat bottom to the vertical side shell.

43.	Define Camber, Sheer, Tumblehome, Flare, and Rise of Floor.

Ans: Camber: transverse upward arch of the weather deck to shed water; Sheer: longitudinal upward curve of the deck toward bow and stern (adds reserve buoyancy); Tumblehome: inward slope of upper side shell; Flare: outward curvature of the forward bow above the waterline; Rise of Floor: slight upward slope of the flat bottom from keel to bilge.

44.	What structural arrangements at the forward end resist Panting?

Ans: Panting stringers (horizontal side girders spaced every ~2m vertically), panting beams tied across the forepeak, perforated wash plates, and 15–20% thicker shell plating forward of the collision bulkhead.

45.	What structural arrangements at the forward bottom resist Pounding (Slamming)?

Ans: Solid floors on every frame space in the pounding region (between 0.05L and 0.25L–0.30L from FP), additional longitudinal side girders spaced closer together, and thickened bottom shell plating.

46.	Where must the Collision Bulkhead be located on a cargo ship?

Ans: Watertight from double bottom to the freeboard deck, located between 0.05L and 0.08L (or 0.05L+3 m, whichever is less) abaft the Forward Perpendicular (FP).

47.	Can any doors or manholes be cut into the Collision Bulkhead below the freeboard deck?

Ans: No doors, manholes, or access openings are permitted; only one single pipe may pierce it to service the forepeak tank, fitted with a screw-down valve on the collision bulkhead operable from above the freeboard deck.

48.	What is the minimum number of Transverse Watertight Bulkheads on a conventional cargo ship?

Ans: At least four: 1) Collision Bulkhead, 2) Forward Engine Room Bulkhead, 3) Aft Engine Room (After Peak) Bulkhead, plus intermediate hold bulkheads according to ship length.

49.	Compare a Solid Floor, Plate (Bracket) Floor, and Watertight Floor in a Double Bottom.

Ans: Solid Floor: a continuous transverse plate with lightening holes and manholes plus stiffeners; Bracket Floor: open frame made of top frame bar and bottom reverse frame connected only by end brackets at the center girder and margin plate; Watertight Floor: solid unperforated plate separating two different double-bottom tanks.

50.	What is a Duct Keel and where is it fitted?

Ans: A hollow watertight box tunnel running along the centerline inside the double bottom from the forward engine room bulkhead to the forward cargo hold, housing bilge/ballast/bunker pipelines so pipes don't pass through cargo holds and remain accessible at sea.

51.	What safety precautions apply before entering a Duct Keel?

Ans: Treat it strictly as a high-risk Enclosed Space: complete an Enclosed Space Entry Permit, ventilate mechanically, test atmosphere (O₂, HC, toxic gases), verify lighting, and check for pipeline leaks.

52.	What is a Bilge Keel and why is it connected to the hull via a Doubler Plate (Flat Bar)?

Ans: A long fin projecting from the turn of the bilge over the midship half-length to dampen rolling; welded to a flat doubling strip rather than directly to the bilge strake so that if the bilge keel is torn off by a tug or berth, it rips the weld off the doubler without tearing a hole in the hull shell plating.

53.	What is a Coffin Plate and a Shoe Plate at the sternframe?

Ans: The Coffin Plate is the U-shaped shell plate wrapping around the aft bossing/sternframe; the Shoe Plate connects the bottom of the sternframe across to the keel plate.

54.	What is the Rudder Carrier Bearing and Pintle Jumping Clearance?

Ans: The Rudder Carrier Bearing inside the steering flat supports the vertical weight of the rudder stock and blade; a Jumping Stopper is fitted under the rudder horn with ~2–3 mm clearance to prevent the rudder pintles from jumping out of their gudgeons in heavy seas.

55.	How is a Rudder Blade tested for watertightness in Dry Dock?

Ans: Compressed air at slight pressure (~0.2 bar) is applied through the top plug while soapy water is brushed along welds, or the rudder is filled with water/kerosene to check for weeping cracks (empty via the bottom drain plug before undocking).

56.	What are Sacrificial Anodes vs. Impressed Current Cathodic Protection (ICCP)?

Ans: Sacrificial Anodes are blocks of Zinc or Aluminum welded to the underwater hull/ballast tanks that corrode preferentially to steel; ICCP uses hull reference electrodes and inert titanium/platinum anodes supplied with low-voltage DC current to suppress galvanic hull corrosion.

57.	Why must Zinc Anodes never be fitted inside Oil Tanker Cargo Tanks?

Ans: If a heavy zinc anode detaches and falls onto rusty steel at the bottom of a tank, the impact creates an incendive spark; Aluminum anodes face strict drop-energy limits (<275 Joules) inside tanks.

58.	Why is the ICCP system switched off when an Oil Tanker berths at a terminal?

Ans: To avoid stray electrical potential differences between the ship's hull and the shore jetty structure during hose connection.

59.	What is a Corrugated Bulkhead and what are its advantages?

Ans: A bulkhead pressed with vertical or horizontal trapezoidal troughs instead of welded stiffeners; eliminates stiffener brackets, reduces structural weight for the same strength, and makes tank/hold washing much faster on chemical tankers and bulkers.

60.	What are Upper and Lower Stools on a Bulk Carrier transverse bulkhead?

Ans: Box-shaped sloped structures at the top and bottom of a vertically corrugated cargo hold bulkhead; the lower stool sheds bulk cargo toward the center for grabs and absorbs huge bending moments at the bulkhead base.

## 4. Dry Docking, Load Lines & Watertight Integrity (Q61)

61.	What are the dimensions of the Load Line Disc (Plimsoll Mark) and Deck Line?

Ans: Deck Line: 300 mm×25 mm painted at the freeboard deck level; Load Line Ring: 300 mm outside diameter (25 mm thick) intersected by a 450 mm×25 mm horizontal line whose upper edge passes through the center of the ring (Summer draft).

## 5. Additional Stability, Rolling & Loading Questions (Q62–80)

62.	What is stability?

Ans: The ability of a ship to return to her original upright position after the external force that heeled her has been removed.

63.	How is KM obtained, and what is BM?

Ans: KM = KB + BM, where BM = I/V (second moment of area of the waterplane about the centreline ÷ underwater volume). KM is read from the hydrostatic curves for the draught.

64.	On what does the free surface effect depend?

Ans: On the length × breadth³ of the free surface and the density of the liquid; it is independent of the quantity of liquid in a slack tank and of the tank's height. Halving the breadth reduces the effect to one-eighth.

65.	What is the approximate formula for the rolling period?

Ans: T ≈ 0.8 × B / √GM (B = breadth). A longer rolling period means a smaller GM, so the period measured on board gives a quick check of GM.

66.	What is synchronous rolling and how do you stop it?

Ans: It occurs when the wave encounter period equals the ship's natural rolling period, so the roll amplitude builds dangerously. Alter course and/or speed to change the encounter period.

67.	What is parametric rolling?

Ans: Roll caused by cyclic changes of waterplane shape (and so of GM) as waves pass, mainly in head or following seas when the encounter period is about half the natural roll period. Container ships with flared bows and wide sterns are especially at risk. Alter course or speed.

68.	What is the effect on GM of adding weight above or below G?

Ans: Weight added above G raises G and reduces GM; weight added below G lowers G and increases GM.

69.	What precautions are needed for a heavy lift?

Ans: Check GM and free surface before the lift (G effectively moves to the point of suspension), secure the boom/crane with guys, ballast to limit heel, and stop in adverse weather or swell.

70.	What is the angle of deck-edge immersion?

Ans: The angle at which the deck edge enters the water. The GZ curve normally has a point of inflection here.

71.	What is the angle of vanishing stability?

Ans: The angle at which GZ returns to zero on the GZ curve; beyond it the ship will capsize.

72.	What is the Weather Criterion?

Ans: A check that the ship can withstand a steady beam wind combined with a gust and a rolling motion in waves, using heeling levers compared with the GZ curve (2008 IS Code).

73.	Why must the stability booklet be kept on board?

Ans: It contains the approved loading conditions, hydrostatic data and KN curves. The Master must check stability before sailing and compare the loaded condition with the approved ones.

74.	What is a lightweight survey and how often is it done?

Ans: A survey to verify the lightship weight and centre of gravity, at intervals not exceeding 5 years. [Verify] If the deviation exceeds about 2% of lightship displacement (or 1% of L in LCG), re-inclining is required.

75.	What are the main steps of a draught survey?

Ans: Read the six draughts, correct for trim, hog/sag and density, find displacement from hydrostatics, deduct ballast, fuel, fresh water and stores, deduct lightship and the constant, and the remainder is the cargo.

76.	What precautions apply when carrying timber deck cargo?

Ans: Stow and lash to the Timber Code, allow for absorbed water (about 10%) in stability calculations, keep a positive GM throughout, and keep a walkway/lifelines for the crew.

77.	How does ice accretion affect stability?

Ans: It adds weight high up, so G rises and GM falls, and increases the windage area. Remove the ice promptly and alter course to reduce spray.

78.	How does consuming bunkers affect stability?

Ans: Weight is removed from low tanks so G rises and GM reduces; free surface effects also appear in partially empty tanks. Check stability for the arrival condition before sailing.

79.	How does high deck cargo or high container stacking affect stability?

Ans: G rises and GM falls, and windage and lashing loads increase. Check against the approved loading conditions and the cargo securing manual.

80.	Why is a satisfactory GM alone not a guarantee of safety?

Ans: The GZ curve may still have a small range or low maximum GZ. Always check the full IS Code criteria, not only the initial GM.

## 6. Additional Ship Construction, Equipment & Surveys (Q81–105)

81.	What are the types of keel?

Ans: Flat plate keel (the most common), bar keel, and duct keel (pipe tunnel).

82.	What are the purposes of the double bottom?

Ans: Protection against bottom damage, carriage of ballast/fuel/fresh water, trimming and heeling control, and a strong base for the hull girder. SOLAS II-1 Reg 9 requires double bottoms on most cargo ships of 500 GT and over (tankers have separate MARPOL requirements).

83.	What precautions apply to watertight doors?

Ans: Keep them closed at sea, check local and remote operation and the bridge indicators, and test them during drills.

84.	What are the types of rudder?

Ans: Unbalanced, balanced and semi-balanced; spade rudders; and rudders with a skeg or horn (e.g. mariner type).

85.	What are hogging and sagging?

Ans: Hogging: the ship is supported at the middle by a wave crest, so the deck is in tension and the keel in compression. Sagging: the ship is supported at the ends by crests, so the deck is in compression and the keel in tension.

86.	What types of corrosion occur on ships?

Ans: Uniform (general), galvanic (dissimilar metals), pitting, crevice, and erosion/abrasion corrosion.

87.	What is the PSPC?

Ans: The IMO Performance Standard for Protective Coatings for ballast tanks (SOLAS II-1 Reg 3-2), with a target coating life of 15 years.

88.	What are freeing ports?

Ans: Openings in the bulwarks to clear water from the deck quickly. They must never be blocked or fitted with unapproved closures.

89.	What must be done with ventilators and air pipes in heavy weather?

Ans: Their weathertight closing appliances must be kept closed to preserve weathertight integrity.

90.	What is the purpose of a sounding pipe?

Ans: To allow tanks and bilges to be sounded. They are fitted with self-closing caps or cocks where required.

91.	What is the purpose of a bulbous bow?

Ans: It reduces wave-making resistance and improves fuel economy near the design speed. Take care with anchors and in berthing.

92.	What are the checks on the stern tube seal?

Ans: Check for leakage at the forward and aft seals, the lubricating oil level and temperature, and the oil sample for water.

93.	What is a classification society and what is IACS?

Ans: A recognised organisation that sets and verifies rules for hull and machinery construction and maintenance. IACS is the International Association of Classification Societies.

94.	What types of survey does a cargo ship undergo?

Ans: Initial, annual, intermediate, special (renewal, every 5 years), dry-dock/bottom survey, and for bulk carriers and tankers the Enhanced Survey Programme (ESP).

95.	What is the dry-docking interval?

Ans: Two bottom inspections in any 5 years, with no more than 36 months between. [Verify] Check flag state requirements.

96.	What is inspected while the ship is in dry dock?

Ans: Hull plating and coating, rudder and bearings, propeller and tailshaft clearances, sea chests and valves, anodes, bilge keels, anchors and cables.

97.	What are the statutory certificates of a cargo ship and their validity?

Ans: Cargo Ship Safety Construction, Safety Equipment, Safety Radio, Load Line, IOPP, ISPP, IAPP, ISSC, SMC and MLC certificates. Most are valid for 5 years with annual/intermediate endorsements.

98.	What are the parts of an anchor cable?

Ans: Shackles of cable, joining shackles, end links and a bitter end secured in the chain locker. A standard shackle is 27.5 m (15 fathoms).

99.	What are the types of anchor?

Ans: Stockless anchors, and HHP and SHHP (high and super high holding power) anchors, which have more holding power for the same weight.

100.	What is the Equipment Number?

Ans: A number calculated from the ship's dimensions and windage area that determines the number, weight and size of anchors, chain and mooring ropes from the class rules.

101.	What do you check when inspecting anchor chain?

Ans: Reduction in link diameter (replace at about 12% wear), stud looseness, cracks, shackle pins, and the condition of the bitter end.

102.	What are the main mooring safety points?

Ans: Know the SWL of lines, keep clear of the snap-back zone, check winch brakes and bollards, and do not mix rope types of different elasticity in one lead.

103.	What are the SOLAS requirements for steering gear?

Ans: The main steering gear must put the rudder from 35° one side to 35° the other at maximum service speed, and 35° to 30° within 28 seconds. The auxiliary gear must go 15° to 15° at half speed or 7 knots (whichever is greater) within 60 seconds. Test before departure (within 12 hours) and hold emergency steering drills every 3 months.

104.	What is the procedure for entering an enclosed space?

Ans: Permit to work, ventilate and test the atmosphere (O₂ 20.9%, flammable gas below 1% LEL, toxic gases), standby person, rescue equipment ready and communications maintained.

105.	What precautions apply to a bow thruster?

Ans: Check the power supply, warn people near the hull, check the tunnel is clear, and use only within the speed limits.

## 7. Load Lines, Freeboard & Tonnage (Q106–116)

106.	What is the purpose of the Load Line Convention (1966)?

Ans: It sets minimum freeboard to ensure adequate reserve buoyancy and watertight integrity for the ship.

107.	Define freeboard.

Ans: The distance from the upper edge of the deck line to the upper edge of the load line mark.

108.	What are the load line marks?

Ans: TF (Tropical Fresh), F (Fresh), T (Tropical), S (Summer), W (Winter) and WNA (Winter North Atlantic). Timber ships have the additional lumber marks.

109.	How are the Winter, Tropical and Fresh marks positioned?

Ans: Winter is 1/48 of the moulded summer draught below S; Tropical is 1/48 above S; Fresh Water is above the corresponding mark by the Fresh Water Allowance.

110.	What are Type A and Type B ships?

Ans: Type A ships carry liquid cargoes in bulk and have high integrity (lowest freeboard). Type B is all other ships.

111.	What are the main conditions of assignment?

Ans: Structural strength and stability information, weathertight and watertight integrity of hatchways, doors, ventilators and air pipes, freeing ports, guard rails, and protection of the crew.

112.	What are the load line surveys?

Ans: Initial, annual (within 3 months either side of the anniversary date) and renewal survey at 5-year intervals.

113.	Define gross tonnage and net tonnage.

Ans: GT is a measure of the total enclosed volume of the ship (GT = K₁V). NT represents the cargo-carrying capacity and cannot be less than 0.30 GT.

114.	Define displacement, lightship and deadweight.

Ans: Displacement is the total weight of the ship; lightship is the weight of the hull, machinery and equipment; deadweight = displacement − lightship (cargo, fuel, water, stores, crew).

115.	What must the Master do if the load line is submerged?

Ans: He must not sail with the load line submerged; reduce the load or ballast until the correct line for the zone and season is at the waterline.

116.	Which tonnage is used for port dues?

Ans: Normally GT or NT, depending on the port tariff.

## 8. Fire Prevention & Fire-Fighting (SOLAS II-2) (Q117–144)

117.	Explain the fire triangle and tetrahedron.

Ans: The fire triangle: fuel, oxygen and heat. The tetrahedron adds the chemical chain reaction.

118.	What are the classes of fire?

Ans: A: solids; B: flammable liquids; C: gases; D: metals; F: cooking oils and fats.

119.	What are the methods of extinguishing a fire?

Ans: Starving (removing fuel), smothering (removing oxygen), cooling, and interrupting the chain reaction.

120.	Name the extinguishing agents and their uses.

Ans: Water (class A), foam (class B), CO₂ (electrical and class B), dry powder (B, C and electrical), wet chemical (class F). Halon is banned.

121.	Why is water not used on electrical or oil fires?

Ans: Water conducts electricity, and on burning oil it can spread the fire.

122.	What are the CO₂ quantities for fixed fire-extinguishing systems?

Ans: [Verify] Cargo spaces: 30% of the gross volume of the largest cargo space; machinery spaces: 40% of the gross volume (35% if the casing volume is excluded).

123.	Describe the CO₂ release procedure for the engine room.

Ans: Muster and count the crew, stop machinery and fuel supply, close ventilators and openings, sound the pre-discharge alarm, open the release cabinet and operate the pilot cylinders, then release the main CO₂. Keep the space closed and cool the boundaries before ventilating.

124.	Why is there a time delay before CO₂ release?

Ans: To allow personnel to leave the space. An audible and visual alarm operates during the delay.

125.	What are the dangers of CO₂?

Ans: It causes asphyxiation and cold burns, and is heavier than air so it collects in low spaces.

126.	Name the types of portable extinguisher and their use.

Ans: Water, foam, dry powder (ABC), CO₂ and wet chemical, each matched to the class of fire.

127.	What are the SOLAS requirements for fire pumps?

Ans: [Verify] Cargo ships of 1000 GT and above: at least two independent power-driven pumps; an emergency fire pump on ships of 2000 GT and over; minimum hydrant pressure 0.27 N/mm² (6000 GT and over) or 0.25 N/mm² (below 6000 GT).

128.	What are the hose and nozzle requirements?

Ans: Each hydrant must have a hose, a dual-purpose (jet/spray) nozzle and a spanner. [Verify] Hose length at least 10 m; maximum 15 m in machinery spaces, 20 m on other decks, and 25 m on open decks with a breadth over 30 m.

129.	What does a fireman's outfit contain?

Ans: Protective clothing, boots, gloves, helmet, electric safety lamp (3 hours), axe, SCBA (about 30 minutes) and a fireproof lifeline. At least two outfits are carried.

130.	What is an EEBD?

Ans: An emergency escape breathing device giving at least 10 minutes of air for escaping from a space with smoke or toxic gas.

131.	How does the fire detection and alarm system work?

Ans: Smoke, heat and flame detectors and manual call points feed an indicator panel on the bridge. The system is tested regularly.

132.	Explain A, B and C class divisions.

Ans: A class: steel or equivalent, resisting the passage of smoke and flame for 60, 30 or 0 minutes (A-60, A-30, A-0). B class: non-combustible panels, 30 or 15 minutes. C class: non-combustible materials.

133.	What is the fire control plan?

Ans: A plan permanently exhibited showing control stations, fire sections, detection and extinguishing systems and means of access. A copy is kept in a prominently marked weathertight container outside the deckhouse for shore fire-fighters.

134.	How often are fire drills held?

Ans: At least monthly, and each crew member should take part in at least one fire drill and one abandon ship drill each month. If over 25% of the crew have not taken part in a drill in the previous month, hold one within 24 hours of leaving port.

135.	What is your first action when you discover a fire?

Ans: Raise the alarm, report to the bridge giving the location and nature of the fire, and fight the fire only if it is safe to do so.

136.	What are the immediate actions for an engine room fire?

Ans: Raise the alarm, shut the fuel quick-closing valves, stop ventilation, boundary cool, evacuate and account for all personnel, and release the fixed system if necessary.

137.	How do you deal with a fire in a cargo hold?

Ans: Stop ventilation, close the hatch, use the fixed CO₂, high expansion foam or inert gas system, and monitor temperatures. Do not open the hold until the fire is out.

138.	How do you deal with a galley fire?

Ans: Switch off the power/gas, smother with a fire blanket or wet chemical extinguisher, and never use water on cooking oil.

139.	Which cargoes and materials are prone to spontaneous combustion?

Ans: Oily rags, coal, DRI, fishmeal, copra and wet cotton. Ventilate and monitor temperatures.

140.	What is required for hot work?

Ans: A hot work permit, a gas-free test, removal of combustibles, fire protection and fire watch ready, and authority from the Master or Chief Officer.

141.	What is boundary cooling and what must you watch?

Ans: Cooling the bulkheads and decks around a fire to stop it spreading. Beware of the free surface effect from the fire-fighting water.

142.	What is the oxygen limit for an inert gas system, and which tankers need one?

Ans: Oxygen not more than 8% by volume, with a positive pressure in the tanks. [Verify] Required for oil tankers of 20,000 DWT and over.

143.	What are low and high expansion foam used for?

Ans: Low expansion foam (up to about 12:1) for deck and tank fires; high expansion foam (over 200:1) for engine rooms and cargo holds.

144.	How are smoke and fire spread controlled?

Ans: Close fire dampers, doors and ventilation to isolate the space and keep smoke from spreading.

## 9. Life-Saving Appliances & Abandon Ship (SOLAS III) (Q145–168)

145.	What is the LSA Code?

Ans: The international code setting out the technical requirements for the life-saving appliances required by SOLAS Chapter III.

146.	What is the abandon ship / general emergency alarm signal?

Ans: Seven or more short blasts followed by one long blast on the whistle or alarm bell.

147.	What does the muster list contain?

Ans: The emergency signals, the duties of each crew member, and substitutes for key personnel. It is posted in conspicuous places.

148.	What are the lifeboat capacity requirements on a cargo ship?

Ans: [Verify] Lifeboats on each side with a total capacity of 100% of persons on board per side, or a free-fall lifeboat at the stern for 100% plus liferafts. Check SOLAS III/31 for the exact arrangement.

149.	What is the equipment of a lifeboat?

Ans: Oars, buoyant lifeline, bailers, sea anchor, compass, rations (10,000 kJ per person), fresh water (3 litres per person), first-aid kit, fishing kit, flares, torch, whistle and radar reflector.

150.	What are the weekly and monthly lifeboat checks?

Ans: Weekly: visual inspection and run the engine for at least 3 minutes ahead and astern. Monthly: check all equipment, swing out the davits, and inspect the lifejackets and immersion suits.

151.	How often are lifeboats launched?

Ans: [Verify] Lowered into the water with the assigned crew at least once every 3 months; free-fall lifeboats are launched at least once every 6 months.

152.	What are the requirements for lifejackets?

Ans: One for every person on board plus extras at watch stations. A lifejacket light of at least 0.75 cd for 8 hours, a whistle and retro-reflective tape.

153.	What performance tests must a lifejacket pass?

Ans: It can be donned in 1 minute, worn after a jump from 4.5 m, and turns an unconscious person face up within a few seconds.

154.	What is the difference between an immersion suit and a thermal protective aid?

Ans: An immersion suit protects against hypothermia in cold water and has buoyancy; a TPA (thermal protective aid) is a bag-type aid for persons without suits.

155.	How many lifebuoys must a cargo ship carry?

Ans: [Verify] By length: under 100 m = 8; 100–150 m = 10; 150–200 m = 12; 200 m and over = 14. Half have self-igniting lights, two have a light and smoke signal, and at least one on each side has a buoyant lifeline.

156.	What markings are required on a lifebuoy?

Ans: The ship's name and port of registry in block capitals, and retro-reflective material.

157.	What are the types and characteristics of pyrotechnics?

Ans: Parachute flares reach about 300 m and burn about 40 seconds; hand flares burn for about 1 minute; buoyant smoke signals last about 3 minutes. The bridge carries 12 parachute flares.

158.	What is the line-throwing appliance requirement?

Ans: Range at least 230 m in calm weather, with 4 rockets and 4 lines.

159.	What is an EPIRB and how is it tested?

Ans: A 406 MHz float-free emergency beacon with a hydrostatic release and a battery life of at least 48 hours. Test monthly using the self-test function; the HRU is replaced every 2 years and the battery at expiry.

160.	What is a SART and how many are required?

Ans: A 9 GHz radar transponder that shows 12 dots on the interrogating radar. Standby of 96 hours and 8 hours transmitting. Cargo ships of 500 GT and over carry at least 2.

161.	How many two-way VHF radiotelephones are carried?

Ans: Three on ships of 500 GT and over, and two on ships of 300 to 500 GT.

162.	What are the main details of a liferaft?

Ans: Inflates within 1 minute, painter of 15 m with a weak link, hydrostatic release unit, and annual servicing at an approved station.

163.	How do you launch a throw-over liferaft?

Ans: Check the painter is secured to the ship, throw the raft overboard, pull the painter to inflate it, and board directly from the ship if possible.

164.	What are the requirements for a rescue boat?

Ans: At least one must be carried and be capable of being launched quickly (within about 5 minutes) and recovering persons from the water.

165.	How often are falls of survival craft renewed?

Ans: At intervals not exceeding 5 years, or earlier if deteriorated. Inspect regularly and turn end for end where fitted.

166.	What are the actions for survival after abandoning ship?

Ans: Muster in the lifeboat, stay close to the ship, deploy the sea anchor, use signals, ration food and water, and keep warm.

167.	What are the actions for a man overboard?

Ans: Throw a lifebuoy, raise the alarm, note the position, execute the appropriate turn (Williamson, Anderson or Scharnow), post lookouts and prepare the rescue boat.

168.	What is a Williamson turn?

Ans: Put the rudder hard over, and when 60° off the original course, put it hard over the other way and steady on the reciprocal course.

## 10. MARPOL & Environmental Protection (Q169–201)

169.	What are the Annexes of MARPOL?

Ans: I Oil; II Noxious liquid substances in bulk; III Harmful substances in packaged form; IV Sewage; V Garbage; VI Air pollution.

170.	What are the discharge conditions for oil from machinery spaces outside special areas?

Ans: Ship of 400 GT and over, proceeding en route, more than 12 NM from the nearest land, oil content not more than 15 ppm, and with the oily water separator and oil content monitor operating.

171.	Name the special areas under Annex I.

Ans: [Verify] Mediterranean, Baltic, Black Sea, Red Sea, Gulfs area, Gulf of Aden, Antarctic, North West European waters, Oman area of the Arabian Sea and Southern South African waters.

172.	What is the Oil Record Book?

Ans: Part I for all ships of 400 GT and over (machinery space operations); Part II for tankers (cargo and ballast). Keep on board for 3 years after the last entry.

173.	Which operations are recorded in Oil Record Book Part I?

Ans: Ballasting or cleaning of fuel tanks, discharge of ballast from fuel tanks, collection and disposal of oil residues, overboard discharge of bilge water, and bunkering.

174.	What are the functions of the OWS and oil content monitor?

Ans: The OWS separates oil from bilge water to 15 ppm. The oil content monitor alarms and stops discharge overboard if the oil content exceeds 15 ppm.

175.	What is SOPEP?

Ans: The Shipboard Oil Pollution Emergency Plan (tankers of 150 GT and over, other ships of 400 GT and over) giving reporting procedures, actions and contacts.

176.	What action do you take on an oil spill?

Ans: Stop the transfer, raise the alarm, implement SOPEP, contain and clean up, and report to the coastal state.

177.	What does a Protocol I report contain?

Ans: Ship identity, time and position, the incident (type), quantity and type of oil, assistance required, and the weather.

178.	What is the IOPP certificate?

Ans: The International Oil Pollution Prevention certificate, valid for 5 years with annual and intermediate surveys.

179.	What are the main tanker design features for pollution prevention?

Ans: Segregated ballast tanks (SBT), crude oil washing (COW), double hull, ODME, and inert gas.

180.	What are the Annex II categories?

Ans: X (major hazard), Y, Z, and OS (other substances). Discharge is controlled by category using the P&A Manual.

181.	What documents are required under Annex II?

Ans: The Procedures and Arrangements (P&A) Manual and the Cargo Record Book.

182.	What does Annex III cover?

Ans: Harmful substances in packaged form: packing, marking, labelling, documentation and stowage. Marine pollutants are identified in the IMDG Code.

183.	What are the Annex IV (sewage) discharge rules?

Ans: [Verify] Comminuted and disinfected sewage: more than 3 NM from the nearest land. Untreated sewage: more than 12 NM at not less than 4 knots. Approved sewage treatment plant: no distance restriction if effluent standards are met.

184.	To whom does Annex IV apply?

Ans: Ships of 400 GT and over, or certified to carry more than 15 persons.

185.	What are the key Annex V (garbage) rules?

Ans: All plastics are prohibited, as are cooking oil, fishing gear and incinerator ash from plastics. Ground food waste (25 mm) at 3 NM or more; other food waste at 12 NM or more; cargo residues (non-HME) at 12 NM or more; animal carcasses at 100 NM or more.

186.	What applies in Annex V special areas?

Ans: Food waste is allowed only at 12 NM or more from the nearest land. Examples: Mediterranean, Baltic, Black Sea, Red Sea, Gulfs area, North Sea, Antarctic and Wider Caribbean.

187.	What records are kept under Annex V?

Ans: A Garbage Record Book for ships of 400 GT and over (or certified for 15 or more persons), kept for 2 years, a Garbage Management Plan, and placards on ships of 12 m and over.

188.	What is the sulphur limit under Annex VI?

Ans: 0.50% m/m globally since 1 January 2020, and 0.10% in Emission Control Areas.

189.	What are the Emission Control Areas?

Ans: [Verify] Baltic Sea, North Sea, North American, US Caribbean Sea, and the Mediterranean Sea SOx ECA (from 1 May 2025). Others may have been added.

190.	What are the NOx tiers?

Ans: Tier I, Tier II (2011) and Tier III (2016, applying in NOx ECAs for new ships).

191.	Explain EEDI, EEXI and SEEMP.

Ans: EEDI applies to new ships, EEXI to existing ships (from 2023) and SEEMP is the ship energy efficiency management plan.

192.	What is the CII?

Ans: The Carbon Intensity Indicator: an annual operational rating from A to E (from 2023). Ships rated D for 3 years or E for 1 year must submit a corrective action plan.

193.	How long are the bunker delivery note and fuel sample kept?

Ans: The BDN for 3 years and the MARPOL sample for at least 12 months.

194.	What is a fuel changeover procedure?

Ans: A written procedure for changing to compliant fuel before entering an ECA, recording the quantity of low-sulphur fuel, date, time and position of the changeover in a logbook.

195.	What are the rules on ozone-depleting substances?

Ans: New installations are prohibited and use must be recorded in the ODS record book.

196.	What does the Ballast Water Management Convention require?

Ans: D-1 exchange (at least 200 NM from land in depths of at least 200 m) or D-2 treatment standard (fewer than 10 viable organisms of 50 µm or more per m³). A BWM plan and record book (kept 2 years) are required.

197.	What does the anti-fouling convention require?

Ans: TBT-based paints are banned (AFS Convention); an AFS certificate is required for ships of 400 GT and over.

198.	What is the Hong Kong Convention?

Ans: It sets standards for safe ship recycling and requires an Inventory of Hazardous Materials (IHM) on board.

199.	What is the Polar Code?

Ans: The mandatory code for ships operating in polar waters, requiring a Polar Ship Certificate.

200.	What are port reception facilities?

Ans: Facilities that MARPOL requires parties to provide for oily wastes, sewage and garbage from ships.

201.	What are the penalties for MARPOL violations?

Ans: Fines or prosecution under flag and coastal state laws, which can include detention of the ship and personal liability for the Master and officers.

## 11. ISM, ISPS, MLC, STCW, Port State Control & Maritime Law (Q202–216)

202.	What is the ISM Code?

Ans: The International Safety Management Code, which sets the standard for safe management and operation of ships and for pollution prevention.

203.	What are the objectives of the ISM Code?

Ans: To ensure safety at sea, prevent injury and loss of life, avoid damage to the environment and property, and continually improve safety management skills.

204.	What are the DOC and SMC?

Ans: The Document of Compliance is issued to the company and the Safety Management Certificate to the ship. Both are valid for 5 years with verification audits.

205.	Who is the DPA?

Ans: The Designated Person Ashore, who has direct access to top management and monitors the safety and pollution prevention aspects of the ship.

206.	What is the Master's responsibility under the ISM Code?

Ans: Implement the safety and environmental policy, motivate the crew, review the SMS and report deficiencies to the company. The Master has overriding authority.

207.	What is a non-conformity?

Ans: An observed situation where objective evidence indicates non-fulfilment of a requirement. Grades: observation, non-conformity, and major non-conformity.

208.	What are the ISPS security levels?

Ans: Level 1 (normal), Level 2 (heightened) and Level 3 (exceptional).

209.	What are the key ISPS roles and documents?

Ans: SSO, CSO and PFSO; the Ship Security Plan (SSP); the Ship Security Alert System (SSAS); and the ISSC, valid for 5 years.

210.	What are the MLC 2006 hours of rest requirements?

Ans: At least 10 hours of rest in any 24-hour period and 77 hours in any 7-day period. Rest may be divided into at most two periods, one of which is at least 6 hours, and the interval between rest periods must not exceed 14 hours.

211.	What other points does MLC 2006 cover?

Ans: Minimum age of 16 (18 for night work and hazardous work), the Seafarer's Employment Agreement, wages, repatriation, accommodation, and the DMLC Parts I and II.

212.	What are the key points of STCW?

Ans: Training, certification and watchkeeping standards, hours of rest, and alcohol limits of 0.05% BAC (0.25 mg/l breath).

213.	What is Port State Control?

Ans: Inspection of foreign ships in port to verify compliance with international conventions. Regional MOUs include Paris, Tokyo and the Indian Ocean MOU. Serious deficiencies can lead to detention.

214.	What are the main Indian maritime laws?

Ans: The Merchant Shipping Act 2025 (in force from 15 March 2026, replacing the 1958 Act), the Coastal Shipping Act 2025, the Carriage of Goods by Sea Act 2025 (replacing the 1925 Act) and the Marine Aids to Navigation Act 2021. The DG Shipping and MMDs administer them.

215.	What are the Master's duties after a collision?

Ans: Render assistance to the other ship, exchange names, ports of registry and details, report to the flag state, make a protest and record the facts in the log book.

216.	What are a Note of Protest, general average and salvage?

Ans: A Note of Protest is the Master's declaration of damage or events, normally made within 24 hours of arrival. General average (York-Antwerp Rules) shares a deliberate sacrifice or extraordinary expense proportionally. Salvage is generally under Lloyd's Open Form on a no cure, no pay basis, with SCOPIC for environmental protection.

## 12. Medical Care & Emergency Response (Q217–226)

217.	What is the CPR ratio?

Ans: 30 chest compressions to 2 rescue breaths, at 100–120 compressions per minute.

218.	How do you treat an unconscious casualty?

Ans: Check danger, response, airway and breathing; place in the recovery position if breathing; and obtain radio medical advice (TMAS).

219.	How do you control severe bleeding?

Ans: Apply direct pressure, elevate the limb and apply a dressing. Do not remove the first dressing; add more over it.

220.	How do you treat burns?

Ans: Cool with running water for at least 10 minutes, cover loosely with a clean dressing and do not burst blisters.

221.	How do you treat hypothermia?

Ans: Remove wet clothing, insulate the casualty, give warm drinks if conscious, and do not rub the limbs.

222.	What do you do for a casualty in an enclosed space?

Ans: Do not enter without breathing apparatus. Raise the alarm and carry out the rescue using trained personnel and the rescue plan.

223.	Which medical guides are carried on board?

Ans: The International Medical Guide for Ships (WHO) and the medicine chest required by the flag state.

224.	What are the first actions for a hull breach?

Ans: Sound the alarm, stop engines, close watertight doors, assess the damage, plug and shore up, counter-flood to control the list, and send a distress message if required.

225.	What action do you take after a collision with a breach forward?

Ans: Reduce speed, keep the bow into the other vessel, shore up the collision bulkhead, and consider beaching only as a last resort.

226.	What are the actions on grounding?

Ans: Stop engines, sound all tanks and surrounding depths, check for damage and pollution, avoid using the engines until the situation is assessed, and take positive action based on the tide and the damage.